3GPP TSG RAN WG2 Meeting #105							    R2-1900277

Athens, Greece, 25th February - 1st March 2019

Source:	CATT

Title:	Data Forwarding with Conditional HO

Agenda Item:	12.3.3

Document for:	Discussion and Decision

1	Introduction

Handover robustness improvement with condition HO was discussed in RAN2#104 and the following agreements were made.

Agreements

1	RAN2 will consider a conditional handover: This is defined as UE having network configuration for initiating access to a target cell based on configured condition(s).

2	Usage of conditional handover is decided by network. UE evaluates when the condition is valid.

=>	FFS on the exact details of the procedures

Exact procedure is yet to be discussed. There are many different aspects which should be analysed in support of conditional HO. How to support data forwarding in conditional HO is one aspect which should be discussed. We analyse possible solutions for providing data forwarding in conditional HO in this contribution.

2. Discussion

Conditional HO is under discussion in RAN2 as a way to improve HO robustness. Basic steps of conditional HO is shown in Figure 1. The UE is provided with early HO configuration for a single or multiple target nodes (FFS on support of multiple target nodes). The condition for HO is also provided to the UE by the network. The network also prepares the single or multiple target nodes for the possible UE arrival. When the HO condition is met, the UE performs the HO towards the prepared target cell.

Figure 1: Basic conditional HO procedure.

The UE accesses the target node, where the target node signals the source node of HO completion. At this point of time the source cell starts forwarding the data towards the target node. Even though, the conditional HO procedure improves the HO robustness (i.e. guarantees that the UE receives the HO command),the data forwarding only starts after the UE accesses to the target cell. Thus the basic conditional HO introduces extra interruptions or jitter to the data delivery.

Observation 1: If data forwarding starts after the completion of conditional HO to the target cell, it may introduce extra interruptions or jitter to the data delivery.

It has been proposed in contributions that the UE signals the source cell that the UE is leaving the cell, prior to the UE access in the target node (prior to step 7) in Figure 1. The signal can be used as the trigger for data forwarding. This solution has a number of drawbacks.

1). At the time of the UE signals the source node that the UE is leaving the cell, the radio condition between the source and the UE is not good. Hence there is no guarantee that the signal would be received by the source node.

2). Even if the signal is received by the source, there is no guarantee that the data forwarding has started before the UE accesses the target cell.

3). The UE may have configured with multiple target nodes for conditional HO. A simple signalling to indicate that the UE is leaving the source cell doesn’t inform the network of which target cell the UE is moving to.

4). Adding more information of the target cell the UE is moving to increases the signalling and also degrades the possibility of successful delivery over a poor quality radio link towards the source cell.

Observation 2:  Solutions based on the UE informing the source cell of execution of conditional HO to trigger data forwarding have number of drawbacks.

In conventional HO, the data forwarding is triggered by the source cell upon the transmission of HO command to the UE. The data forwarding is not based on UE action, rather left to the network procedure when and how to start data forwarding to the prepared target cell. Even for the conditional HO, it is worth maintaining network procedure for data forwarding. There are a number of different methods which could be implemented at the network to mitigate data interruption or jitter.

In one method, the source and the target may maintain a data forwarding window. The source may configure data forwarding window in early HO request message (in step 3). The data forwarding configuration provides the necessary information for the data forwarding path as well as data forwarding window size. The source node starts forwarding data to the target node upon the transmission of conditional HO command to the UE or upon the acknowledgement of the correct reception of conditional HO command by the UE. The target node receives the data and keeps the data which fall within the data forwarding window and discards any data fall outside of the window. The data forwarding window is a sliding window maintained at the target node. The target node keeps the newest data received at the target node. With the data forwarding configuration, the source can reconfigure the window size as required or commands the target node to release all the data kept at the target node.

In another method, the source cell provides the target cell with ACKed and/or NACKed PDUs periodically. Based on this information, the target cell can release already ACKed PDUs in the buffer.

Network based solutions could overcome the data forwarding issues in conditional HO. As the data is already available at the target node when the UE accesses the target cell, the target cell could start transmission of data to the UE without any delay, hence avoiding data interruption and jitter.

Proposal 1: RAN2 is requested to agree that network based solution(s) should be considered to mitigate possible data interruption and jitter in conditional HO.

If proposal 1 is agreeable, the network based solutions should be discussed in RAN3. The above described data forwarding window based solution and ACKed/NACKed based solution are given as examples which could be considered in the discussion. RAN3 should be requested for investigation of network based solutions for data forwarding and mitigation of data interruption in conditional HO.

Proposal 2: RAN2 is requested to send a LS to RAN3.  RAN3 should be requested for investigation of network based solution for data forwarding and mitigation of data interruption in conditional HO.

3	Conclusion

In this contribution, we have discussed data forwarding in conditional HO in order to mitigate data interruption and jitter. Analysis suggested that network based solutions are favourable over UE signalling based solutions. The following observations and proposals were made.

Observation 1: If data forwarding starts after the completion of conditional HO to the target cell, it may introduce extra interruptions or jitter to the data delivery.

Observation 2:  Solutions based on the UE informing the source cell of execution of conditional HO to trigger data forwarding have number of drawbacks.

Proposal 1: RAN2 is requested to agree that network based solution(s) should be considered to mitigate possible data interruption and jitter in conditional HO.

Proposal 2: RAN2 is requested to send a LS to RAN3.  RAN3 should be requested for investigation of network based solution for data forwarding and mitigation of data interruption in conditional HO.