3GPP TSG-RAN2 #105	      R2-1900373

Athens, Greece, February 25th – March 1st 2019

Agenda Item:	7.2 – Narrowband IoT

Source:	NTT DOCOMO Inc.

Title:	Ambiguities on the Minimum Length of HARQ RTT Timer

Document for:	Discussion

Introduction

The HARQ RTT (round-trip time) Timer specifies the minimum number of subframes before a HARQ retransmission is expected by the UE [1]. For DL assignments, the timer is simply referred to as the HARQ RTT Timer, while for UL there is a separate timer with the same function called the UL HARQ RTT Timer.

For NB-IoT, these timers are defined separately within the same section of TS 36.321. Due to ambiguities within these definitions, we have encountered interoperability issues related to a) what the minimum length for each timer is expected to be; and b) when the UE should start monitoring NPDCCH after the timer expires. With the way that the current definitions are written, we feel that the RAN2 specification contradicts what the RAN1 specification says about the minimum length of these timers.

In this paper we will look at the current definitions in the specifications, previous RAN2 discussions on clarifying the timer definitions, and lastly we will propose that RAN2 clarify the specification to resolve these ambiguities.

Discussion

Minimum Timing Requirements According to RAN1

For HARQ, TS 36.213 subclause 16.6 defines the minimum amount of time that the UE is not required to monitor NPDCCH as follows:

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3;

-	else if the UE is not configured with higher layer parameter edt-parameters or if the UE is configured with higher layer parameter edt-parameters and

-	if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3.

Put more succinctly, from the time that an NPUSCH transmission (the HARQ feedback transmission) ends at subframe n, the UE is not required to monitor NPDCCH over the interval n+1 to n+3. This implies that the earliest period from which the UE may begin monitoring NPDCCH is from n+4. With the way that the RAN1 specification is written, this understanding comes across clearly.

Observation 1: If a HARQ transmission ends in subframe n, the RAN1 specification states that the earliest that a UE may begin monitoring NPDCCH is from n+4.

Minimum Timing Requirements According to RAN2

Compared to the RAN1 specification, the RAN2 definitions in TS 36.321 subclause 7.7 are written in a way that we feel are not entirely straightforward:

For NB-IoT the HARQ RTT Timer is set to k+3+N+deltaPDCCH subframes, where k is the interval between the last subframe of the downlink transmission and the first subframe of the associated HARQ feedback transmission and N is the transmission duration in subframes of the associated HARQ feedback, and deltaPDCCH is the interval from the last subframe of the associated HARQ feedback transmission plus 3 subframes to the first subframe of the next PDCCH occasion.

For NB-IoT, the UL HARQ RTT timer length is set to 4+deltaPDCCH subframes, where deltaPDCCH is the interval from the last subframe of the PUSCH transmission plus 4 subframes to the first subframe of the next PDCCH occasion.

For each case, assume that DRX is configured.

UL HARQ RTT Timer

Let us first look at the shorter of the two – the definition for the UL HARQ RTT Timer. The last subframe of the PUSCH transmission refers to the last subframe of the HARQ feedback transmission, which would make this subframe n. Based on the existing wording alone, if deltaPDCCH is set to 0, then it is unclear if the UL HARQ RTT Timer expires at subframe n+4, and the UE may begin monitoring NPDCCH from n+4, (which aligns with the RAN1 specification), or if the interval runs from n+1 to n+4 and the timer expires at subframe n+5, meaning that the UE may begin monitoring NPDCCH from n+5.

Observation 2: The following two interpretations exist for the minimum monitoring time for the UL HARQ RTT Timer, where n is the last subframe of the associated HARQ feedback transmission:

- The first subframe from which the UE may begin monitoring NPDCCH is n+4.

- The first subframe from which the UE may begin monitoring NPDCCH is n+5.

It should be clear that differing understandings would cause IOT issues. For example, if deltaPDCCH = 0, if the NW’s interpretation is 1) and the UE’s interpretation is 2), the NW may transmit NPDCCH in subframe n+4, while the UE does not begin monitoring NPDCCH until n+5, missing the NW transmission.

DL HARQ RTT Timer

Now we return to the HARQ RTT Timer used for DL. We feel that the main issue stems from the way that the two intervals k and 3+deltaPDCCH are defined. K denotes the interval before the HARQ feedback transmission, while 3+deltaPDCCH is the interval after the transmission.

In the definition of k, it is clear to us that the last subframe of the downlink transmission and the first subframe of the associated HARQ feedback transmission, the two endpoints of the interval, are not counted. If we abstract this to a separate example, where stations on a train line are ordered as [1st] – [2nd] – [3rd] – [4th], and one is asked for the number of stations in between [1st] and [4th] station (an interval), it should be clear that [1st] and [4th] station are not counted, and that the interval length is two stations ([2nd] and [3rd] station).

However, the length of the 3+deltaPDCCH interval is more ambiguous. The starting point is clear – “…the last subframe of the associated HARQ feedback transmission (subframe n)” – while the ending point is not. Based on the way that it is written – “…plus 3 subframes to the first subframe of the next PDCCH occasion” – there are two possible interpretations:

- The interval’s end point is “the first subframe of the next PDCCH occasion” exclusive. Because the end points in an interval are not counted towards that interval’s length, then that means that if deltaPDCCH is 0, the HARQ RTT Timer expires at subframe n+3, and the UE may begin monitoring NPDCCH beginning from that subframe. However, a minimum interval n+3 conflicts with the RAN1-defined minimum interval of n+4.

- The interval’s end point is “the first subframe of the next PDCCH occasion” inclusive. If this is in fact the intended case, then while this correctly matches up with the RAN1 specification, we believe that this does not align with the conventional counting method for an interval. Looking again at the current definition, this interpretation is not immediately the clear and correct one.

If one were to simply set deltaPDCCH as 0 to try and ascertain the minimum timer length, looking at the formula alone, we think that the above 1) would be mistakenly understood, the HARQ RTT Timer would expire 1 subframe early, and the UE would begin monitoring NPDCCH in a way that seems to obey the RAN2 specification but clearly violates the RAN1 specification.

Observation 3: Based on the wording, the correct interpretation for the DL HARQ RTT Timer is not immediately clear.

Previous Discussions on Clarification for the DL Timer

At RAN2#97, this issue was discussed in R2-167652 [3] and R2-1700764 [4]. [3] similarly identified ambiguities for the 3+deltaPDCCH interval, and the company behind that paper provided their understanding of the timer counting as follows.

Figure 1: Understanding of the HARQ RTT Timer in R2-167652

The discussion paper proposed that the spec be clarified accordingly such that it is clear that at the earliest, the UE may begin monitoring from n+4 subframes after the associated HARQ feedback transmission (although Figure 1, their proposed understanding, admittedly does not reflect that minimum monitoring time if deltaPDCCH is set to 0).

In R2-1700764, another interpretation was presented in [4] as follows:

Figure 2: Understanding of the HARQ RTT Timer in R2-1700764

According to the meeting notes for RAN2#97, both papers were simply “noted”. What we find strange about the interpretation in [4] is the way that k is defined, in particular that the starting subframe is counted exclusively as we would expect for an interval, but that the ending subframe is counted inclusively. As to why k should be counted this way, the company behind [4] cited the RAN1 specification below (TS 36.213 16.4.2) to justify their interpretation.

The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, at the end of  DL subframe transmission of the NPUSCH carrying ACK/NACK response using NPUSCH format 2 …

If this is in fact the correct way in which k should be interpreted, we think that that is not immediately obvious from the current definition in the RAN2 specification. At the very least, a NOTE should be added pointing to the relevant section of the RAN1 specification and k’s length should be more clearly defined.

On the other hand, the interpretation in [4] seems aligned with the minimum timer length as defined by RAN1 – if deltaPDCCH is set to 0 in their figure, then the earliest subframe from which the UE may begin monitoring NPDCCH from n+4.

Observation 4: The following two interpretations exist for counting k and deltaPDCCH for the (DL) HARQ RTT Timer:

1)  As defined in [3], the last subframe of k is counted exclusively, while the last subframe of deltaPDCCH is counted inclusively.

2)  As defined in [4], the last subframe of k is counted inclusively, while the last subframe of deltaPDCCH is counted exclusively.

While the resulting timer length in each interpretation is the same, we would prefer that a common understanding be reached between companies.

Furthermore, neither discussion paper addressed the minimum monitoring time. Looking at each figure, the results of each previous discussion paper would be as follows:

Observation 5: The following two interpretations exist for the minimum monitoring time for the (DL) HARQ RTT Timer, where n is the last subframe of the associated HARQ feedback transmission:

- As defined in [3], the first subframe from which the UE may begin monitoring NPDCCH is n+3.

- As defined in [4], the first subframe from which the UE may begin monitoring NPDCCH is n+4.

Considering the above with the current RAN2 specification, we think that this causes another difference in understandings, because 1) would be satisfied by using a deltaPDCCH value of 0, while 2) would be satisfied by using a deltaPDCCH value of 1.

Depending on the timing of the next NPDCCH occasion, an interoperability issue may be caused. If the NW is implemented according to 2) and the UE is implemented according to 1), then the UE will begin monitoring NPDCCH 1 subframe early than it needs to but will not miss the next NPDCCH occasion when transmitted by the NW. However, if the NW is implemented according to 1) and the UE is implemented according to 2), then if the NW transmits NPDCCH beginning at subframe n+3, then the UE will not be monitoring NPDCCH at that occasion.

Based on the above discussion we propose that RAN2 should clarify the specification.

Proposal 1: RAN2 to clarify the timer definitions in the MAC specification to align with the RAN1 specification.

If RAN2 agrees to clarify, then we have a proposed clarification in draft CR R2-1900374 [5], where we propose specifying the minimum allowable values of deltaPDCCH for the DL and UL timers, and adding a sentence of clarification to each specifying that the UE shall begin monitoring PDCCH at subframe n+4 in accordance with TS 36.213.

Conclusion

Observation 1: If a HARQ transmission ends in subframe n, the RAN1 specification states that the earliest that a UE may begin monitoring NPDCCH is from n+4.

Observation 2: The following two interpretations exist for the minimum monitoring time for the UL HARQ RTT Timer, where n is the last subframe of the associated HARQ feedback transmission:

- The first subframe from which the UE may begin monitoring NPDCCH is n+4.

- The first subframe from which the UE may begin monitoring NPDCCH is n+5.

Observation 3: Based on the wording, the correct interpretation for the HARQ RTT Timer is not immediately clear.

Observation 4:  The following two interpretations exist for counting k and deltaPDCCH for the (DL) HARQ RTT Timer:

1)  As defined in [3], the last subframe of k is counted exclusively, while the last subframe of deltaPDCCH is counted inclusively.

2)  As defined in [4], the last subframe of k is counted inclusively, while the last subframe of deltaPDCCH is counted exclusively.

Observation 5: The following two interpretations exist for the minimum monitoring time for the (DL) HARQ RTT Timer, where n is the last subframe of the associated HARQ feedback transmission:

- As defined in [3], the first subframe from which the UE may begin monitoring NPDCCH is n+3.

- As defined in [4], the first subframe from which the UE may begin monitoring NPDCCH is n+4.

Proposal 1: RAN2 to clarify the timer definitions in the MAC specification to align with the RAN1 specification.

References

[1] TS 36.213 Physical Layer Procedures, v.15.4.0.

[2] TS 36.321 Medium Access Control (MAC) Protocol Specification, v15.4.0.

[3] R2-167652, Discussion on HARQ RTT, HTC, RAN2#96, November 2016.

[4] R2-1700764, HARQ RTT Timer definition for NB-IoT, Ericsson, RAN2#97, February 2017.

[5] R2-1900374, Correction to Minimum Length for DL HARQ RTT Timer, NTT DOCOMO INC., RAN2#105, February 2019.