3GPP TSG-RAN WG2 #105	Tdoc R2-1900398

Athens, Greece, 25th Feb – 1st Mar 2019

Agenda Item:	11.9.1

Source:	Ericsson

Title:	Scope and ways of working for the NR mobility enhancements WI

Document for:	Discussion

# Introduction

RAN2#105 is the first meeting of the Rel-16 NR mobility enhancements work item which aims to enhance NR mobility by reducing handover interruption time and improve handover robustness.

In this contribution we discuss the scope of this new work item and the similarities and differences compared to the ongoing Rel-16 work item on LTE mobility enhancements. We propose some principles for ways of working to avoid duplicate work between the two work items.

# Discussion

## Time plan

Both the NR and LTE WI contain the same high-level objectives, namely reducing handover interruption and improving mobility robustness. The LTE WI started already in RAN2#103bis while the NR WI starts three meetings later in RAN#105. Both WIs are expected to finish at the same time at RAN2#108.

## Ways of working

The present situation reminds to some extent to when LTE connected to 5GC was specified in Rel-15. In that work item, we would typically try to align LTE connected to 5GC with NR and after an agreement had been made for NR the same agreement would typically be adopted for LTE without much discussion. Here the situation is slightly reversed since we started with LTE and the NR agreements might follow the LTE ones instead of the other way around. Regardless of who follows whom, it is beneficial if the LTE and NR solutions are aligned to reduce the evaluation and specification effort. The NR work item also explicitly states that “LTE mobility enhancements should be used for baseline for fast handover failure recovery, Make-before-break and RACH-less handover”.

In general, we should try to align the solutions for NR and LTE to reduce the evaluation and specification effort.

This should be possible considering that NR and LTE are similar to a large extent and if we for example look at the basic handover procedure in LTE and NR they appear almost identical to each other. Of course, there are details of the solution that will differ between NR and LTE due to NR specific aspects such as beamforming, higher frequencies, CU/DU split, etc. However, the high-level principles of the solution can probably be common. When agreeing to a high-level principle for one RAT we should therefore check if the same principle can be agreed also for the other RAT.

When making an agreement for one RAT check if the same agreement can be made also for the other RAT.

A good starting point is to go through the agreements made so far in the LTE WI and see which ones are applicable to NR.

The NR WI should start by checking the existing agreements in the LTE WI and see which ones that are applicable to NR.

Then regarding how to capture the agreements, we can follow the same approach as in the LTE WI and rely on the chairman’s notes. Later, once the solutions have progressed some more, we can create a running stage-2 CR to capture what has been agreed.

Rely on chairman’s notes to capture agreements in the initial phase of the work item. Later on a running stage-2 CR should be created.

In the next sections we discuss the NR objectives in more detail and see how they differ from the LTE ones.

## Objective #1 – Mobility interruption enhancements

According to the NR WI, the solutions for reducing handover interruption include but are not limited to:

- Handover with simultaneous connectivity with source cell and target cell.

- Make-before-break

- RACH-less handover

The first two solutions are also being discussed in the LTE WI. The first solution refers to handover based on dual connectivity (DC) and the second solution refers to a possibly enhanced version of the make-before-break handover introduced in LTE Rel-14. The DC based solution and the enhanced make-before-break solution are also known as the split and non-split bearer solution, respectively, in the LTE discussions. The third solution is not a solution on its own but should rather be seen as a complement to the two other solutions.

Based on the email discussion [3], it appears the second solution (make-before-break) is the solution that is favoured by most companies for LTE. Following the principle in proposal 1, it would be beneficial if the same solution is specified also for NR.

Aim to specify a common solution for enhanced make-before-break handover (aka non-split bearer solution) in LTE and NR.

The third solution listed above (RACH-less handover) is not able to reach the ~0ms interruption target on its own but for this we must use one of the other two solutions (DC based handover or make-before-break). Furthermore, neither DC based handover nor make-before-break rely on RACH-less and can reach the ~0ms interruption target even without it. So in this sense RACH-less handover is not essential for the WI objective. However, RACH-less handover may still be useful as a complement since it reduces the time it takes to establish the connection to the target cell, which is important if for example the source cell connection is deteriorating quickly. RACH-less handover is already supported in LTE since Rel-14 and is therefore not listed as an objective in the LTE WI.

Discuss if RACH-less handover should be specified in NR as a complementary solution for reduced handover interruption.

Another difference compared to the LTE WI is that the NR WI also aims to reduce interruption timer during SCG change. In our view it is more important to avoid interruption during a regular handover since at SCG change the UE can continue to transmit and receive data via the MCG. However, if the additional work to specify this is small we are open to consider this.

Solutions for reducing interruption time during SCG change can be considered provided the additional specification work is limited.

## Objective #2 – Mobility robustness enhancements

According to the NR WI, the solutions for improving handover robustness include but are not limited to:

- Conditional handover

- Fast handover failure recovery

Conditional handover is also the main solution for mobility robustness in LTE. For the same reasons mentioned for handover interruption, it is beneficial if the conditional handover solution is aligned between LTE and NR.

Aim to specify a common solution for conditional handover in LTE and NR.

The second solution is only mentioned in the NR work item and is actually not a single solution but is rather a placeholder for all types of enhancements that reduces the time it takes for the UE to recover from a handover failure. One such enhancement is the T312 timer in LTE which causes the UE to trigger RRC re-establishment early if physical layer problems has been detected and a measurement report has been sent but no handover command has been received.

Mechanisms to improve handover failure recovery (e.g. T312 timer) can be studied but with less priority.

Unlike the LTE WI the NR WI also aims to improve mobility robustness at SCG change. At the moment it is not clear what type of enhancements that should be considered here.

# Conclusion

In this contribution we discussed the scope and ways of working for the new WI on NR mobility enhancements.  Based on the discussion in section 2 we propose the following:

Proposal 1	In general, we should try to align the solutions for NR and LTE to reduce the evaluation and specification effort.

Proposal 2	When making an agreement for one RAT check if the same agreement can be made also for the other RAT.

Proposal 3	The NR WI should start by checking the existing agreements in the LTE WI and see which ones that are applicable to NR.

Proposal 4	Rely on chairman’s notes to capture agreements in the initial phase of the work item. Later on a running stage-2 CR should be created.

Proposal 5	Aim to specify a common solution for enhanced make-before-break handover (aka non-split bearer solution) in LTE and NR.

Proposal 6	Discuss if RACH-less handover should be specified in NR as a complementary solution for reduced handover interruption.

Proposal 7	Solutions for reducing interruption time during SCG change can be considered provided the additional specification work is limited.

Proposal 8	Aim to specify a common solution for conditional handover in LTE and NR.

Proposal 9	Mechanisms to improve handover failure recovery (e.g. T312 timer) can be studied but with less priority.

# References

RP-181433, New WID: NR mobility enhancements, RAN#80

RP-181475, New Work Item on even further Mobility enhancement in E-UTRAN, RAN#80

R2-1900619, Email discussion [104#61][LTE/feMOB] Solution directions for minimizing user data interruption for UL/DL (Nokia)