3GPP TSG-RAN WG2#105	R2-1900414

# Athens, Greece, 25th February – 1st March, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0821 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.331 on T380 expiration while UE is out of coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_core |  |  |  |  |  |  | Date: |  | 2019-02-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The scenario when the timer T380 expires but the UE is out of coverage is not completely described in 38.331. Two cases are possible: - The UE does not find a suitable cell. The scenario is already covered by Ch.5.3.13.8 where the UE shall “perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other'”. In this case, since the UE enters RRC_IDLE, no other actions are needed. - The UE finds a suitable cell In this case the UE shall perform the RNAU procedure, because of the T380 expiry. This is not described in 38.331 Only the first case is described in 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Chapter 5.3.13.8 has been modified to specify that in case the timer T380 expires and the UE does not camp in a suitable cell, the variable pendingRnaUpdate is set to to true. In the same chapter it is also specified that when the UE finds a suitable cell (that is, it goes back in coverage), the UE will trigger RNAU if the pendingRnaUpdate is set to to true Impact analysis Impacted functionality: RNA update If the UE implements the change but the NW does not, no issue on the NW side is expected, since the impacts are only in the UE. If the NW implements the change but the UE does not, the UE will not perform RNAU when it is back in coverage which may cause the network to move the UE to RRC_IDLE state which in turn causes a state mismatch between UE and network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consequence is the specification does not cover one of the scenarios triggering the RNAU. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13.8 RNA update |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.3.13.8	RNA update

In RRC_INACTIVE state, the UE shall:

1>	if T380 expires; or

1>	if RNA Update is triggered at reception of SIB1, as specified in 5.2.2.4.2:

2> if the UE camps in a suitable cell:

3>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause set to rna-Update;

2> else

3> set the variable pendingRnaUpdate to true

1>	if barring is alleviated for Access Category '8', as specified in 5.3.14.4:

2>	if upper layers do not request RRC the resumption of an RRC connection, and

2>	if the variable pendingRnaUpdate is set to true:

3>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause value set to rna-Update.

If the UE in RRC_INACTIVE state finds a suitable cell to camp after being in any cell selection state as defined in [20], the UE shall:

1>	if the variable pendingRnaUpdate is set to true:

2>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause value set to rna-Update.

If the UE in RRC_INACTIVE state fails to find a suitable cell and camps on the acceptable cell to obtain limited service as defined in TS 38.304 [20], the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other'.

END OF CHANGES