3GPP TSG-RAN WG2 Meeting #105	 R2-1900446

Athens, Greece, 25 Feb - 1 Mar 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0824 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of SlotFormatCombinations in case of SCell Release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-2-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For a serving cell, the IE PDCCH-ServingCellConfig configures UE specific PDCCH parameters, in which the field slotFormatIndicator further configures a list of SlotFormatCombinations for other serving cells. When the serving cell is an SCell and the SCell is released, it is possible that other serving cells in slotFormatCombToAddModList are not released. In this case, it is unclear whether to keep or release SlotFormatCombinations given for other serving cells in the slotFormatIndicator of this SCell. According to current principle of SCell release, all the configurations stored for the SCell​should be released. It implies SlotFormatCombinations for other serving cells is also released. Therefore, the network should guarantee that slotFormatIndicator of the SCell to be released should not include other serving cells’ SlotFormatCombinations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in the field description that if the network releases an SCell which provides SlotFormatCombinations for other serving cell(s), the SlotFormatCombinations are released. Impact Analysis Applicable to EN-DC or SA Both Impacted functionality PDCCH configuration in RRC_CONNECTED Inter-operability for SA - If the network is implemented according to the CR and the UE is not, no inter-operability issue is foreseen. - If the UE is implemented according to the CR and the network is not, UE releases the SlotFormatCombinations for the existing serving cells when a SCell is released and SlotFormatCombinations is given in slotFormatIndicator of this released SCell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE doesn’t have valid configuration of SlotFormatCombinations for the existing serving cells, if a SCell is released and the SlotFormatCombinations for the existing serving cells is given in slotFormatIndicator of the released SCell. PDCCH monitoring for those serving cells is not workable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 6.3.2	Radio resource control information elements

#### –	SlotFormatIndicator

The IE SlotFormatIndicator is used to configure monitoring a Group-Common-PDCCH for Slot-Format-Indicators (SFI).

SlotFormatIndicator information element

-- ASN1START

-- TAG-SLOTFORMATINDICATOR-START

SlotFormatIndicator ::=     SEQUENCE {

sfi-RNTI                    RNTI-Value,

dci-PayloadSize             INTEGER (1..maxSFI-DCI-PayloadSize),

slotFormatCombToAddModList  SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell OPTIONAL, -- Need N

slotFormatCombToReleaseList SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF ServCellIndex                 OPTIONAL, -- Need N

...

}

-- TAG-SLOTFORMATINDICATOR-STOP

-- ASN1STOP

SlotFormatIndicator field descriptions

dci-PayloadSize Total length of the DCI payload scrambled with SFI-RNTI (see TS 38.213 [13], clause 11.1.1)'

sfi-RNTI RNTI used for SFI on the given cell (see TS 38.213 [13], clause 11.1.1).

slotFormatCombToAddModList A list of SlotFormatCombinations for the UE's serving cells (see TS 38.213 [13], clause 11.1.1). NOTE: If the network releases an SCell which provides SlotFormatCombinations for other serving cell(s), the SlotFormatCombinations are released.