3GPP TSG-RAN WG2 Meeting #105												R2-1900506

Athens, Greece, 25th February - 1st March 2019

Source: 	vivo

Title:  	Discussion on the ROHC procedure

Agenda Item:	12.3.2

Document for:	Discussion and Decision

# Introduction

According to the email discussion report [1], the solution direction would be to use the non-split bearer option for setting up the connection simultaneously between the source cell and target cell. Most companies consider that the ROHC should be handled by the source and the target node separately. However the UE behaviours regarding the ROHC is still not clear. In this contribution, we focus on the UE behaviours regarding the ROHC.

# Discussion

## UE behaviors regarding the ROHC

Regarding the ROHC, RAN2 needs to firstly discuss whether the ROHC profile is different between the source node and the target node. According to the current ROHC configuration, the network configures a set of ROHC profiles, as quoted in the Annex. Which ROHC profile/context is currently being used by the UE is only known by the ROHC entity, and it is not very clear if the in-use ROHC profile/context could be changed during the handover procedure. According to the legacy handover, the ROHC continue function (i.e. drb-ContinueROHC) is only used for the handover within the same eNB.

Observation: In the legacy handover, the ROHC continue function (i.e. drb-ContinueROHC) is only used for the handover within the same eNB.

As such, we would assume that if the handover is between different eNBs, then the ROHC profile should be different between the source node and the target node.

Proposal 1: For the DC handover, the ROHC continue function (i.e. drb-ContinueROHC) is only used for a handover within the same eNB, as the legacy handover.

Proposal 2: For the DC handover, the ROHC profile(s) between the source node and the target node are expected to be different if the handover is between different eNB(s), as the legacy handover.

As such we would expect that the UE should have two ROHC entities, one for the source node and one for the target node, if the UE is required to receive/transmit PDCP data from/to both the source node and the target node. The details regarding how to differentiate the ROHC entity for compression/decompression depending on the discussion on when the UE starts the data transmission/reception to/from the target cell and when the UE stops the data transmission/reception to/from the source cell.

Proposal 3: If the UE is required to receive/transmit PDCP PDU(s) from/to both the source node and the target node simultaneously, the UE should have two ROHC entities, one for the source node and one for the target node

# Conclusions

Based on the analysis given above, we have the following Observations and Proposals：

Observation: In the legacy handover, the ROHC continue function (i.e. drb-ContinueROHC) is only used for the handover within the same eNB.

Proposal 1: For the DC handover, the ROHC continue function (i.e. drb-ContinueROHC) is only used for a handover within the same eNB, as the legacy handover.

Proposal 2: For the DC handover, the ROHC profile(s) between the source node and the target node are expected to be different if the handover is between different eNB(s), as the legacy handover.

Proposal 3: If the UE is required to receive/transmit PDCP PDU(s) from/to both the source node and the target node simultaneously, the UE should have two ROHC entities, one for the source node and one for the target node

# Reference

[1] [104#61][LTEfeMOB]  Solution directions for minimizing user data interruption for ULDL (Nokia)

# Annex

The followings are quoted for the specification 36.331:

headerCompression CHOICE { notUsed NULL, rohc SEQUENCE { maxCID INTEGER (1..16383) DEFAULT 15, profiles SEQUENCE { profile0x0001 BOOLEAN, profile0x0002 BOOLEAN, profile0x0003 BOOLEAN, profile0x0004 BOOLEAN, profile0x0006 BOOLEAN, profile0x0101 BOOLEAN, profile0x0102 BOOLEAN, profile0x0103 BOOLEAN, profile0x0104 BOOLEAN }, ... } },