3GPP TSG-RAN WG2 #105	Tdoc R2-1900544

Athens, Greece, 25th February - 1st March 2019

Agenda Item:	11.6.4.2

Source:	Ericsson

Title:	Tracking area management and update for NTN LEO

Document for:	Discussion

# Introduction

In RAN#80, a new SI “Solutions for NR to support Non-Terrestrial Network” was agreed [2]. It is a continuation of the preceding SI “NR to support Non-Terrestrial Networks” (RP-171450), where the objective was to study the channel model for the non-terrestrial networks, to define deployment scenarios, parameters and identify the key potential impacts on NR. The results are summarized in [1]. The new study item has the objective at evaluating potential solutions addressing the minimum necessary identified key impact areas from the previous activity and to study impact on RAN protocols/architecture. The SID is updated in [4] and has the objectives for layer 2 and above are:

- Study the following aspects and identify related solutions if needed: Propagation delay: Identify timing requirements and solutions on layer 2 aspects, MAC, RLC, RRC, to support non-terrestrial network propagation delays considering FDD and TDD duplexing mode. This includes radio link management. [RAN2] - Handover: Study and identify mobility requirements and necessary measurements that may be needed for handovers between some non-terrestrial space-borne vehicles (such as Non Geo stationary satellites) that move at much higher speed but over predictable paths [RAN2, RAN1] - Dual connectivity [RAN3 aspects] involving - NTN-based NG-RAN (Transparent GEO or LEO satellites) and terrestrial based NG-RAN access: Xn terminated on the ground - or two NTN-based NG-RAN access (between Regenerative LEO satellites): Xn over ISL - Architecture: Identify needs for the 5G’s Radio Access Network architecture to support non-terrestrial networks (e.g. handling of network identities) [RAN3] - Paging: procedure adaptations in case of moving satellite foot prints or cells

In RAN2#103bis, it is agreed to study the following UP and CP aspects:

UP Impacts to study

- DRX

- HARQ

- Random access response

- RLC/PDCP reordering (e.g. timers and SN space)

- SDAP => no impact

Impacts to study for CP

- Mobility

- TA management and update

In RAN2#104, the following is agreed for tracking area and paging topic:

Agreements:

- RAN2 to study solutions for two different type of UE categories 1) with GNSS support, 2) without GNSS support

- For GEO, the current tracking area management is assumed as a baseline

- For LEO with moving beams study fixed and moving tracking area solutions

In this paper, we discuss tracking area management and update for LEO NTN. RAN3 considerations can be found in [7].

# Discussion

After RAN2#103bis, the complete list of 6 scenarios in TR 38.821 is:

Scenario A – GEO, transparent satellite, Earth-fixed beams;

Scenario B – GEO, regenerative satellite, Earth fixed beams;

Scenario C1 – LEO, transparent satellite, Earth-fixed beams;

Scenario C2 – LEO, transparent satellite, Earth-moving beams;

Scenario D1 – LEO, regenerative satellite, Earth-fixed beams;

Scenario D2 – LEO, regenerative satellite, Earth-moving beams.

The updated SID affects the list of scenarios that are intended to be covered during the Rel-16 study item:

Targeted usage-scenarios with satellite access are pedestrian, stationary in building, on board vehicle, high speed train or airplanes. Stationary in building, on board vehicle, high speed train or airplanes usage-scenarios may require a relay UE. The priority of the Rel-16 study will be on pedestrian and on board vehicle usage-scenarios.

The possible reference scenarios and architecture options for non-terrestrial networks will be identified and described. However, the scope of the release 16 study item will be limited to key issues and solutions associated with transparent GEO satellite and LEO based non-terrestrial access network (moving beam on earth). The CU-DU and IAB feature as well as the combined operation of GEO / LEO networks for idle mode will not be considered in this study during release 16.

Thus, the scope would be reduced to the following set of scenarios:

Scenario A – GEO, transparent satellite, Earth-fixed beams;

Scenario C2 – LEO, transparent satellite, Earth-moving beams;

Scenario D2 – LEO, regenerative satellite, Earth-moving beams.

After down scoping the SID, there are two LEO scenarios C2 and D2 to be considered in the SI.

The tracking area (TA) can be assumed to be fixed on earth or to move with the cells as both were agreed to be considered in the SI. If TA is assumed fixed on earth, there need to be network side updates for the identities either very close to satellite gateway or at further nodes. TA can also be assumed to be moving with the satellites in which case the UE needs to update TAI frequently.

In case of earth moving beams, the NR DL reference signals SSBs that are transmitted via the satellite(s) will sweep the earth. Overall description of the SSB is specified in TS 38.300 in 5.2.4. where the most interesting parts are those related to cell identity. The physical reference signal consists of primary and secondary synchronization signals (PSS, SSS) and together form the PCI which is the physical cell identity. Other network identities are as listed in the same specification in Section 8.2:

-	NR Cell Global Identifier (NCGI): used to identify NR cells globally. The NCGI is constructed from the PLMN identity the cell belongs to and the NR Cell Identity (NCI) of the cell.

-	gNB Identifier (gNB ID): used to identify gNBs within a PLMN. The gNB ID is contained within the NCI of its cells.

-	Global gNB ID: used to identify gNBs globally. The Global gNB ID is constructed from the PLMN identity the gNB belongs to and the gNB ID. The MCC and MNC are the same as included in the NCGI.

Further, the SSB has the following specified link to the NCGI which is constructed from the PLMN identity the cell belongs to and the NR cell identity NCI. That is, when an SSB is associated with an RMSI, the SSB corresponds to an individual cell, which has a unique NCGI. Such an SSB is referred to as a Cell-Defining SSB (CD-SSB). The RMSI is essentially SIB1 which includes tracking area code, PLMN ID and cell ID. The SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms. SIB1 is always cell specific whereas other SIBs may be SI area specific that can include more than one cells.

All system information is stored with associated PLMNIdentity and cellIdentity and valueTag. UE may use valid stored SI except MIB and SIB1. If UE receives SI modification indication UE needs to reacquire SIB1.

PCI is used in all RRM measurement reports and for HOs thus in all connected mode mobility related RRC towards the UE.

# Earth fixed TA for transparent LEO satellite - C2

As discussed in [5], for transparent LEO, the feeder link switching is visible to the UE while for regenerative case it most likely is not, at least from standard perspective. Thus, for transparent LEO, we discuss separately the cases when feeder link stays fixed and when it switches. From mobility perspective the feeder link switch is discussed in [5][6].

## Case feeder link stays fixed

Figure 1 Earth fixed tracking area while LEO satellite is connected to one gNB.

For simplicity we assume one satellite is associated to one PCI.

- Satellite 5(PCI5) moves from TA1 to TA2 while keeping the feeder link connection to gNB1. It is noted that there will be in practice fluctuation where geographically the tracking area boarder goes due to the hard switching while satellite gradually moves on its orbit.

- After the move, the SI broadcasted via Satellite 5(PCI5) is updated to broadcast TAC2 instead of TAC1

  - Option 1: the cellID associated to PCI5 stays the same when moving under TA1 area.

    - The needed NG RAN node configuration updates and/or Xn update depends on whether cellID has been mapped to TA1 and TA2 or only TA1. (RAN3 discussion)

  - Option 2: the cellID associated to satellite 5 (PCI5) is changed to a cell ID belonging to TA2

    - Xn update is needed as cellID to PCI mapping changes

    - Needs a pool of cellIDs belonging to gNB1 some mapped to TA1 and some TA2.

    - NG RAN node configuration update is not needed

From UE perspective, the Option 1 is simpler as only TAC in the system information would change. This could be supported by assuming UE rereads TAC upon SI change notification or there would be specific change notification for TAC update. The Option 2 means that PCI-cellID association changes while UE is connected to the cell. It would require RAN2 discussion to study if this requires specification changes by RAN2.

## Case feeder link switches

When feeder link switches, another gNB starts to serve the UE and which typically means HO and establishment of all user plane protocols with related security keys. This would hint that it is better to actually perform the HO for connected mode UE in this case as concluded in [5].

The resulting option is that a satellite can broadcast more than one PCI during feeder link switch. Satellite can broadcast two PCIs as two DC-SSBs on different sync raster locations during the switch. The PCIs are coming from different gNBs on the ground which for a while connect to same satellite.

Figure 2 Feeder link switch for regenerative LEO

During the switch, a connected mode UE could perform HO from PCI9 belonging to gNB2 to PCI4 belonging to gNB1. RAN3 aspects of this are discussed in [6]. For the tracking area handling, there are two options. Option 1, the feeder link switches while the satellite belongs to the earth fixed TA1 and both CD-SSB SIB1 or PCI4 and PCI9 broadcast the same TAC. Option 2 assumes that tracking area covered by the satellite is switched when feeder link is switched. In this option, the CD-SSB SIB1 of PCI9 broadcasts TAC2 and CD-SSB SIB1 of PCI4 broadcasts TAC1.

Both feeder link switch and fixed tracking area are discussed in RAN3 [6][7] thus RAN2 could wait for that discussion to conclude.

RAN2 to wait for RAN3 to progress on earth fixed TA aspects for transparent LEO before capturing consequences from RAN2 perspective.

# Moving tracking area

In TR 38.821, Section 8.3 Registration Update Paging Handling describes moving tracking area solutions. Section 8.3.2 a moving tracking area solution assuming UE is not capable to determine its location is described. The principle is based on timing window based registration update. It is based on the assumption UE is not able to know its location yet it is based on “AMF determining which TAC covers UE location for a given time period”. It is unclear it itworks in a similar manner for stationary and moving UEs. If it is based on UE reading TAC and performing tracking area update as per current procedures it is unclear why UEs capability of knowing its location is relevant.

TR 38.821 Section 8.3.2 describes that if UE is able to determine its locations, UE could be paged based on UE locations. However, it is not described how the network is kept updated on UE location and what is the paging unit in this case. If it is based on UE reading TAC and performing tracking area update as per current procedures it is unclear why UEs capability of knowing its location is relevant.

RAN2 should discuss if further clarifications for the moving tracking area solutions are needed or if RAN2 is able to conclude the needed UE actions.

RAN2 should clarify whether there is enough information in TR 38.821 for the moving tracking area case in order to discuss the needed UE actions and other RAN2 implications for this case.

# Conclusion

We made the following observations:

Observation 1	After down scoping the SID, there are two LEO scenarios C2 and D2 to be considered in the SI.

We propose the following:

Proposal 1	RAN2 to wait for RAN3 to progress on earth fixed TA aspects for transparent LEO before capturing consequences from RAN2 perspective.

Proposal 2	RAN2 should clarify whether there is enough information in TR 38.821 for the moving tracking area case in order to discuss the needed UE actions and other RAN2 implications for this case.

# References

TR 38.811 Study on New Radio (NR) to support non-terrestrial networks

RP-181370, New SID on Solutions for NR to support Non Terrestrial Network, Thales, RAN#80, La Jolla

R3-186269, TR 38.821, Ver 020

RP-182880, Study on solutions for NR to support non-terrestrial networks (NTN)

R2-1900548, Feeder link switch for transparent and regenerative LEO, Ericsson, RAN2#105, Athens Greece

R3-181xxx, Switching the Feeder Link Toward the Satellite, Ericsson, RAN3#103, Athens Greece

R3-18xxx, Are Tracking Areas Stationary or Not?, Ericsson, RAN3#103, Athens Greece