3GPP TSG-RAN WG2 #105	R2- 1900551

# Athens, Greece, 25 Feb – 1 March 2019	                                           Revision of R2-1817247

Agenda Item:	11.7.2.1

Source:	OPPO

Title:	Time Synchronization for IIoT

Document for:	Discussion, Decision

Introduction

In RAN #81, a revised SID on Study on NR Industrial Internet of Things (IoT) is approved. The potential enhancements for TSN network on accurate reference timing are discussed and captured in this SI, as shown in the following [1]:

2)	Time Sensitive Networking related enhancements:

a)	Accurate reference timing: Delivery & related process (e.g. SIB delivery or RRC delivery to UEs, Multiple Transmission points) (RAN2/RAN3/RAN1).

And the baseline for this topic is achieved in RAN2 latest meeting, which is shown below:

We reuse the LTE approach for time distribution by broadcast RRC as a baseline, Unicast is FFS

0.25us granularity can be starting point, FFS finer granularity than 0.25us

In this paper, we will discuss the issue about accurate time synchronization and give our proposals.

Discussion

The requirement of clock synchronization accuracy for IIoT is defined in clause 8.1.6 of TR 22.804, in which the stringent clock synchronicity requirement is 1 µs. As we understood, whether time synchronization accuracy can be fulfilled at UE side is related to the following aspects: 1. synchronization accuracy over non-radio interface, 2. synchronization accuracy over Uu interface. These aspects are analyzed as below:

- Aspect 1: synchronization accuracy over non-radio interface

According to RAN3 LS, for the one over non-radio interface, the maximum absolute time error (TE) between TSN GM clock and gNB is summarized by RAN3, as shown below:

| Synchronization source | Synchronization accuracy |
| --- | --- |
| Local on-site GNSS receiver (GPS is TSN GM clock) | ¦TE¦ = 100 ns absolute, 200ns relative between nodes. |
| Local on-site TSN GM clock | TE is negligible. |
| Remote TSN GM clock entity using cascaded PTP capable transport network connections | ¦TE¦ ~N*40ns, where N is number of PTP hops. |

It means that it the maximum synchronization error over non-radio interface is in the unit of ns, and it is much less than 1µs.

- From the RAN3 perspective, the maximum synchronization error over non-radio interface is in the unit of ns.

- Aspect 2: synchronization accuracy over Uu interface

For the one over Uu interface, clock synchronization timing difference between UE and network is depended on:

- The one on gNB side: Accuracy on system layer synchronization information transferred by network, for which a finer granularity helps to improve the accuracy.

- The one on UE side: The delta from RAN1 synchronization error, i.e., the errors may come from link level synchronization estimation error. Physical layer synchronization estimation error is induced by many factors, such as propagation loss, equipment limit, etc.

Figure 1: Clock synchronization timing relation between UE and network.

For gNB side, synchronization accuracy information is on the system level, which is transferred by the network. In current NR RRC specification, the time accuracy is 10ms, and the IE is defined in SIB9.

In LTE Rel-15, some enhancement on accurate reference timing is introduced in HRLLC, and 0.25µs clock synchronicity granularity has been achieved. Considering 0.25µs is much less than 1µs clock synchronicity requirement defined in TR 22.804 and similar mechanism can be easily extended to NR IIoT without much complexity, 0.25us granularity is agreed to be starting point in NR, which is captured in last RAN2 meeting, and 0.25µs granularity can be easily achieved in NR.

- From the perspective of system level, 0.25µs clock synchronicity granularity can be easily achieved in NR, by using similar mechanism as the one applied in LTE R15.

For UE side, one LS reply is sent from RAN1 on requirement on TSN requirement evaluation shown as below:

RAN1 has performed analysis on the achievable time synchronization accuracy over Uu interface. A timing synchronization error between a gNB and a UE no worse than 540ns is achievable based on the RAN1 agreed evaluation assumptions for Rel-15 NR with 15kHz SCS. It is RAN1´s conclusion, that the synchronization accuracy is improved when using higher SCS. For small service areas with dense small cell deployments a propagation delay compensation by the UE would not be required. The propagation delay compensation needs to be applied by the TSN UEs for larger service areas with more sparse cell deployments (e.g. for inter-site distances >200m the gNB-to-UE timing synchronization accuracy without propagation delay compensation may be worse than 1us).

RAN1 conclude that the timing synchronization error between a gNB and a UE is no worse than 540ns, and the synchronization accuracy is improved in 30kHz (and higher) SCS. It means that the worst synchronization accuracy can be achieved by RAN1 is in the unit of ns. Thus, it is concluded that no much difficulty for RAN1 to maintain the similar synchronization accuracy level as in LTE R15, the delta from RAN1 synchronization error is still tolerant for synchronization accuracy requirement.

- From the RAN1 perspective, the maximum synchronization error is no worse than 540ns.

Thus, it can be summarized that 1µs granularity can be achieved, and no need to consider finer granularity for synchronization accuracy in RAN2.

No need to consider finer granularity than 0.25µs at this stage.

RAN can further study finer granularity than 0.25µs if there is a requirement from other work group.

RAN2 work first focus on how to obtain available time information from gNB.

In RAN2 last meeting, it is agreed that we reuse the LTE approach for time distribution by broadcast RRC as a baseline, and unicast is FFS. Then, two aspect left here to consider:

- Issue1: by which SIB 0.25µs granularity information is informed.

Currently, SIB9 is defined to contain information related to GPS time and Coordinated Universal Time (UTC). UE may use the parameters provided in this system information block for numerous purposes, possibly involving upper layers e.g. to assist GPS initialisation, to synchronise the UE clock. Thus, it is a most straightforward way to add accurate reference timing information in current SIB9 message.

Re-using similar solution on accurate time granularity supported in LTE HRLLC for IIoT, i.e. improve sync accuracy in SIB9.

- Issue2: whether unicast method, i.e. dedicated RRC signalling is adopted.

Some restriction exists for dedicated RRC signalling: accurate reference timing information is mandatory for UEs in all RRC mode (i.e. idle mode, inactive mode, and connected mode), but only UE in RRC connected mode can identify dedicated RRC message. One may argue that unicast way is more suitable to multiple clock accuracy scenario, and different time synchronization accuracy can be supported by different UEs in one cell. We think RAN2 should first request SA whether different synchronization accuracy is required in TSN system, and then start to discuss it if the reply is positive. And, currently different synchronization accuracy requirement is not required from other work group for different UEs connected to one cell.

- Currently different synchronization accuracy requirement is not required from other work group for different UEs connected to one cell.

RAN2 send a LS to SA1/SA2 on whether multiple clock synchronization accuracy requirement is needed in TSN system.

Conclusion

Based on the discussion above, we made the following observations:

Observation 1	From the RAN3 perspective, the maximum synchronization error over non-radio interface is in the unit of ns.

Observation 2	From the perspective of system level, 0.25µs clock synchronicity granularity can be easily achieved in NR, by using similar mechanism as the one applied in LTE R15.

Observation 3	From the RAN1 perspective, the maximum synchronization error is no worse than 540ns.

Observation 4	Currently different synchronization accuracy requirement is not required from other work group for different UEs connected to one cell.

And propose the following:

Proposal 1	No need to consider finer granularity than 0.25µs at this stage.

Proposal 2	RAN can further study finer granularity than 0.25µs if there is a requirement from other work group.

Proposal 3	RAN2 work first focus on how to obtain available time information from gNB.

Proposal 4	Re-using similar solution on accurate time granularity supported in LTE HRLLC for IIoT, i.e. improve sync accuracy in SIB9.

Proposal 5	RAN2 send a LS to SA1/SA2 on whether multiple clock synchronization accuracy requirement is needed in TSN system.

References

RP-182090, Revised SID: Study on NR Industrial Internet of Things (IoT), 3GPP RAN#81, Gold Coast, Australia, September 10-13, 2018

TR 22.804, Study on Communication for Automation in Vertical Domains

R2-1816043, LS on TSN requirements evaluation, contact: Nokia

RAN2#104 Chinaman notes

R1-1901470, Reply LS on TSN requirements evaluation, contact: Nokia

R3-186706, Reply LS on TSN requirements evaluation, contact: Nokia