3GPP TSG-RAN WG2 Meeting #105	 					   R2-1900583

Athens, Greece, 25th February - 1st March 2019

Source:	Panasonic

Title:                       Cell Reselection Procedure for NR-U

Agenda Item:         11.2.2.1

Document for:	Discussion and Decision

# Introduction

In the previous meeting RAN2 agreed to increase the paging transmission opportunity per DRX cycle for the UE to receive the paging. The additional transmission opportunity can be provided in time domain by configuring an extended paging occasion (i.e. a paging window) or configuring multiple paging occasions to a UE. This was captured in TR 38.889 [1].

# Discussion

In RRC Idle state, a UE can acquire system information, monitor paging information, or performs random access procedure. Network sends paging message whenever it has DL data for the UE or it needs to change of system information. UE monitors paging message in a corresponding paging occasion. However, due to LBT mechanism in NR-U, there is no guarantee that the channel on the unlicensed cell is always available for gNB to send paging message to the UE on the UE’s paging occasion. As a result, UE may have a delay in receiving paging message in case LBT failure rate is higher in the serving cell. Similarly as paging, UE might not finish RACH procedure and acquisition of system information successfully in case the unlicensed channel is too busy.  Therefore, there is no point for UE to stay in a same serving cell for longer period of time.

Observation 1: UE may not be able to receive a paging message, acquire system information, or perform RACH procedure within a reasonable time when the unlicensed channel is too congested.

In NR, UE in RRC Connected reports RLF to upper layer when it misses SSB & CSI-RS or SSB (depending on the configuration) for N number of times, where the value of N is configured by the network. For NR-U, we think it could be useful to apply similar kind of mechanism for UE in RRC Idle. This allows UE to perform cell reselection faster in order to acquire system information, finish RACH procedure or receive paging message faster from the gNB. Therefore for NR-U, UE in RRC Idle should initiate the process of cell reselection once it misses DRS in the serving cell for the Mth time. Here we assume gNB transmits DRS (discovery reference signal) within a periodically occurring time window similarly as in LAA/eLAA.

Proposal 1: for NR-U, new condition to trigger cell resection is beneficial for UE to receive paging message, system information, or to perform RACH in RRC Idle mode.

Proposal 2: for NR-U, UE should change the serving cell to neighbouring cell in different frequency once it misses DRS in the serving cell for the Nth time.

# Conclusions

In this contribution, we discussed LBT impact for UE in RRC Idle. Additionally, we ask RAN2 to discuss the following observation and proposal:

Observation 1: Observation 1: UE may not be able to receive a paging message, acquire system information, or perform RACH procedure within a reasonable time when the unlicensed channel is too congested.

Proposal 1: for NR-U, new condition to trigger cell resection is beneficial for UE to receive paging message, system information, or to perform RACH in RRC Idle mode.

Proposal 2: for NR-U, UE should change the serving cell to neighbouring cell in different frequency once it misses DRS in the serving cell for the Nth time.

# References

[1] TS 38.889, “Study on NR-based access to unlicensed spectrum”, Rel-15, 3GPP technical report.