3GPP TSG-RAN WG2#105	Tdoc R2-1900640

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	10.4.2.2

Source:	Ericsson

Title:	Remaining issues for Inter System Intra RAT Handover

Document for:	Discussion, Decision

# Introduction

During Rel-15 inter-RAT handover between NR and E-UTRA as well as inter-system intra-RAT handover between E-UTRA/EPC and E-UTRA/5GC. This contribution discusses a few leftover issues for intra-system intra-RAT handover.

# Discussion

When a UE is instructed to perform a handover from E-UTRA/EPC to E-UTRA/5GC or vice versa, the target node prepares an RRCConnectionReconfiguration message which it sends to the source node via CN. The target node in turn prepares an MobilityFromEUTRACommand with the field targetRAT-MessageContainer containing the RRCConnectionReconfiguration message and the field targetRAT-Type set to eutra. The source node then sends the MobilityFromEUTRACommand message to the UE.

MobilityFromEUTRACommand-r9-IEs ::=	SEQUENCE {

cs-FallbackIndicator				BOOLEAN,

purpose								CHOICE{

handover							Handover,

cellChangeOrder						CellChangeOrder,

e-CSFB-r9							E-CSFB-r9,

...

},

nonCriticalExtension				MobilityFromEUTRACommand-v930-IEs	OPTIONAL

}

Handover ::=						SEQUENCE {

targetRAT-Type						ENUMERATED {

utra, geran, cdma2000-1XRTT, cdma2000-HRPD,

nr, eutra, spare2, spare1, ...},

targetRAT-MessageContainer			OCTET STRING,

nas-SecurityParamFromEUTRA			OCTET STRING (SIZE (1))	OPTIONAL,	-- Cond UTRAGERANEPC

systemInformation					SI-OrPSI-GERAN				OPTIONAL	-- Cond PSHO

}

Upon reception of the MobilityFromEUTRACommand message the UE initiates the procedures in section 5.4.3.3 i.e.:

5.4.3.3	Reception of the MobilityFromEUTRACommand by the UE

<< skipped parts>>>

1>	if the MobilityFromEUTRACommand message includes the purpose set to handover:

<< skipped parts>>

2>	else if the targetRAT-Type is set to eutra:

3>	consider inter-system mobility as initiated towards E-UTRA;

3>	forward the nas-SecurityParamFromEUTRA to the upper layers, if included;

3>	access the target cell indicated in the inter-RAT message in accordance with section 5.4.2.3;

If the UE receives an MobilityFromEUTRACommand message with targetRAT-Type set to eutra, it will perform procedures for reception of RRCConnectionReconfiguration message in section 5.4.2 (Handover to E-UTRA)

In section 5.4.2.3, the UE will process the RRCConnectionReconfiguration message according to

5.4.2.3	Reception of the RRCConnectionReconfiguration by the UE

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

<<skipped parts>>

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

Since the inter-system, intra-RAT handover is modelled as both a Mobility from E-UTRA and a Handover to E-UTRA, if the UE fails, it may trigger the failure handling in both sections.

If a UE fails during an inter-system, intra-RAT handover, it will trigger failure handling in both section 5.4.2 (Handover to E-UTRA) and section 5.4.3 (Mobility from E-UTRA)

The UE may fail with the embedded RRCConnectionReconfiguration message according to two separate failure cases:

- Reconfiguration failure

- T304 expiry

The failure handling in this case is according to section 5.4.2.4 and 5.4.2.5:

5.4.2.4	Reconfiguration failure

The UE shall:

1>	if the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message:

2>	if the source RAT is E-UTRA:

3>	perform the actions as specified in 5.3.5.5;

5.4.2.5	T304 expiry (handover to E-UTRA failure)

The UE shall:

1>	upon T304 expiry (handover to E-UTRA failure):

2>	if the source RAT is E-UTRA:

3>	perform the actions as specified in 5.3.5.6;

If the UE receives an RRCConnectionReconfiguration inside the MobilityFromEUTRACommand message, it can fail due to:

  - Inability to comply with RRCConnectionReconfiguration message (according to section 5.4.2.4)

  - Expiry of timer T304 in MobilityControlInfo in RRCConnectionReconfiguration message (according to section 5.4.2.5)

In either case, the UE will according to section 5.3.5.5 or 5.3.5.6 trigger re-establishment:

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

If the UE fails due to failure of the embedded RRCConnectionReconfiguration message in the MobilityFromEUTRACommand, it will trigger a re-establishment

Since the RRCConnectionReconfiguration procedure was triggered by the Mobility from E-UTRA procedure (section 5.4.3), if the UE fails in the target, the failure handling in section 5.4.3.5 will also be triggered. This section in addition checks:

1>	if T304 configured in the MobilityFromEUTRACommand message expires (mobility from E-UTRA failure); or

1>	if the UE does not succeed in establishing the connection to the target radio access technology; or

1>	if the UE is unable to comply with (part of) the configuration included in the MobilityFromEUTRACommand message; or

1>	if there is a protocol error in the inter RAT information included in the MobilityFromEUTRACommand message, causing the UE to fail the procedure according to the specifications applicable for the target RAT (i.e. according to sub-clause 5.3.5.6 if the targetRAT-Type in the received MobilityFromEUTRACommand is set to eutra):

The first condition (T304 configured in MobilityFromEUTRACommand expires) is not applicable, since it is only configured for cell change order and not for handover. However, the other conditions will apply to inter-system intra-RAT handover.

For inter-system, intra-RAT handover, failure handling will be triggered in section 5.4.3.5 (Mobility from E-UTRA failure) for:

  - Failure to establish connection to target RAT

  - Inability to comply with (part of) configurations in MobilityFromEUTRACommand message

  - Protocol error in inter RAT information in MobilityFromEUTRACommand when targetRAT-Type is set to eutra

If failure is trigger in section 5.4.3.5 (Mobility from E-UTRA failure), the UE will:

…

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, mac-MainConfig and sps-Config;

2>	initiate the connection re-establishment procedure as specified in 5.3.7;

Since the UE would already have triggered reestablishment if failure occurred with the embedded RRCConnectionReconfiguration message, the UE would trigger re-establishment once again.

If the UE fails with the embedded RRCConnectionReconfiguration message in the MobilityFromEUTRACommand it will perform failure procedures in both section 5.4.3 and 5.4.2, i.e. perform re-establishment twice

To avoid this extraneous re-establishment, the procedures for Mobility from E-UTRA failure need to be updated to not trigger re-establishment if the error occurs due to the embedded RRCConnectionReconfiguration message in the MobilityFromEUTRACommand.

Section 5.4.3.5 (Mobility from E-UTRA failure) shall not trigger re-establishment if the failure is due to failure of the embedded RRCConnectionReconfiguration message in case of inter-system intra-RAT handover

We have provided a CR in R2-1900639 [] detailing the required updates to the procedures.

Adopt the CR R2-1900639

# Conclusion

In section 2 we made the following observations:

Observation 1	If the UE receives an MobilityFromEUTRACommand message with targetRAT-Type set to eutra, it will perform procedures for reception of RRCConnectionReconfiguration message in section 5.4.2 (Handover to E-UTRA)

Observation 2	If a UE fails due during an inter-system, intra-RAT handover, it will trigger failure handling in both section 5.4.2 (Handover to E-UTRA) and section 5.4.3 (Mobility from E-UTRA)

Observation 3	If the UE receives an RRCConnectionReconfiguration inside the MobilityFromEUTRACommand message, it can fail due to:

a	Inability to comply with RRCConnectionReconfiguration message (according to section 5.4.2.4)

b.	Expiry of timer T304 in MobilityControlInfo in RRCConnectionReconfiguration message (according to section 5.4.2.5)

Observation 4	If the UE fails due to failure of the embedded RRCConnectionReconfiguration message in the MobilityFromEUTRACommand, it will trigger a re-establishment

Observation 5	For inter-system, intra-RAT handover, failure handling will be triggered in section 5.4.3.5 (Mobility from E-UTRA failure) for:

a.	Failure to establish connection to target RAT

b.	Inability to comply with (part of) configurations in MobilityFromEUTRACommand message

c.	Protocol error in inter RAT information in MobilityFromEUTRACommand when targetRAT-Type is set to eutra

Observation 6	If the UE fails with the embedded RRCConnectionReconfiguration message in the MobilityFromEUTRACommand it will perform failure procedures in both section 5.4.3 and 5.4.2, i.e. perform re-establishment twice

Based on the discussion in section 2 we propose the following:

Proposal 1	Section 5.4.3.5 (Mobility from E-UTRA failure) shall not trigger re-establishment if the failure is due to failure of the embedded RRCConnectionReconfiguration message in case of inter-system intra-RAT handover

Proposal 2	Adopt the CR R2-1900639

# References

R2-1900639, Remaining issues for Inter System Intra RAT Handover, Ericsson, 3GPP WG2#105, Athens, Greece, 25th February – 1st March 2019