3GPP TSG-RAN2 Meeting #105	R2-1900688

# Athens, Greece, 25 Feb – 1 Mar 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3853 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Release of NGEN-DC at RRC Resume |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current procedure for RRC Resume instructs the UE to release the EN-DC configurations upon initiation of the procedure if the UE is resuming from a suspended RRC Connection. As it has been agreed that NGEN-DC will follow the same principles, the procedures must be updated to release the NGEN-DC configurations even when the UE is resuming from RRC_INACTIVE. In addition, the section for EN-DC release in TS 38.331 have been extended in email discussion 104#64 to include all affected MR-DC options. The current procedure have first a check whether the UE resumes from suspended RRC Connection or RRC_INACTIVE, and then a second check whether the UE resumes from suspended RRC Connection. Since the MR-DC release applies to both these scenarios, the second check can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified procedures for initiation of RRC Resume to release MR-DC configurations when resuming from both a suspended RRC Connection as well as from RRC_INACTIVE. Impact analysis Impacted functionality: - (NG)EN-DC during initiation of RRC Conection Resume Inter-operability: If the network is implemented according to this CR while the UE is not, the UE may keep SRB3 and SCG configurations in NGEN-DC, while the network will have assumed that the UE has released these configurations, which might lead to leading to inabilities to comply with messages. If the UE is implemented according to this CR while the network is not, the network may keep SRB3 and the SCG configurations in NGEN-DC, while the UE has released these configurations, which might lead to leading to inabilities to comply with messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will not release the NGEN-DC configurations which will then be resumed when the UE receives the RRCConnectionResume message, even though these configurations cannot be reconfigured in that message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3 RRC connection establishment 5.3.3.2 Initiation |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 5.3.3	RRC connection establishment

#### 5.3.3.2	Initiation

<< skipped parts >>

START OF CHANGES

Except for NB-IoT, upon initiating the procedure, if connected to EPC or 5GC, the UE shall:

1>	if the UE is resuming an RRC connection from a suspended RRC connection or from RRC_INACTIVE:

2>	if the UE was configured with (NG)EN-DC:

3>	perform MR-DC release, as specified in TS 38.331 [82], clause 5.3.5.10;

2>	release the MCG SCell(s), if configured, in accordance with 5.3.10.3a;

END OF CHANGES