3GPP TSG-RAN WG2 #105	R2-1900689

Athens, Greece, 25th Feb – 1st March 2019

Agenda item:	10.4.1.3.6

Source:	Ericsson

Title:	State of MCG SCells on resume

Document for:	Discussion and Decision

# 1	Introduction

In LTE, when a UE resumes from dormant state, the MCG SCells and SCG configurations are released. In NR, however, only the SCG is released and the MCG SCells are kept (unless the network explicitly releases them within the optional master cell group configuration it can include in the RRCResume message). Currently, it is not clear on what state (activated or deactivated) the MCG SCells start with when the UE resumes. This contribution discusses this aspect.

# 2	Discussion

When a UE is suspended, it saves the UE AS context. This is shown in the RRCRelease procedure below:

**********************************************************************************************

5.3.8.3	Reception of the RRCRelease by the UE

The UE shall:

<<skipped part>>

1>	if the RRCRelease includes suspendConfig:

<<skipped part>>

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

<<skipped part>>

2>	else:

3>	store in the UE Inactive AS Context the received suspendConfig, all current parameters configured with RRCReconfiguration or RRCResume, the current KgNB and KRRCint keys, the ROHC state, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell;

<<skipped part>>

**********************************************************************************************

As shown above, the current state of the RRC configuration with RRCReconfiguration or RRCResume are stored in the AS context. Hence, MCG SCell configurations, being part of the RRC configuration, are stored upon suspension.

When a UE is suspended, the MCG SCell configuration are stored in the UE Inactive AS context.

When the UE initiates the resume procedure by sending RRCResumeRequest, it restores the UE Inactive AS context except the cell group configuration, as shown below:

**********************************************************************************************

5.3.13.3	Actions related to transmission of RRCResumeRequest or RRCResumeRequest1 message

<<skipped parts>>

1>	restore the RRC configuration and the KgNB and KRRCint keys from the UE Inactive AS context except the cellGroupConfig and pdcp-Config;

<<skipped parts>>

**********************************************************************************************

However, upon the reception of RRCResume from the network, the UE will restore the MCG SCells from the UE Inactive AS context, unless the RRCResume includes a full configuration flag, as shown below:

**********************************************************************************************

5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

1>	stop timer T319;

1>	stop timer T380, if running;

1>	if the RRCResume includes the fullConfig:

2>	perform the full configuration procedure as specified in 5.3.5.11;

1>	else:

2>	if drb-ContinueROHC is included:

3>	indicate to lower layers that drb-ContinueROHC is configured;

2>	restore the masterCellGroup and pdcp-Config from the UE Inactive AS context;

1>	discard the UE Inactive AS context except the ran-NotificationAreaInfo;

- if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

<<skipped parts>>

**********************************************************************************************

When a UE resumes, the MCG SCell configuration are restored from the UE Inactive AS context, unless the RRCResume contains the fullConfig flag.

It is currently ambiguous in NR which state (deactivated/activated) each restored SCell would start after receiving an RRCResume where at least one SCell is maintained (as the state of the SCells is a MAC level information and is not part of the UE Inactive AS context). The problem is not relevant in case the network releases all SCells in RRCResume by removing all SCells in the masterCellGroup, or for SCells that are being added in the masterCellGroup, since the specifications define that newly added SCells shall start with deactivated state, as shown below (and they have to be activated via MAC CE later).

**********************************************************************************************

5.3.5.5.9	SCell Addition/Modification

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated;

2>	configure lower layers to consider the SCell to be in deactivated state;

Editor's Note: FFS Check automatic measurement handling for SCells.

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the sCellConfigDedicated.

**********************************************************************************************

In the current specs, the state of the MCG SCells on resumption from suspended state is ambiguous, as the state of the SCells is not part of the UE Inactive AS context.

One possibility is to consider enhancements like those agreed in the LTE euCA WI, where it is possible to configure the state of the SCells via RRC (e.g. RRCConnectionReconfiguration), also in the RRCResume. However, considering this is the last RAN WG meeting for NR rel-15 and we are starting to discuss the rel-16 WI on CA/DC enhancements where the LTE euCA work will be adopted whenever appropriate, a more practical approach for rel-15 is to hard code the MCG SCells states on resume in the specs so that both the UE and network will resume with the same SCell state. There are two options:

- Put all the SCells to be in activated state on resumption

- Put all the SCells to be in deactivated state on resumption

One reason for the adoption of starting SCells in deactivated state is to prevent unnecessary UE power consumption, and to activate the SCells only when the network is sure the UE can benefit from activating the SCells (e.g. based on measurements, traffic volume, etc.). The same considerations apply in the context of resume in rel-15, where the network doesn’t have measurements yet to decide the SCell state. Thus, we propose:

For NR rel-15, when the UE resumes from suspended state, all MCG SCells are put in deactivated state.

In [1], we have provided a CR to capture this.

Agree on the CR provided in R2-1900690

# 3	Conclusions

In section 2 we discussed state of MCG SCells on RRC Resume and made the following observations:

Observation 1	When a UE gets suspended, the MCG SCell configuration are stored in the UE Inactive AS context.

Observation 2	When a UE resumes, the MCG SCell configuration are restored from the UE Inactive AS context, unless the RRCResume contains the fullConfig flag.

Observation 3	In the current specs, the state of the MCG SCells on resumption from suspended state is ambiguous, as the state of the SCells is not part of the UE Inactive AS context.

Based on the discussion in section 2 we propose the following:

Proposal 1	For NR rel-15, when the UE resumes from suspended state, all MCG SCells are put in deactivated state.

Proposal 2	Agree on the CR provided in R2-1900690.

# References

[1] R2-1900690, “State of MCG SCells on resume (CR to 38.331)”, Ericsson, RAN2 #105