3GPP TSG-RAN WG2 Meeting #105	R2-1900698

# Athens, Greece, 25 Feb – 1 Mar 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | CRNum | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuration of SCG during RRC Resume in (NG)EN-DC (36.331) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2019-02-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | If a UE can be configured with (NG)EN-DC already in the RRC Resume message, the UE can much faster utilize the SCG resources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5.3.3.4a Reception of the RRCConnectionResume by the UE - Added procedures to configure NR SCG 6.2.2 - RRCConnectionResume message, - Added field nr-SecondaryCellGroupConfig with field description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration of (NG)EN-DC during RRC Resume will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.4a Reception of the RRCConnectionResume by the UE 6.2.2 - RRCConnectionResume |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

### 5.3.3	RRC connection establishment

#### 5.3.3.4a	Reception of the RRCConnectionResume by the UE

The UE shall:

1>	stop timer T300;

1>	stop timer T309, if running, for all access categories;

1>	stop T380 if running;

1>	except if the RRCConnectionResume is received in response to an RRCConnectionResumeRequest for EDT:

2>	if resuming an RRC connection from a suspended RRC connection:

3>	restore the PDCP state and re-establish PDCP entities for SRB2, if configured with E-UTRA PDCP, and for all DRBs that are configured with E-UTRA PDCP;

3>	if drb-ContinueROHC is included:

4>	indicate to lower layers that stored UE AS context is used and that drb-ContinueROHC is configured;

4>	continue the header compression protocol context for the DRBs configured with the header compression protocol;

3>	else:

4>	indicate to lower layers that stored UE AS context is used;

4>	reset the header compression protocol context for the DRBs configured with the header compression protocol;

3>	discard the stored UE AS context and resumeIdentity;

2>	else if the RRCConnectionResume message includes the fullConfig (for resuming an RRC connection from RRC_INACTIVE):

3>	perform the radio configuration procedure as specified in 5.3.5.8;

2>	else (for resuming an RRC connection from RRC_INACTIVE)

3>	re-establish PDCP entities for SRB2 and all DRBs;

3>	restore the physical layer configuration, the MAC configuration, the RLC configuration and the PDCP configuration from the stored UE Inactive AS context;

3>	if drb-ContinueROHC is included:

4>	indicate to lower layers that drb-ContinueROHC is configured;

3>	discard the stored UE Inactive AS context, except ran-NotificationAreaInfo;

1>	else:

2>	discard the stored UE AS context and resumeIdentity;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

NOTE:	When performing the radio resource configuration procedure, for the physical layer configuration and the MAC Main configuration, the restored RRC configuration from the stored UE AS context is used as basis for the reconfiguration.

1>	if the received RRCConnectionResume message includes the sk-Counter:

2>	perform key update procedure as specified in TS 38.331 [82], clause 5.3.5.8;

1>	if the received RRCConnectionResume message includes the nr-RadioBearerConfig1:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	if the received RRCConnectionResume message includes the nr-RadioBearerConfig2:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	if the received RRCConnectionResume message includes the nr-SecondaryCellGroupConfig:

2>	perform NR RRC Reconfiguration as specified in TS 38.331 [82], clause 5.3.5.3.

1>	except if the RRCConnectionResume is received in response to an RRCConnectionResumeRequest for EDT:

2>	resume SRB2 and all DRBs, if any, including RBs configured with NR PDCP;

END OF FIRST CHANGE

START OF SECOND CHANGE

### 6.2.2	Message definitions

#### –	RRCConnectionResume

The RRCConnectionResume message is used to resume the suspended RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

RRCConnectionResume message

-- ASN1START

RRCConnectionResume-r13 ::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

rrcConnectionResume-r13				RRCConnectionResume-r13-IEs,

spare3								NULL,

spare2								NULL,

spare1								NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionResume-r13-IEs ::=		SEQUENCE {

radioResourceConfigDedicated-r13		RadioResourceConfigDedicated	OPTIONAL,	-- Need ON

nextHopChainingCount-r13				NextHopChainingCount,

measConfig-r13							MeasConfig						OPTIONAL,	-- Need ON

antennaInfoDedicatedPCell-r13			AntennaInfoDedicated-v10i0		OPTIONAL,	-- Need ON

drb-ContinueROHC-r13					ENUMERATED {true}				OPTIONAL,	-- Need OP

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

rrcConnectionResume-v1430-IEs			RRCConnectionResume-v1430-IEs	OPTIONAL

}

RRCConnectionResume-v1430-IEs ::= SEQUENCE {

otherConfig-r14						OtherConfig-r9					OPTIONAL,		-- Need ON

rrcConnectionResume-v1510-IEs		RRCConnectionResume-v1510-IEs	OPTIONAL

}

RRCConnectionResume-v1510-IEs ::= SEQUENCE {

sk-Counter-r15						INTEGER (0.. 65535)			OPTIONAL,	-- Need ON

nr-RadioBearerConfig1-r15			OCTET STRING				OPTIONAL,	-- Need ON

nr-RadioBearerConfig2-r15			OCTET STRING				OPTIONAL,	-- Need ON

nonCriticalExtension					RRCConnectionResume-v1530-IEs			OPTIONAL

}

RRCConnectionResume-v1530-IEs ::= SEQUENCE {

fullConfig-r15						ENUMERATED {true}			OPTIONAL, 	-- Need ON

nonCriticalExtension					RRCConnectionResume-v16xx-IEs					OPTIONAL

}

RRCConnectionResume-v16xx-IEs ::= SEQUENCE {

nr-secondaryCellGroupConfig				OCTET STRING				OPTIONAL,	-- Need ON

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- ASN1STOP

RRCConnectionResume field descriptions

drb-ContinueROHC This field indicates whether to continue or reset the header compression protocol context for the DRBs configured with the header compression protocol. Presence of the field indicates that the header compression protocol context continues while absence indicates that the header compression protocol context is reset.

fullConfig Indicates that the full configuration option is applicable for the RRCResume message.

nr-RadioBearerConfig1, nr-RadioBearerConfig2 Includes the NR RadioBearerConfig IE as specified in TS 38.331 [82]. The field includes the configuration of RBs configured with NR PDCP.

nr-SecondaryCellGroupConfig Includes the NR RRCReconfiguration message as specified in TS 38.331 [82]. In this version of the specification, the NR RRC message only includes fields secondaryCellGroup and/ or measConfig.

sk-Counter A one-shot counter used upon initial configuration of security for EN-DC as well as upon refresh of S-KgNB. E-UTRAN provides this field upon establishment of first SN-terminated bearer using S-KgNB.

END OF SECOND CHANGE