3GPP TSG-RAN WG2 #105	R2-1900734

Athens, Greece, 25th February – 1st March

Agenda Item:	12.1.2

Source:	Ericsson

Title:	MT early data after preamble

Document for:	Discussion, Decision

# 1	Introduction

In the Rel-16 WIDs for both LTE-M [1] and NB-IoT [2], one of the objectives is to specify support for MT EDT:

Improved DL transmission efficiency and/or UE power consumption:

- Specify support for mobile-terminated (MT) early data transmission (EDT) [RAN2, RAN3]

MT EDT was discussed in recent RAN2#103bis and RAN2#104 meetings with following agreements [3]:

- RAN2 intends to support MT-initiated EDT for both CP and UP solutions.

- The intention to use MT-EDT is for user data, i.e. not for NAS signalling.

- MT EDT are evaluated at least based on battery life, network resource efficiency, security, reliability and potential impact on core network.

- MT-EDT is intended for DL data which can be transmitted in one transport block.

- Use cases that require DL data transmission with or without UL data transmission as a response should be supported for MT-EDT.

In addition, different options for MD-EDT are discussed in Email discussion [104#49][eMTC & NB-IoT R16] [4]

[104#49][eMTC & NB-IoT R16] MT MDT (Huawei)

  - To evaluate the solution options based on the agreed criteria

  - Intended outcome: Email discussion report to next meeting

  - Deadline: Thursday 2019-02-07

As indicated in our replies in the email discussion [104#49], our first preference is DL data in Msg4 option and then DL data after preamble. This contribution discusses in detail our UP and CP solutions for DL data after preamble. Detailed solutions for DL data in Msg4 are discussed in [5]. A draft LS to other WGs asking relevant questions for the two MT-EDT options is provided in [6].

# 2	Discussion

## 2.1	Common Aspects

This section discusses some aspects applicable to both CP and MT UP-EDT. Some aspects specific to UP and CP solutions will be discussed in next section.

At a high-level description (see also Figure 1 and Figure 2), the procedure to enable MT DL data transmission after preamble can be summarized in some steps as below:

- The UE is paged with a paging message that contains a dedicated contention free preamble (CFRA).

- The UE responds to the paging message with PRACH transmission using the CFRA preamble to the eNB from which the paging message is received.

- The eNB, upon reception of the CFRA preamble, communicates with the core network (MME/S-GW/SCEF) to retrieve the DL data for the UE. In case of UP solution, the eNB may need to obtain UE context from another network node where the context is stored for protection of DL data transmission.

- The eNB then transmits the DL data in response to the PRACH transmission to the UE.

- In case the reliable delivery of MT DL data is needed, the UE needs to provide the network with a security protected response to the DL data transmission.

In MT calls, the MME is notified about the arrival of DL data and then pages the UE in both UP and CP solutions. Thus, the initiation of MT EDT can be determined by MME. In addition, as agreed in recent RAN2 meeting, MT EDT is intended for DL data that can be transmitted in one transport block. It is proposed that MME determines initiation of MT-EDT based on DL data size.

MME determines initiation of MT-EDT based on DL data size.

For transmission of DL data in response to the CFRA preamble in Msg1, the legacy Random Access Response (in Msg2) cannot be directly used since it was not designed for carrying DL data to UE. Instead, as detailed in the following, Msg2 should combine DL data, timing advance and other required information.

For example, as agreed in recent meeting, there should be a possibility to send UL feedback or potential UL data transmission in response to the DL transmission containing DL data. Therefore, similar to the legacy random access, timing advance command is needed for the UE to obtain synchronization with network. In addition to timing advance command, the UE also needs UL grant at least in case of UL application data in response to DL data transmission. This UL grant can also be used for UL feedback in case of no UL application data.

UE needs timing advance command for any UL ACK or UL data transmission in response to DL data. UE also needs UL grant in case of UL transmission in response to DL data transmission.

To reduce the signaling overhead and minimize latency for UE, it is proposed to combine the timing advance, UL grant in the DL transmission containing or scheduling DL data.

Timing advance command and UL grant are provided together with DL transmission containing or scheduling MT data.

In addition, in legacy RA-RNTI used to scramble (M)PDCCH for Random Access Response message is associated with the PRACH in which the CFRA preamble is transmitted. Thus, this should not be used to schedule DL data transmission for a specific UE.

RA-RNTI cannot be used to schedule DL transmission containing data for a specific UE.

A UE specific RNTI (e.g. MT-EDT RNTI or E-RNTI) should be introduced to schedule the DL transmission. The network can provide an RNTI to the UE via paging message together with the CFRA preamble.

A UE-specific RNTI is provided to the UE in paging message for scheduling DL transmission containing data.

There is a security aspect with using CFRA preamble as a means to authenticate UE for DL data transmission: Any unintended UE in the same tracking area receiving the paging message containing information about the CFRA preamble can respond the paging with a CFRA PRACH transmission as if it is the intended UE.

Unintended UEs can respond a paging message using the dedicated CFRA preamble.

It is therefore proposed to protect the CFRA preamble sent in paging message so that unintended UEs cannot simply use the received preamble in the reply PRACH transmission. A detailed protection mechanism can be considered after this principle has been agreed. This aspect may need to be analyzed in SA3.

In Rel15 MO-EDT, DL data is available at MME in CP solution or eNB in UP solution after the eNB receives Msg3 from the UE. However, in this MT EDT option, it is necessary to have DL data at MME/eNB before the eNB sends the DL transmission containing or scheduling DL data, i.e., before eNB receives Msg3. This requires the establishment/resumption of the UE-associated logical S1-connection significantly earlier than in legacy so that S-GW/SCEF can forward DL data to MME/eNB before DL transmission.

MT DL data can only be forwarded to MME/eNB if there exists a UE-associated logical S1-connection between eNB and MME in CP/MT UP-EDT solution.

Thus, to have DL data available for MT EDT, upon reception of Msg1, the eNB should establish/resume S1-connection in CP and UP solution, respectively.

S1-connection is established/resumed upon reception of Msg1 to have the DL data available for MT-EDT.

Currently, once the preamble is transmitted, the UE monitors PDCCH for random access response(s) in the RAR window, whose length is defined by ra-ResponseWindowSize. In MT EDT, it takes time for the eNB to get DL data from S-GW/SCEF before it can schedule DL transmission in response to the PRACH. In addition, in UP solution, the eNB may need to obtain the UE context before the DL transmission. Thus, the value range for the legacy  RAR window size may not be applicable to the case for MT EDT.

The RAR window size may not be sufficient for the eNB to be able to respond a CFRA PRACH transmission.

Therefore, it is proposed to consider having a suitable response window dedicated to MT EDT. For example, the value range for the legacy ra-ResponseWindowSize can be extended for MT EDT only. Similar to ra-ResponseWindowSize, this is the duration in which a MT EDT can wait for a DL response to the transmitted CFRA preamble.

For the network to know that its DL data was successfully delivered to the intended UE, there should be a UL feedback in response to DL transmission. A HARQ ACK over PUCCH is not sufficient since such feedback can be sent by any UEs, i.e., with no authentication. Thus, an integrity protected UL transmission in response to DL data may be needed.

For the network to know if DL data was successfully sent to the intended UE, secure acknowledgment from the UE is needed.

UE shall send secure acknowledgment with or without UL data in response to MT DL data transmission.

Similar to the discussion in MT early data in Msg4 in [4], DL data may arrive after the paging procedure resulting in the situation that network needs to send more DL data transmission to the UE after the MT EDT DL transmission. It would be beneficial to have an updated information about DL data condition before sending DL data. For this MT EDT option, RAN2 should send an LS to other WGs asking how to inform MME and eNB the information about pending DL data before the DL transmission containing DL data, e.g., as in [6] .

It is beneficial if the MME/eNB can be aware of DL data condition before DL transmission containing MT data to determine whether to send the UE back to RRC_IDLE.

RAN2 send an LS to RAN3/SA2/CT1 asking how to inform the MME and/or eNB about DL data condition before it sends DL transmission containing DL data.

## 2.1	MT UP-EDT

Figure 1 shows an example of signaling flow of the proposed MT UP-EDT solution. One UP-specific aspect is the context handling. Different from legacy resume procedure in which UE sends a resume request that triggers the context retrieval at the target eNB. In this MT EDT option, eNB still needs UE context for at least protection of data before DL data transmission. However, the information needed for retrieval of UE context is not available upon reception of the PRACH transmission.

In MT UP-EDT, eNB needs to obtain UE context before DL data transmission.

There are different ways for context retrieval. Considering X2 context retrieval as a baseline, it is possible that the source eNB, MME, and target eNB communicate and exchange information needed for context retrieval during suspend procedure and S1 paging procedure.

In MT UP-EDT, to allow for target eNB to obtain UE context, it is proposed that

  - Source eNB provides MME with information needed for retrieval of UE context at suspension

  - MME provides target eNB in S1 paging message with the information needed for retrieval of UE context

  - Upon reception of Msg1 with CFRA PRACH, target eNB obtains context from source eNB using the information provided by MME.

Figure 1: Example of signaling flow for MT UP-EDT in case of resume to a new eNB (s_eNB: source eNB, t_eNB: target eNB)

The information needed for retrieval of UE context can be resumeID to locate the UE context and security information such as an authentication code with same functionality as the shortResumeMAC-I in Msg3 in legacy, i.e., to allow the source eNB to trust the target eNB before providing UE context. However, to avoid revealing resumeID to unintended eNBs as well as avoiding the need for the security information, the UE context can be identified in another way, for example, using eNB UE S1AP ID allocated at source eNB. That is, the MME provides the target eNB with the eNB UE S1AP ID received from the source eNB and the ID of source eNB itself instead of resumeID and the security information. Note that, in this case, the target eNB needs eNB ID of the source for performing context retrieval. This approach also does not require any changes at suspension.

In MT UP-EDT, consider following options for the information needed for context retrieval

  - resumeID and security information that allows the source eNB to trust target eNB

  - eNB UE S1AP ID and source eNB ID.

The X2 based context fetch has some aspects to be considered. The MME would provide identities of UE to multiple eNBs before knowing which is the intended one, i.e., the one with UE being camping on. Another aspect is the overhead of exchanging whole UE context, while maybe part of it is enough for MT EDT transmission.

In addition to retrieval of full UE context via X2 interface, another way is to define a temporary context which is needed only for protection of DL data in MT-EDT at source eNB or MME, which is made known at the UE as well. Then, the MME provides such context to the target eNB for performing MT-EDT. This is to avoid the need for context retrieval procedure, shortening the time the eNB needs to send the DL data in response to a PRACH transmission.

In MT UP-EDT, to avoid overhead from context retrieval, temporary context can be defined and provided to target eNB for protection of MT-EDT data.

In MT UP-EDT, there is no need for an explicit indication for MT EDT. This is because the reception of the UE ID with or without security information in the S1 paging described above can already serve as an indication for MT UP-EDT.

In MT UP-EDT, S1 paging message with information for retrieval of UE context can be used as an indication for eNB to initiate MT-EDT.

## 2.2	MT CP-EDT

Figure 2 shows an example of signaling flow of the proposed MT CP-EDT solution. This section discusses some CP-specific aspects.

In legacy, the eNB triggers the S1-connection establishment upon reception of an RRC message from the UE with a UL NAS-PDU. However, in this MT CP-EDT option, without receiving an RRC message from the UE, the eNB still needs to establish S1-connection for having DL data from S-GW/SCEF before it can send Dl data to the UE. It is proposed that upon reception of CFRA preamble, the eNB establishes S1-connection.

In MT CP-EDT, the eNB triggers the establishment of S1-connection upon reception of CFRA preamble.

It is proposed that RAN2 should send an LS and ask RAN3 how this can be done.

RAN2 send an LS to RAN3 asking how eNB can initiate S1-connection establishment upon reception of PRACH transmission.

Figure 2: Example of signaling flow for MT CP-EDT

Different from MT UP-EDT, the indication for initiation of MT EDT should be explicit in the S1 paging message (see Figure 2).

In MT CP-EDT, MME sends an indication in S1 paging message for initiation of MT-EDT.

# Conclusion

In the previous sections we made the following observations:

Observation 1	UE needs timing advance command for any UL ACK or UL data transmission in response to DL data. UE also needs UL grant in case of UL transmission in response to DL data transmission.

Observation 2	RA-RNTI cannot be used to schedule DL transmission containing data for a specific UE.

Observation 3	Unintended UEs can respond a paging message using the dedicated CFRA preamble.

Observation 4	MT DL data can only be forwarded to MME/eNB if there exists a UE-associated logical S1-connection between eNB and MME in CP/MT UP-EDT solution.

Observation 5	The RAR window size may not be sufficient for the eNB to be able to respond a CFRA PRACH transmission.

Observation 6	For the network to know if DL data was successfully sent to the intended UE, secure acknowledgment from the UE is needed.

Observation 7	It is beneficial if the MME/eNB can be aware of DL data condition before DL transmission containing MT data to determine whether to send the UE back to RRC_IDLE.

Observation 8	In MT UP-EDT, eNB needs to obtain UE context before DL data transmission.

Observation 9	In MT UP-EDT, to avoid overhead from context retrieval, temporary context can be defined and provided to target eNB for protection of MT-EDT data.

Observation 10	In MT UP-EDT, S1 paging message with information for retrieval of UE context can be used as an indication for eNB to initiate MT-EDT.

Based on the discussion in the previous sections we propose the following:

Proposal 1	MME determines initiation of MT-EDT based on DL data size.

Proposal 2	Timing advance command and UL grant are provided together with DL transmission containing or scheduling MT data.

Proposal 3	A UE-specific RNTI is provided to the UE in paging message for scheduling DL transmission containing data.

Proposal 4	S1-connection is established/resumed upon reception of Msg1 to have the DL data available for MT-EDT.

Proposal 5	UE shall send secure acknowledgment with or without UL data in response to MT DL data transmission.

Proposal 6	RAN2 send an LS to RAN3/SA2/CT1 asking how to inform the MME and/or eNB about DL data condition before it sends DL transmission containing DL data.

Proposal 7	In MT UP-EDT, to allow for target eNB to obtain UE context, it is proposed that

a.	Source eNB provides MME with information needed for retrieval of UE context at suspension

b.	MME provides target eNB in S1 paging message with the information needed for retrieval of UE context

c.	Upon reception of Msg1 with CFRA PRACH, target eNB obtains context from source eNB using the information provided by MME.

Proposal 8	In MT UP-EDT, consider following options for the information needed for context retrieval

a.	resumeID and security information that allows the source eNB to trust target eNB

b.	eNB UE S1AP ID and source eNB ID.

Proposal 9	In MT CP-EDT, the eNB triggers the establishment of S1-connection upon reception of CFRA preamble.

Proposal 10	RAN2 send an LS to RAN3 asking how eNB can initiate S1-connection establishment upon reception of PRACH transmission.

Proposal 11	In MT CP-EDT, MME sends an indication in S1 paging message for initiation of MT-EDT.

# References

RP-182891, “Revised WID: Additional MTC enhancements for LTE”, Qualcomm, RAN#82, Sorrento, Italy, December 2018

RP-182902, “WID revision: Additional enhancements for NB-IoT”, Huawei, RAN#82, Sorrento, Italy, December 2018

R2-1815744, “RAN2 agreements for Rel-16 NB-IoT and MTC after RAN2-103bis”, RAN2#103bis, October 2018

R2-1901143, “Report of email discussion [104#49][eMTC & NB-IoT R16] MT EDT”, RAN2#105, February 2019

R2- 1900733, “MT early data in Msg4”, source Ericsson, RAN2#105, February 2019.

R2-1900735, “Draft LS on Mobile-terminated Early Data Transmission” source Ericsson, RAN2#105, February 2019

S2-185713, “Introducing Early Data Transmission for Control Plane CIoT EPS optimization”, source Qualcomm Incorporated, Huawei, HiSilicon, SA2#127bis, May 2018