3GPP TSG-RAN WG2 #105	Tdoc R2-1900744

Athens, Greece, 25 Feb – 1 Mar 2019

Agenda Item:	12.1.12

Source:	Ericsson

Title:	LTE-M Connectivity to 5GC

Document for:	Discussion, Decision

# 1	Introduction

In revised WID [1] the following objective was added based on completed SA2 study:

Connection to 5GC: - Specify support for the following features [RAN2, RAN3] - Support of extended DRX in CM-IDLE - Support of extended DRX in CM-CONNECTED with RRC_INACTIVE (support of sleep cycles up to the NAS and SMS retransmission timers) - Support of EDT for Data over NAS and UP solution (see Note) - Support of restriction of use of Enhanced Coverage - Delivery of Expected UE Behaviour information to the RAN - Additional information in SIB to indicate supported CIoT features; indication of CIoT features supported by the UE in RRC Note: Based on the outcome of RAN2/SA2 liaison exchanges, UP solution to be supported for connection to 5G-CN may be later updated.

In addition, SA2 has sent an LS to RAN/RAN2/RAN3 with the following description and actions:

1. Overall Description: SA2 has completed the Study on Cellular IoT support and evolution for the 5G System (FS_CIoT_5G). The conclusions reached during the study are documented in TR 23.724. Furthermore, SA2 has agreed a work item to specify 5GS enhancements based on the study conclusions to enable Cellular IoT functionalities for 5GS capable devices that support WB-EUTRA (eMTC) or NB-IoT or both. SA2 would like to point out the following key RAN impacts resulting from the study (details can be found in TR 23.724): - Support of extended DRX in CM-IDLE - Support of extended DRX in CM-CONNECTED with RRC_INACTIVE (support of sleep cycles up to the NAS and SMS retransmission timers) - Support of EDT for Data over NAS and user-plane transmission - Support of Inter-UE QoS for data over NAS (resource prioritisation between different NB-IoT UEs) - Support of restriction of use of Enhanced Coverage - Delivery of Expected UE Behaviour information to the RAN - Additional information in SIB to indicate supported CIoT features; indication of CIoT features supported by the UE in RRC SA2 would like to kindly ask RAN working groups to take the above into account during their work on Cellular IoT. SA2 has also discussed whether to support of 5GS UP optimization (support for transfer of user plane data from CM-IDLE without the need for using the Service Request procedure to establish Access Stratum (AS) context in RAN and UE, similar to EPS) for NB-IoT and eMTC connected to 5GC. A number of companies in SA2 see value in supporting 5GS UP optimization (as described in TR 23.724 clause 6.19) for NB-IoT and eMTC connected to 5GC. A number of companies see instead value in supporting only RRC_INACTIVE, as introduced in 5GS but only with sleep cycles up to NAS/SMS retransmission timers, for NB-IoT and eMTC connected to 5GC. SA2 kindly requests RAN2 to inform SA2 whether they intend to support UP optimization and to support RRC_INACTIVE for NB-IoT and eMTC connected to 5GC. 2. Actions: To RAN, RAN2, RAN3 groups. ACTION: SA2 kindly asks RAN working groups to take the above into account during their work on Cellular IoT. To RAN2 group. ACTION: SA2 kindly requests RAN2 to inform SA2 whether they intend to support UP optimization and RRC_INACTIVE for NB-IoT and eMTC connected to 5GC.

In this contribution we discuss how RAN2 should reply to SA2 on their LS relating to the new objective on 5GC connectivity. Further RAN2 work is expected be started and discussed at earliest in RAN2#105bis.

Version of the paper for NB-IoT [2] is submitted to agenda item 12.2.10. The content of the NB-IoT version is the same as in this paper. A draft reply LS to SA2 [3] covering both LTE-M and NB-IoT is in [3] .

# 2	Discussion

One RAN impact identified in SA2 CIoT study is to support extended DRX (eDRX) in both CM-IDLE and CM-CONNECTED with RRC_INACTIVE when LTE-M UEs are connected to 5GC. These topics were added to the revised WID as well, i.e., the intention of the 5GC connectivity objective is to specify support of LTE-M UEs connecting to 5GC with support of extended DRX in CM-IDLE and CM-CONNECTED with RRC_INACTIVE.

RRC_INACTIVE has already been specified for eLTE case, i.e., RRC_INACTIVE is part of TS 36.331 since Rel-15. Therefore, from specification point of view, it should be relatively straightforward to introduce RRC_INACTIVE support for LTE-M.

When considering use of RRC_INACTIVE as a solution for infrequent small data transmissions, one drawback is the potentially limited eDRX cycle length highlighted in the SA2 LS. For cellular IoT, and specifically for LTE-M UEs, it is vital to have as long battery life as possible. Extended DRX in RRC_INACTIVE with cycle lengths up to NAS and SMS retransmission timers is not enough if the theoretical device/battery lifetime is expected to be one par what can be achieved with EPS. Therefore, longer extended DRX cycles, similar to what is possible in EPS, should be supported.

Currently, in EPS, the RRC suspend/resume procedure is supported from RAN and CN side as the solution for UP optimization. A similar solution for 5GS has been discussed in the SA2 study TR [4], which could be considered as alternative solution for infrequent data transmission over user plane. In this solution the UE stays in idle mode (cf. connected in inactive) and extended idle mode DRX cycles could be used (if specified for 5GS). As a UE power saving solution such a solution would be better than RRC_INACTIVE if the eDRX cycle lengths for inactive are not on par.

We think it would be beneficial if only one UP optimization that can address the use cases that have been identified so far, is specified. It seems straightforward to enhance the RRC_INACTIVE UP optimization which has already been specified for 5GS, so that it not only provides support for frequent small data but also for infrequent small data over user plane. The current drawback is the eDRX cycle lengths that are not supported to be on par with LTE idle mode eDRX cycles.

Reply to SA2 that RAN2 intends to support RRC_INACTIVE as the sole UP solution for connecting LTE-M to 5GC assuming that extended DRX cycles with length on par with the LTE/EPC case are supported.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reply to SA2 that RAN2 intends to support RRC_INACTIVE as the sole UP solution for connecting LTE-M to 5GC assuming that extended DRX cycles with length on par with the LTE/EPC case are supported.

# References

RP-182891, “Revised WID: Additional MTC enhancements for LTE”, Ericsson, Gold Coast, Australia, 10th – 13th Sep, 2018

R2-1900745, “NB-IoT connectivity to 5GC”, Ericsson, RAN2#105

R2-1900746, “[Draft] Reply LS on Completion of study on Cellular IoT support and evolution for the 5G system”, Ericsson, RAN2#105

TR 23.742, v.16.0.0