3GPP TSG-RAN WG2 Meeting #105                                  		R2-190080

Athens, Greece, 25th February - 1st March 2019

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on the scenarios to be considered

Agenda item:		11.9.4

Document for:	    Discussion and Decision

# Introduction

Two mobility enhancement work items were agreed at RAN#80: one for NR (NR_Mob_enh) [1] and the other for LTE (LTE_feMob) [2]. During the discussion of LTE_feMob at the past two RAN2 meetings, questions were raised whether to include LTE/5GC in the scope of the WI. Finally we reached the following agreements at RAN2#104:

Agreements

1	We will prioritize solutions for LTE/EPC in this WID. Can discuss LTE/5GC support based on Stage-3 details.

2	Do not consider solutions for handover between LTE/EPC and LTE/5GC.

The above agreements take only into account LTE. However, with the discussion of LTE_feMob &NR_Mob_enh in parallel, we will have much more handover types, including intra-system intra-RAT, intra-system inter-RAT, inter-system intra-RAT, inter-system inter-RAT. So before going to the solution details, the scope of the considered scenarios should be clarified first.

# Discussion

| Scenarios | Considered in the scope or not |
| --- | --- |
| Scenario1: Intra LTE/EPC | Yes (as part of LTE_feMob) |
| Scenario2: Intra LTE/5GC | Yes (Per agreement at RAN2#104) |
| Scenario3: LTE/EPC<->LTE/5GC | No (Per agreement at RAN2#104) |
| Scenario4: LTE/EPC<->NR | FFS |
| Scenario5: LTE/5GC<->NR | FFS |
| Scenario6: NR<->NR | Yes (as part of NR_Mob_enh) |

Table 1. Mobility scenarios

Table 1 lists all the possible mobility scenarios. Among the listed 6 scenarios, of course Scenario1 and Scenario6 should be considered within LTE_feMob and NR_Mob_enh respectively. And per agreements at RAN2#104, Scenario2 should also be considered with relative low priority while Scenario3 is ruled out. So there are only Scenario4 & Scenario5 remaining for further clarification.

Scenario4:

Scenario4 is for inter-system inter-RAT mobility. In this case, all the handover signaling (or role change signaling in case of dual connectivity based solutions) should be relayed by the core network. Different PDCP would be used in the source and target, i.e. one with LTE PDCP and the other with NR PDCP. Given that, it’s impossible to achieve lossless HO in this case. So, same as what we have agreed for Scenario3, Scenario4 should not be considered in the WI of NR_Mob_enh.

Proposal1: Do not consider solutions for handover between LTE/EPC and NR in the WI for NR_Mob_enh.

Scenario5:

Scenario5 is for intra-system inter-RAT mobility. In this case, the handover signaling (or role change signaling in case of dual connectivity based solutions) can be exchanged over the direct interface between the source and target, i.e. Xn interface. Meanwhile, both the source and target would use NR-PDCP. So lossless HO can be achieved in this case. Given that, there seems no big obstacle to include Scenario5 in the scope of NR_Mob_enh. However, it should be noted that Scenario5 is inter-RAT mobility. So, if mobility enhancements are applied also for this case, the impact on the inter-RAT mobility procedure in 36.331 and 38.331 should be carefully considered. Given the above, we think that Scenario5 can be considered in the scope of NR_Mob_enh, even if with lower priority.

Proposal2: Mobility between LTE/5GC and NR can be considered in the scope of NR_Mob_enh, with low priority.

# Conclusion and proposals

In this contribution, the scenario scope of the NR_Mob_enh WI is clarified with the following proposals:

Proposal1: Do not consider solutions for handover between LTE/EPC and NR in the WI for NR_Mob_enh.

Proposal2: Mobility between LTE/5GC and NR can be considered in the scope of NR_Mob_enh, with low priority.

# References

- RP-181433 New WID: NR mobility enhancements Intel

- RP-181544, New Work Item on even further Mobility enhancement in E-UTRAN, China Telecom