3GPP TSG-RAN WG2 Meeting #105												R2-1900838

Athens, Greece, 25th February - 1st March 2019

Source: 	vivo

Title:  	Potential enhancements on the reference time accuracy

Agenda Item:	11.7.2.1

Document for:	Discussion and Decision

# Introduction

According to the LS [1] from SA2, RAN2 is asked to provide the reference time information to the UE, and the “clock synchronisation service performance requirement” is defined in the TR 22.804 [2] and TR 22.104 [3]. The latest requirements in TR 22.104 can be found in the Annex. In the RAN2#104 meeting, RAN2 made the following agreement:

We reuse the LTE approach for time distribution by broadcast RRC as a baseline, Unicast is FFS 0.25us granularity can be starting point, FFS finer granularity than 0.25us

In this contribution, we discuss the potential enhancements in RAN regarding the synchronisation requirement, based on the latest SA scenarios and the RAN1 agreements.

# Discussion

## Analysis on the clock synchronization error

In the RAN2#104 meeting, RAN2 discussed the synchronisation requirements based on [3]. According to discussion in RAN1, the typically assumed SCS for the I-IOT evaluation is 30 kHz [4]. In this section, we provide an updated table based on [2] [3] and the latest RAN1 agreements.

Table 1: Timing error in clock synchronisation

| Factors | Timing error type | Requirement for different SCS (kHz) (unit: ns) |
| --- | --- | --- |
| 1 | Inaccuracy caused by downlink/uplink frame timing alignment |  |
| 1.1 | Time alignment error of gNB transmitter | 65 |
| 1.2 | Inaccuracy caused by UE detection | 260 |
| 1.3 | Inaccuracy caused by propagation delay when propagation delay is decided from TA. (1.3.1+1.3.2+1.3.3)/2 | 325 |
| 1.3.1 | Inaccuracy of UE transmitting | 260 |
| 1.3.2 | Inaccuracy of gNB detection | 260 |
| 1.3.3 | Inaccuracy caused by TA indication | 130 |
| 1.4 | Inaccuracy caused by propagation delay when considering 20 km2 service area | 8410 |
| 2 | Inaccuracy caused by time reference information delivery from gNB to UE | 125 |
| 3 | Inaccuracy caused by the synchronisation between the gNB and external clock | 100 |
| Total inaccuracy for 20 km2 service area (with propagation error) | Intra-gNB (2*(1.1+1.2+1.4+2)) | 17720 |
|  | Inter-gNB (2*(1.1+1.2+1.4+2+3)) | 17920 |

According to the analysis given above, the synchronization error is mostly caused by the propagation delay.

Observation 1: The 1us requirement of clock synchronisation cannot be fulfilled in the current NR system.

Observation 2: According to the latest TSN scenarios in TR 22.104, the synchronization error is mostly caused by the propagation delay.

One possible way to correct the error caused by the propagation delay is to use NTA/2 as the estimated propagation delay.

Proposal 1: NTA/2 is used to correct the reference time inaccuracy caused by the propagation delay.

# Conclusions

Based on the analysis given above, we have the following Observation and Proposal：

Observation 1: The 1us requirement of clock synchronisation cannot be fulfilled in the current NR system.

Observation 2: According to the latest TSN scenarios in TR 22.104, the synchronization error is mostly caused by the propagation delay.

Proposal 1: TA/2 is used to correct the reference time inaccuracy caused by the propagation delay.

# Reference

[1] LSin, SA2, R2-1813554_S2-189051, “LS on TSN integration in the 5G System”.

[2] 3GPP TR 22.804, “Study on Communication for Automation in Vertical Domains”.

[2] 3GPP TR 22.104, “Service requirements for cyber-physical control applications in vertical domains”.

[3] R2-1818254	Time Synchronisation for IIOT	NTT DOCOMO, INC.	discussion.

[4] R1-1814279, Nokia, “Proposal for the simulation assumptions for RAN2 LS on TSN requirements evaluation”

# Annex

The following requirements are extracted from the TR 22.104:

### 5.6.2 Clock synchronisation service performance requirements Table 5.6.2-1: Clock synchronization service performance requirements User-specific clock synchronicity accuracy level; Number of devices in one Communication group for clock synchronisation; Clock synchronicity requirement; Service area; Scenario. 1; Up to 300 UEs; < 1 µs; ≤ 100 m x 100 m; - Motion control - Control-to-control communication for industrial controller. 2; Up to 10 UEs; < 10 µs; ≤ 2500 m2; - High data rate video streaming. 3; Up to 100 UEs; < 1 µs; < 20 km2; - Smart Grid: synchronicity between PMUs.