3GPP TSG-RAN WG2 Meeting#105	R2-1900881

Athens, Greece, 25th February – 1st March 2019

Agenda item:	11.4.2.2

Source:		Intel Corporation

Title:		On mode 1 and mode 2 simultaneous operation and switching

Document for:	Discussion

Introduction

In the email discussion on resource allocation RAN2#104#60, simultaneous support of mode 1 and mode 2 operation at the UE was briefly discussed. In this contribution, we aim to lay out further details to support this aspect.

Discussion

  - Simultaneous mode support

### During the last meeting, the following note considering the simultaneous support of network controlled and UE autonomous sidelink transmission and resource allocation was agreed to be added to TR 38.885. In LTE, only one mode could be supported at a given time by the UE.

### “

Editor's note: Whether to support CBR measurement and reporting, and simultaneous configuration of mode 1 and mode 2 for resource allocation for the UE has RAN1 dependency.

“

Supporting different services with different QoS requirements (e.g. 3ms or 100ms) seems to be the motivation among different companies to introduce this type of configuration for NR V2X. In LTE V2X, for both NW scheduled and autonomous cases, the transmission was broadcast based, i.e. no dedicated connection was assumed by the UE when sending the packet over sidelink. With NR V2X and the possibility of supporting both unicast and broadcast based communication, there is an added dimension of complexity to study when considering resource configuration design and procedures e.g. in case when both the NW scheduled and UE autonomous modes are supported simultaneously.

  - Connection establishment for multiple mode operation

### In order to avoid complexity, the V2X UE can be assumed to be in RRC_CONNECTED state to support both network scheduled and UE autonomous resource allocation modes simultaneously, however, if a UE is in-coverage and has the ability to support mode-1, it would be practical to initiate a switch (from idle/inactive to connected) so the required QoS can be met as necessary. Furthermore, when the UE is out-of-coverage it only has the ability to use mode-2 (UE autonomous mode) but if the QoS requirement for the use case indicates that mode-1 could be used, the UE should try to switch to mode-1 as soon as it is in-coverage.

### Proposal 1: 	RAN2 to allow simultaneous resource allocation mode operation for RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED UEs.

### In order to enable the switch to connected state, the V2X UE should initiate RRC connection establishment as mode-1 is only possible in RRC_CONNECTED state. This can be achieved in one of the following ways:

- Option 1: UE based RRC connection initiation for support of mode-1 when in-coverage (using a new establishment/resume cause value e.g. mo_V2X).

  - It incurs minimal signalling overhead as it is only initiated as necessary to support a specific QoS.

  - It is already supported as per LTE, however, not explicitly (as shown in the note below from TS 36.331, where it is left to UE implementation) while in NR, we could define a new cause value (establishment or resume cause value wherein multiple spares are available) as part of RRC setup request or RRC resume request messages.

### “

NOTE:	Upper layers initiate an RRC connection. The interaction with NAS is left to UE implementation.

### “

- Option 2: Network keeps the UE always in RRC_CONNECTED even when there is no data exchanged over the Uu link (as long as the UE is in-coverage)

  - As the V2X UEs are expected to perform a lot of handovers, this might incur additional signalling overhead, but would ensure the necessary QoS for the given advanced use case.

  - UE can suggest its required QoS parameters for a given service (and thereby if it prefers mode-1 operation) via UE assistance information to ensure that the network allows the UE to stay in connected mode.

- Option 3: UE could continue to maintain a signalling-only connection with the network.

  - This would ensure that there is minimal delay in switching modes (avoiding connection establishment) while also reducing signalling overhead. Currently, the signalling-only connection [R2-1818916] does not support handovers and RRC_INACTIVE state, however, we could introduce the necessary changes to support V2X use case.

    - As stated in option 2, the V2X UEs are expected to perform a lot of handovers, therefore, this might incur additional signalling overhead (unless optimizations are done), but would ensure the necessary QoS for the given advanced use case.

### Proposal 2: 	RAN2 is asked to discuss the three options and select a suitable one to ensure support of simultaneous resource allocation modes: 1) A V2X UE can initiate RRC Connection establishment using explicit establishment/resume cause value within RRC setup/resume request message (MSG3) to enter RRC_CONNECTED 2) Keep a V2X UE in the RRC_CONNECTED state 3) Keep a V2X UE in signalling-only connection-based RRC_CONNECTED state.

  - Switching modes

When simultaneous resource allocation mode is supported, it needs to be discussed about which layer or entity is responsible for deciding the specific resource allocation mode to be used for a given service. This can be suggested by the network in a deterministic way or pre-configured or potentially hinted by the upper layers dynamically. The classification of whether the data can use mode-1 only or mode-2 only or both modes can be done on per-packet or per-flow or per-service basis although if done on a per-packet basis, it might incur a lot of overhead e.g. BSR updates and processing. If the data is classified to be supportive of either modes, it may be up to the network or UE implementation to switch the mode. This can be treated as configuration information similar to SL RB configuration and thereby NW-controlled when in-coverage and connected mode; and UE-initiated when in-coverage but in idle or inactive mode.

The easiest methodology is to always use mode-1 when in-coverage and mode-2 when out-of-coverage. However, this is not always the best approach due to the support of services with varying QoS requirements (e.g. stringent delay with relaxed reliability or relaxed delay with stringent delay and data rate). It is sometimes more effective to use mode-2 even while in-coverage as there could be minimal delay in obtaining resources and sending the data packet. Some companies suggested that mode-1 can be used for licensed operations while mode-2 can be used for unlicensed operations wherein switching between modes is not needed but certain sidelink enhancements e.g. BSR updates are necessary to support the simultaneous operation. The network may maintain the mapping between the specific QoS and authorized services that are supported by the UE to determine UE’s request to use a specific mode and suggest a change in mode accordingly.

### Proposal 3: 	RAN2 to consider network-controlled mode switching when the V2X UE is in-coverage and RRC_CONNECTED and UE-initiated mode switching when the V2X UE is in-coverage and RRC_IDLE or RRC_INACTIVE.

Conclusion

In this contribution, we provide further considerations on simultaneous resource allocation mode operation for NR V2X sidelink and have the following proposals:

### Proposal 1: 	RAN2 to allow simultaneous resource allocation mode operation for RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED UEs.

### Proposal 2: 	RAN2 is asked to discuss the three options and select a suitable one to ensure support of simultaneous resource allocation modes: 1) A V2X UE can initiate RRC Connection establishment using explicit establishment/resume cause value within RRC setup/resume request message (MSG3) to enter RRC_CONNECTED 2) Keep a V2X UE in the RRC_CONNECTED state 3) Keep a V2X UE in signalling-only connection-based RRC_CONNECTED state.

### Proposal 3: 	RAN2 to consider network-controlled mode switching when the V2X UE is in-coverage and RRC_CONNECTED and UE-initiated mode switching when the V2X UE is in-coverage and RRC_IDLE or RRC_INACTIVE.

References

[1] RP-181429, "New SID: Study on NR V2X", Vodafone.

[2] TR 22.186, Enhancement of 3GPP support for V2X scenarios; Stage 1.

[3] TR 23.786, Study on architecture enhancements for EPS and 5G System to support advanced V2X services

(Release 16)