3GPP TSG-WG2 Meeting #105	R2-1900889

# Athens, Greece, 25th February – 1st March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3864 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on handling data in SRB and data in DRB in Msg4 for EDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Core |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | 15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently in UP-EDT, Msg4 contains RRC message (RRCConnectionRelease message) in SRB1 plus optionally DL data in DRB. The RRC message in SRB1 is integrity protected and encrypted whereas the DL data in DRB is only encrypted. When MAC receives Msg4, it de-multiplexes RRC message in SRB1 and DL data in DRB and forwards them to upper layers. Based on implementation, following cases may happen. - PDCP first processes RRC message - PDCP first processes the DL data in DRB In fact, if the integrity protection check of the RRC message fails, UE does not need to process the DL data in DRB, which logically becomes invalid. However, the issue with processing RRCConnectionRelease message first is that UE may proceed to discard stored UE context and release DRBs which could prevent UE from de-ciphering the DL data in DRB or forwarding the DL data in DRB to upper layers. BL UE or UE in CE may not wait for 1.25 seconds and NB-IoT UE for 10 seconds and immediately proceed release procedure when STATUS reporting, as defined in TS 36.322 [7], has not been triggered and the UE has sent positive HARQ feedback (ACK) for RRCConnectionRelease message, as defined in TS 36.321 [6]. Therefore, a clarification is needed that RRC delays the release procedure if the DL data in DRB has not been processed yet especially. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.3.8.3, a note is added to clarify that in case of UP-EDT, UE also delays the RRC connection release procedure if the DL data in DRB in Msg4 has not yet been forwarded to upper layers as follows. NOTE: In case of UP-EDT, UE delays the following actions defined in this sub-clause until the DL data in DRB, if present in Msg4, has been processed as defined in TS 36.323 [8]. Impact analysis Impacted functionality: RRC connection release process in UP-EDT Inter-operability: Since there is no impact to the network, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may release stored UE context and DRBs before the any received DL data in DRB in Msg4 for EDT is forwarded to upper layers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

### 5.3.8	RRC connection release

#### 5.3.8.1	General

Figure 5.3.8.1-1: RRC connection release, successful

The purpose of this procedure is:

-	to release the RRC connection, which includes the release of the established radio bearers as well as all radio resources; or

-	to suspend the RRC connection for both suspended RRC connection or RRC_INACTIVE, which includes the suspension of the established radio bearers.

#### 5.3.8.2	Initiation

E-UTRAN initiates the RRC connection release procedure to a UE in RRC_CONNECTED or in RRC_INACTIVE.

#### 5.3.8.3	Reception of the RRCConnectionRelease by the UE

The UE shall:

1>	except for NB-IoT, BL UEs or UEs in CE, delay the following actions defined in this sub-clause 60 ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	for BL UEs or UEs in CE, delay the following actions defined in this sub-clause 1.25 seconds from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	for NB-IoT, delay the following actions defined in this sub-clause 10 seconds from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier.

NOTE:	For BL UEs, UEs in CE and NB-IoT, when STATUS reporting, as defined in TS 36.322 [7], has not been triggered and the UE has sent positive HARQ feedback (ACK), as defined in TS 36.321 [6], the lower layers can be considered to have indicated that the receipt of the RRCConnectionRelease message has been successfully acknowledged. In case of UP-EDT, UE delays the following actions defined in this sub-clause until the DL data in DRB, if present in Msg4, has been processed as defined in TS 36.323 [8].

<End of change>