3GPP TSG-RAN WG2 Meeting #105	R2-1901064

Athens, Greece, 25th February - 1st March 2019

Agenda Item:	11.4.2.1

Source:	MediaTek Inc.

Title:	Connection control for unicast

Document for:     Discussion and Decision

# 1 Introduction

In RAN2#104 meeting, it is discussed that for V2X unicast, how we model the framework for AS-level information exchange and whether we need AS-layer connection establishment procedure. The following agreements are made:

Agreements on unicast

1:	For AS-level information required to exchange among UEs via sidelink for SL unicast, RAN2 can consider the followings as a baseline and will check if the AS-level information can be agreed and the details after some progress in RAN2, SA2 and RAN1:

- UE ID, UE capability, Radio/Bearer configuration, PHY information/configuration (e.g. HARQ, CSI), Resource information/configuration and QoS info

2:	AS-level information for SL unicast can be exchanged between gNB and UE for RRC configuration. RAN2 assumes that a UE can provide network with QoS related information and will check if the AS-level information can be agreed and the details after some progress in RAN2, SA2 and RAN1.

3:	AS-level information is exchanged via RRC signalling (e.g. PC5-RRC) among UEs via sidelink for SL unicast. New logical channel (SCCH: SL Control Channel) in addition to STCH (SL Traffic Channel) will be also introduced. SCCH carriers PC5-RRC messages.

4:	RAN2 will consider both options during SI phase. Further discussion on the definition, procedure and information for each option is needed.

- Option 1: AS layer connection establishment procedure by PC5-RRC is also needed.

- Option 2: Upper layer connection establishment procedure is enough.

5:	RAN2 will study a kind of RRM or RLM based AS level link management. RAN2 will not consider a kind of PC5-RRC level keep alive message based management. Further discussion on possible detailed options is needed.

Agreements on groupcast

6:	Further discussion is needed on whether groupcast follows same mechanism for unicast, which are agreed in the above.

7:	No AS-level mechanism to determine a group manager (i.e. head UE) is stuided. FFS for platooning, on the visibility of a group manager (head UE) to AS and AS-level functionalities.

In this paper, we provide our view on connection control for unicast.

# 2 Discussion

## 2.1 Whether to introduce AS layer connection establishment procedure by PC5-RRC

In the email discussion, companies have expressed their preference on this issue.

- Option 1: AS layer connection establishment procedure by PC5-RRC is also needed. - Option 2: Upper layer connection establishment procedure is enough.

As already addressed by companies, option 1, although with more overhead than option 2, enable UE to manage AS-level configuration easily (i.e. reduce interaction with upper layer when there is AS-layer configuration update), and support differentiated QoS for different SRBs and DRBs. So, we think introducing AS layer connection is beneficial.

In addition, security for PC5-RRC message is required. In Uu, the procedure to activate AS security is applied when only SRB1 is established, i.e., prior to establishment of SRB2 and/ or DRBs (TS 38.331 subclause 5.3.4.2). For option 1, PC5-RRC message can be integrity protected and ciphered after AS security activation as in Uu. But for option 2, it is not yet clear when and how AS security is introduced for the PC5-RRC message to deliver AS layer configuration (i.e. RRC reconfiguration) and UE capability.

Observation 1: Introducing AS layer connection establishment procedure will introduce additional signalling overhead, but can support differentiated QoS for different SRBs and DRBs, and decouple AS-level signalling from upper layers.

Observation 2: Without AS-layer connection establishment procedure, it is not yet clear when and how AS security is introduced for the PC5-RRC message to deliver AS layer configuration and UE capability.

Proposal 1: Introduce AS layer connection establishment procedure by PC5-RRC.

## 2.2 AS layer Information Exchange

In the email discussion [1], companies have consensus that PC5-RRC signalling is required for the Tx/Rx UE to exchange the information of UE capability and AS layer configuration. A question is who (Tx or Rx) initiate the procedure for information exchange and/or AS layer configuration.

Our view is that, as company addressed, a unicast connection is bi-directional, so each UE needs to know the UE capability/ AS layer configuration of its peer UE regardless of whether it is the Tx or Rx UE at this moment. Moreover, UE can store the UE capability/ AS layer configuration of peer UEs and use them for later V2X communication. So, it seems no need to restrict the signalling initiator to be the Tx UE only or the Rx UE only.

Proposal 2: Both the Tx/Rx UE can initiate signalling for acquiring UE capability and AS layer configuration of the peer UE.

# 3 Conclusion

Based on the observation:

Observation 1: Introducing AS layer connection establishment procedure will introduce additional signalling overhead, but can support differentiated QoS for different SRBs and DRBs, and decouple AS-level signalling from upper layers.

Observation 2: Without AS-layer connection establishment procedure, it is not yet clear when and how AS security is introduced for the PC5-RRC message to deliver AS layer configuration and UE capability.

We propose:

Proposal 1: Introduce AS layer connection establishment procedure by PC5-RRC.

Proposal 2: Both the Tx/Rx UE can initiate signalling for acquiring UE capability and AS layer configuration of the peer UE.

# 4 Reference

- R2-1900180, Summary of [104#55] V2X Unicast (OPPO)