3GPP TSG-RAN WG2#105	Tdoc R2-1901085

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	12.3.2

Source:	Ericsson

Title:	Enhancements to Make-Before-Break

Document for:	Discussion, Decision

# Introduction

At the RAN#80 meeting in June 2018, a new Rel-16 WI was approved [1] with the purpose to further enhance LTE mobility by reducing handover interruption time and improve mobility robustness. The work starts with a study phase where potential solutions are studied, followed by a work phase where the chosen solution will be specified.

As part of the study phase, an email discussion has been held to progress the work with reduced handover interruption time and to discuss and compare potential solutions for the so-called split and non-split bearer architecture solutions (the latter a.k.a. enhanced Make-Before-Break (MBB) based on the MBB handover solution introduced in Rel-14).

In this paper we explore on some further details on the enhanced MBB handover solution as described in section 2.2.4 in the email discussion report [2].

# Enhanced MBB in Rel-16

Even if MBB and RACH-less HO (both concepts introduced in Rel-14) are combined in the same handover procedure, it is still not possible to accomplish seamless mobility, i.e. true 0ms HO interruption time, in all scenarios. In order to meet this target, the UE need to maintain the connection with the source eNB until the connection to the target eNB is successfully established.

Until now, the UE ability to simultaneously send and receive to more than one cell has been focused on hardware aspects such as number of Rx or Tx chains. But according to RAN1 LS response in R1-1814411, a UE with 1 Rx chain may for instance be able to receive from two cells simultaneously under some conditions, e.g. if both cells share the same frequency or if they are located within the same frequency band.

So rather than discussing Rx/Tx capabilities, the enhanced MBB solution in Rel-16 should be focused on the UE ability to simultaneously receive and transmit to/from the source cell and the target cell.

The enhanced MBB procedure in Rel-16 is based on the UE ability to simultaneously receive and transmit to/from the source cell and the target cell.

In any case, the UE need to keep its connection with the source cell until it is able to receive and transmit packets in the target cell. In principle, this means that the source eNB, after sending the RRCConnectionReconfiguration message to the UE, continues to send DL data packets to the UE while forwarding the same DL data packets to the target eNB.

Starting data forwarding at an early stage (e.g. when sending the RRCConnectionReconfiguration message) means that the target eNB can start DL transmission immediately when the UE has completed the handover to the target cell, which (due to X2 latency) may not be the case if data forwarding is started later in the Handover Execution phase. But an early start of data forwarding also means that the number of DL PDCP SDU duplications buffered in the target eNB (and possibly sent to the UE in the target cell) will increase compared to a late start of data forwarding (e.g. when the UE performs random-access in the target cell). However, we think this can be resolved with PDCP duplication check in the target eNB based on the UE sending a PDCP Status Report [3] together with, or directly after, the RRCConnectionReconfigurationComplete message in the target cell.

To achieve seamless handover, the UE keeps the connection to the source cell until the connection with the target cell is completed.

The principle to support 0ms HO interruption time by means of an enhanced MBB procedure is illustrated in Figure 1 below.

Figure 1 Enhanced MBB procedure to support 0ms HO interruption time.

The key steps to support 0ms HO interruption time by means of enhanced MBB procedure are as follows:

Step 1-2:	The source eNB decides to request for a handover with enhanced MBB, thus a HANDOVER REQUEST message with an eMBB indicator is sent to the target eNB.

Step 3-5:	As in legacy, the target eNB performs admission control and generates the RRCConnectionReconfiguration message (Handover Command) to be sent by the source eNB to the UE. The presence of an eMBB indicator in the RRC message serves as a request to the UE to perform a handover with enhanced MBB.

Since some MBB Rel-16 specific interaction is likely needed between source and target node (e.g. early sending of the SN STATUS TRANSFER message, see step 6 below), the source eNB need to know whether the target eNB supports enhanced MBB for each individual handover request. For that purpose, an “eMBB support indicator” (or similar) may be needed in the HANDOVER REQUEST ACK message. Consequently, if the target eNB (for some reason) do not support enhanced MBB for this specific handover request, a fallback to a legacy handover (or legacy MBB as defined in Rel-14) is an option.
Obviously, this need to be addressed in the upcoming RAN3 discussions.

Step 6-7:	Source eNB starts forwarding DL PDCP SDUs to the target eNB (as in legacy, the exact point in time when the source eNB starts data forwarding is left for implementation) and continues to send and receive PDCP PDUs to/from the UE. The source eNB does not apply any form of selective data forwarding, i.e. all DL PDCP SDUs arriving from the Serving Gateway (SGW) are sent to the UE and forwarded to the target eNB.
As before the Handover Command is sent to the UE, the source eNB continues to send UL PDCP SDUs to the Serving Gateway (SGW) on the old S1-U path.

Since all DL PDCP SDUs are forwarded to the target eNB it is sufficient to indicate the PDCP SN of the first forwarded DL PDCP SDU, which is done using the SN STATUS TRANSFER message. The SN STATUS TRANSFER message also need to be sent to convey HFN status for COUNT preservation in the target eNB.

Sending the SN STATUS TRANSFER message at this point is a deviation from legacy handover in which case the X2AP message is only sent after the source eNB considers the transmitter/receiver status to be frozen. Obviously, the conditions for sending the SN STATUS TRANSFER message when enhanced MBB is configured need to be discussed in RAN3.

DL PDCP SDUs which have been PDCP processed in the source eNB but not yet acknowledged by the UE may also be forwarded to the target eNB (in this case the PDCP SN is indicated in the GTP-U extension header as in a legacy HO). Forwarding of DL PDCP SDUs will continue from the source eNB until the “end marker” packet is received from the SGW on the old path (as a result of the DL path switch request triggered by the target eNB).

In case the source eNB has undelivered uplink PDCP SDUs (e.g. PDCP SDUs received out of sequence which have not been sent to the SGW on the “old” S1-U path), the source eNB can optionally forward these SDUs to the target eNB for in-sequence delivery on the new S1-U path.

Step 8-9:	UE starts synchronizing with the target cell while it continues to receive and transmit packet data in the source cell. UE performs random-access in the target cell and target eNB schedules uplink resources.

Step 10-11:	In purpose to assist the target eNB with PDCP duplication check, the UE sends a PDCP SN Status Report for each RLC-AM bearer configured to the UE. The PDCP SN Status Report is either sent together with the RRCConnectionReconfigurationComplete message (multiplexed on MAC level if the provided UL grant is sufficiently large) or it is sent immediately after the RRC message. Based on the FMS field (first missing PDCP SN) in the PDCP Status Report, the target eNB starts sending DL PDCP PDUs to the UE from the first missing PDCP SN (i.e. from the last PDCP SN +1 received in the source cell) whereby sending of duplicated packets over the radio interface is avoided.
If the bitmap is present in the PDCP Status Report indicating one or more PDCP SDUs stored out-of-sequence in the UE, then the PDCP PDUs of the missing SNs need to be delivered.

As in legacy, the target eNB now sends a DL path switch request to the MME/SGW.

Step 12:	The UE stops transmission/reception to/from the source cell. The exact point when the UE stops receiving DL transmission and stops UL transmission to/from the source cell is FFS. One (of many) alternative is to stop DL reception from the source cell when receiving the first DL packet in the target cell. Similarly, UL transmission to the source cell can be stopped when the UE transmits the first UL packet in the target cell.
Other alternatives are conceivable, including combining the two switching points for UL and DL into one common switching point, see further DP on user plane aspects [4].

The advantage of defining a given switching point when the UE switch from source cell to target cell is that only one of the protocol stacks (i.e. source and target PDCP/RLC/MAC) will be active in the UE at any given point in time. Potential benefits with such a solution are e.g. simplified specification and UE implementation.

Besides, given a distinct switching point (from source to target) can be defined for the UE, a network-controlled release by means of an explicit release/reconfiguration message seems not to be needed.

Step 13:	Immediately after receiving the RRCConnectionReconfigurationComplete message, the target eNB sends a (new) X2AP message to the source eNB to indicate that the handover has completed, thus based on this information the source eNB can stop the transmission/reception to/from the UE.

In many deployments, this could be a better alternative than relying on autonomous detection in the source eNB (missing MAC HARQ or RLC ARQ feedback), with the obvious drawback that transmission resources may be tied up in vain.

Yet another trigger alternative to stop transmission/reception to/from the UE could potentially be at reception of the “end marker” packet in the source eNB (received from SGW as a result of the DL path switch request triggered by the target eNB). However, it is expected that the ”end marker” packet will have a longer turn-around time compared to an indicator sent directly over X2, thus with the proposed X2AP message the transmission/reception in the source eNB will be stopped at an earlier point.

Also this issue need to be addressed in the upcoming RAN3 discussions.

As can be understood from above, the UE need to receive and transmit on two cells simultaneously for a short while during the Handover Execution phase, e.g. when performing synchronization and random-access procedure in the target cell while at the same time communicating with the source cell.

User plane data, on the other hand, is received and sent to/from the source cell until a given switching point. After this point, the UE will only send and received UP data via the target cell. In other words, UP data do not need to be sent or received simultaneously on the two cells by the UE.

User plane aspects in connection to enhanced MBB is further discussed in [4].

Based on the above analysis, we propose the following:

The source eNB should start data forwarding of DL PDCP SDUs to target eNB shortly after receiving the HANDOVER REQUEST ACKNOWLEDGE message, i.e. the legacy behavior is kept.

The conditions for sending the SN STATUS TRANSFER message when enhanced MBB is configured need to be updated. Final decision is up to RAN3.

All DL PDCP SDUs received from the Serving Gateway are forwarded to the target eNB, i.e. the source eNB does not apply any form of selective data forwarding.

The increased number of DL PDCP SDU duplications buffered in the target eNB and potentially sent to the UE, can be resolved with PDCP duplication check in the target node.

To assist the target eNB with PDCP duplication check, the UE sends a PDCP Status Report for each RLC-AM bearer configured to the UE. The PDCP Status Report is sent together with the RRCConnectionReconfigurationComplete message (multiplexed on MAC level) or immediately after the RRC message.

Given a definite “switching point” (when communication to the source cell is dropped) can be defined for the UE, a network-controlled release by means of an explicit release/reconfiguration message is not needed.

A new X2AP message sent from target eNB to source eNB indicating “handover completed” should be considered. The new message will trigger the source eNB to stop transmitting DL data and receiving UL data to/from the UE. Final decision is up to RAN3.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The enhanced MBB procedure in Rel-16 is based on the UE ability to simultaneously receive and transmit to/from the source cell and the target cell.

Proposal 2	To achieve seamless handover, the UE keeps the connection to the source cell until the connection with the target cell is completed.

Proposal 3	The source eNB should start data forwarding of DL PDCP SDUs to target eNB shortly after receiving the HANDOVER REQUEST ACKNOWLEDGE message, i.e. the legacy behavior is kept.

Proposal 4	The conditions for sending the SN STATUS TRANSFER message when enhanced MBB is configured need to be updated. Final decision is up to RAN3.

Proposal 5	All DL PDCP SDUs received from the Serving Gateway are forwarded to the target eNB, i.e. the source eNB does not apply any form of selective data forwarding.

Proposal 6	The increased number of DL PDCP SDU duplications buffered in the target eNB and potentially sent to the UE, can be resolved with PDCP duplication check in the target node.

Proposal 7	To assist the target eNB with PDCP duplication check, the UE sends a PDCP Status Report for each RLC-AM bearer configured to the UE. The PDCP Status Report is sent together with the RRCConnectionReconfigurationComplete message (multiplexed on MAC level) or immediately after the RRC message.

Proposal 8	Given a definite “switching point” (when communication to the source cell is dropped) can be defined for the UE, a network-controlled release by means of an explicit release/reconfiguration message is not needed.

Proposal 9	A new X2AP message sent from target eNB to source eNB indicating “handover completed” should be considered. The new message will trigger the source eNB to stop transmitting DL data and receiving UL data to/from the UE. Final decision is up to RAN3.

# References

RP-181475, New Work Item on even further Mobility enhancement in E-UTRAN, RAN#80

Email discussion [104#61][LTE/feMOB] Solution directions for minimizing user data interruption for UL/DL (Nokia)

3GPP TS 36.323, Packet Data Convergence Protocol (PDCP) specification, Release 15

R2-1901090, User plane aspects of Make-Before-Break handover, RAN2#105