3GPP TSG-RAN WG2#105	Tdoc R2-1901088

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	12.3.4

Source:	Ericsson

Title:	Scenarios for mobility interruption enhancements

Document for:	Discussion, Decision

# Introduction

At the RAN#80 meeting in June 2018, a new Rel-16 WI was approved [1] with the purpose to further enhance LTE mobility by reducing handover interruption time and improve mobility robustness. The work starts with a study phase where potential solutions will be studied, followed by a work phase where the chosen solution will be specified.

Although the WI description defines a range of handover scenarios to be considered in the study, there are more scenarios to take into account. At the RAN2#104 meeting, the discussions on the scope of the WI led to the following agreements.

Agreements

1	We will prioritize solutions for LTE/EPC in this WID. Can discuss LTE/5GC support based on Stage-3 details.

2	Do not consider solutions for handover between LTE/EPC and LTE/5GC.

This contribution addresses one more handover scenario not listed in the WID [1].

# Mobility enhancements in Rel-16

The Rel-16 WI description [1] covers a range of handover scenarios other than those currently supported by e.g. Make-Before-Break and RACH-less HO in Rel-14:

-	Inter and intra-frequency handover

-	Inter and intra-eNB handover

-	Sync and async deployments

As agreed at the RAN2#104 meeting, the study should prioritize on intra-EPC handovers. To rule out any possible impact to the core network, also intra-LTE inter-system HO between EPS and 5GS are excluded from the study.

Since intra-LTE intra-5GC handover may involve changes to the Xn interface (e.g. if RAN2 decides to reduce handover interruption time by means of enhanced Make-Before-Break solution), it seems reasonable to study this handover scenario in the corresponding study on mobility enhancements in NR. In other words, we propose intra-LTE intra-5GC handover and the potential impact to Xn to be studied in the Rel-16 WI on mobility enhancements in NR [2].

Study intra-LTE intra-5GC handover in the corresponding Rel-16 study on mobility enhancements in NR.

Another aspect to consider is whether to include heterogeneous deployments in the study, i.e. networks with macro cells in combination with small cells served by low-power base stations providing increased bitrates. Small cells are primarily added to increase capacity in hot spots with high user demand and to fill in areas not covered by the macro network – both outdoors and indoors. They also improve network performance and service quality by offloading the macro cells. These characteristics are important for services requiring ultra-reliability and low latency, i.e. services requiring improved mobility robustness and very low interruption time.

For that reason, we propose to include heterogeneous deployments in the LTE study on mobility enhancements.

Heterogeneous deployments shall be considered in the LTE study on mobility enhancements.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Study intra-LTE intra-5GC handover in the corresponding Rel-16 study on mobility enhancements in NR.

Proposal 2	Heterogeneous deployments shall be considered in the LTE study on mobility enhancements.

# References

RP-181475, New Work Item on even further Mobility enhancement in E-UTRAN, RAN#80

RP-181433, New Work Item on NR Mobility enhancements, RAN#80