3GPP TSG-RAN WG2 #105	R2-1901096

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.7.3

Source:	MediaTek Inc.

Title:	Uplink intra-UE prioritisation

Document for:	Discussion and decision

# 1 Introduction

As part of the discussion on prioritisation of different categories of traffic in a UE, emails discussions [1] and [2] were triggered from the last RAN2 meeting. In this contribution, we elaborate on how and why Intra UE prioritisation of different categories of traffic and their associated signalling should be performed.

# 2 Discussion

## 2.1 UL data prioritisation

In IIoT, it is expected that traffic of different priorities are exchanged between the UE and the gNB. Some of the traffic categories have very stringent latency requirements, while other do not. This leads to the requirement of an arbitration mechanism to decide on the data to be sent when data from multiple traffic categories are available.

On the DL, the scenario is fairly straight-forward. Both the knowledge of data priority and the choice of resource scheduling are available to the gNB. The means for arbitration can therefore reside in the gNB, leaving the UE to only monitor for the choice that the gNB has made. This flexibility is not available on the UL, as the knowledge of available traffic priority resides in the UE, which doesn’t control the resource scheduler. When two overlapping grants (that can carry different traffic) are provided by the scheduler, the UE behaviour is not defined. IIoT deployments require deterministic behaviour in these cases, which in turn require the definition of a mechanism to determine the traffic to prioritise.

If two overlapping grants have been provided dynamically by the gNB, the scheduler is aware that the latter grant overlaps with the former. It is a safe assumption that the gNB has determined that the latter grant is of higher importance for it to be scheduled in conflict with an earlier grant. Therefore the UE ought to prioritise the latter grant and cancel the former transmission.

Proposal 1: In case of overlapping dynamically provided UL grants, the latest grant is prioritised by the UE.

Figure 1: Prioritisation mechanism for configured grants

In case one of the overlapping grants is a configured grant, the gNB has limited visibility into its usage, i.e. it does not know if the configured grant, that may or may not be used, contains higher priority data than other grants. This is especially true for the case of multiple overlapping configured grants over which the scheduler exercises limited control. In these cases, as the UE has a better view of the category of traffic included in these grants, it is well placed to decide on the grant to be used.

A simple way to do so is with the usage of the logical channel (LCH) priorities as shown in Figure 1. LCH priorities map to the priority of the traffic carried, and is used to determine the order in which traffic is served when a grant is received. The UE can use this priority information to determine the grant to be used. For example, if as a result of LCH restrictions, a configured grant G1 contains a LCH that bears a higher priority than the LCHs multiplexed into another overlapping grant G2, G1 is prioritised over G2.

Proposal 2: The priorities of the LCHs included in a grant determine whether a configured grant can be prioritised over another overlapping grant.

## 2.2 Prioritisation of Scheduling Request over UL data

When high priority traffic is sporadic in nature, a Scheduling Request (SR) is needed to indicate to the gNB that high priority data is available for transmission. The requirement of an SR indication before the UL data can be transmitted introduces delays to the transmission of the UL data. If the SR cannot be transmitted due to an overlapping transmission, the delay in UL data transmission can exceed the delay budget. For this reason, a means to prioritise SR for high priority channels over UL data transmissions need to be considered.

Figure 2: Prioritisation mechanism for scheduling requests

Similar to the method described in section 2.1, LCH priorities can be used to determine whether an SR is prioritised as shown in Figure 2. For example, an SR triggered for LCH1, which is of a higher priority than the highest priority LCH multiplexed into an overlapping grant G1, is prioritised over the overlapping grant. As the gNB scheduler is unaware of the time at which the SR is triggered, a prioritised SR should be prioritised over all types of grants.

Proposal 3: The priority of the LCH that triggered the SR is compared with the priority of the LCH included in a grant, to determine whether an SR can be prioritised over an overlapping grant.

## 2.3 Other control information

Other information transmitted over the uplink are CSI reports and HARQ feedback. If this information is not transmitted, only resource efficiency would be affected. For example, if the HARQ is missed, the gNB would assume that the TB on the DL was not decoded and retransmits the TB to the UE. Similarly if the CSI report is missing, DL resource allocation may have to compensate for this missing information. Given that the impact of the loss of such control information is limited to resource efficiency, while the impact of the loss of high priority data is a failure to meet its delay budget requirements, data on LCHs carrying high priority information, or its associated SR should be prioritised over other control information.

Proposal 4: UL data transmission from a high priority LCH should be prioritised over the transmission of HARQ feedback and CSI reports.

# 3 Conclusion

In this contribution we make the following proposals:

Proposal 1: In case of overlapping dynamically provided UL grants, the latest grant is prioritised by the UE.

Proposal 2: The priorities of the LCHs included in a grant determine whether a configured grant can be prioritised over another overlapping grant.

Proposal 3: The priority of the LCH that triggered the SR is compared with the priority of the LCH included in a grant, to determine whether an SR can be prioritised over an overlapping grant.

Proposal 4: UL data transmission from a high priority LCH should be prioritised over the transmission of HARQ feedback and CSI reports.

# 4 References

- [104#38][NR/IIOT] Intra UE prioritization UL Data Data (Interdigital)

- [104#39][NR/IIOT] Intra UE prioritization UL Control Data (Huawei)