3GPP TSG-RAN WG2 #105	R2-1901185

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	12.2.6

Source:	Ericsson [rapporteur]

Title:	[NB-IoT] Email discussion report on RLF for SON

Document for:	Discussion, Decision

# Introduction

At RAN#80, the Rel-16 work item on Rel-16 enhancements for NB-IoT was approved [1]. One of the objectives in this work item is to support SON reporting for random access performance and radio link failure for network management.

Network management tool enhancement: - SON support for reporting of [RAN2, RAN3] - Random access performance - Radio link failure (RLF), if needed

It has been agreed to progess this part through an email discussion.

[104#46][NB-IoT R16] RLF Report (Ericsson)

Purpose and content of RLF report.

Intended outcome: email discussion report

Deadline: Thursday 2019-02-07

The deadline of the email discussion is Thursday 2019-02-07; companies are welcome to provide their comments before the deadline to conclude the discussion (with a set of proposals) on time.

Below is the agreement from RAN2 on SON.

RAN2#103bis agreements: - ANR reporting for NB-IoT only uses idle-mode measurements (i.e. we won’t introduce connected mode measurements) - Support RACH report for NB-IoT - Will consider whether information in LTE RACH report is extended RAN2#104 agreements: SON-ANR: - RAN2 understanding is that the purpose of SON/ANR reporting in NB-IoT is network optimisation rather than immediately updating neighbour relations like with LTE ANR, and is therefore not time critical. - SON reporting does not trigger RRC connection establishment/resume - FFS whether this includes EDT. - SON information can be reported along with EDT, FFS what and how. - RACH report: - In addition to legacy parameters for RACH reporting, the first selected resource pool (E.g. CE level, EDT) is included in the RACH report. This agreement is also applicable for eMTC. - TDD and FDD NB-IoT are distinguished by the relative location of NPSS and NSSS.

# Discussion

In LTE, the RLF Report from the UE is primarly used for mobility robustness optimization, however it is also possible to use it for coverage optimization, i.e. to discover poor network coverage. The UE stores the latest RLF related information and indicates RLF report availability at each subsequent LTE RRC connection (re-)establishment until the RLF report is fetched by the network or for 48 hours after the RLF or handover failure is detected. In LTE, UE appends the Location information along with time stamp when the RLF occurred. Thus, a deferred reporting of RLF works without any problem in identifying the region of coverage issue.

Procedure to indicate RLF report in LTE is depicted in below diagram.

Figure.1 UE reporting procedure for RLF

In NB-IoT, connected mode measurements, mobility, handover etc. is not supported. Network in many cases is unaware of prevailing power and quality level of signals received by UE. In such cases, when Radio Link Failure occurs, the RLF failure report may help network to discover the coverage hole and provide means to enhance the network performance. However, NB-IoT device may not append the location info all the time, in such cases would an immediate SON reporting be desired?

Considering that in most cases re-establishment attempt would be triggered soon after RLF is declared, i.e. not with the intention to report SON (coverage issues), but to attempt to re-establish; should reporting of RLF be appended with the re-establishment attempt?

Question 1: 	Companies are encouraged to provide their view whether immediate reporting of RLF be supported, considering that the location info may not be available or would be power consuming to obtain one.

| Company | Views on Immediate Reporting of RLF |
| --- | --- |
| ZTE | As mentioned by rapporteur, the RLF report in NB-IoT may mainly help network to discover the coverage hole and provide means to enhance the network performance. These are not urgent problems but long-term optimization targets. Therefore, we think new kind of immediate RLF reporting is not necessary. In order to reduce UE power consumption, the RLF could be logged and reported later at subsequent LTE RRC connection (re-)establishment. For RLF reporting along with the later LTE RRC connection (re-)establishment, PCI information may be not enough to identify the cell that triggers the RLF, e.g., the problem cell. We think the CGI can be included in the reporting for identifying the problem cell. But the location information reported in legacy LTE is not needed for NB-IoT. |
| Huawei, HiSilicon | We think that reporting at the time of re-establishment should be the baseline. 1. We cannot understand the comment ‘considering that the location info may not be available or would be power consuming to obtain one’. In our understanding, in LTE, the information in the RLF report is stored, if available, at the time RLF is detected and the report has the same contents regardless of the report being sent after the connection re-establishment or in a later connection establishment (at the exception of an additional time information). 2. Once RLF occurs, the UE performs RRC Connection Re-establishment (except for a UE supporting the CP solution but not the CP re-establishment procedure), so we think that reporting the RLF report at this time is the right time. It has the benefit of minimising the amount of information to be reported, e.g. the eNB can translate the PCI, included in the RRC Connection Re-establishment Request, to the full Cell ID. We cannot understand the benefit of logging the information in the UE for later reporting. 3. We are open to discuss the case where the re-establishment fails or cannot be initiated, but we think these are rare cases and the RLF report is used for optimisation purpose, thus it is not critical if in some cases the information is not reported. |
| NEC | We think in LTE, exactly when to collect the RLF report is up to network implementation given the UE informs the network of the RLF information availability (rlf-InfoAvailable) at the time of RRC connection (re-)establishment. This can be the baseline. Possible modification e.g. considering battery consumption is discussed under Question 2. Regarding the location information availability, it might be true that the NB-IoT UE does not have/obtain the location info at RLF detection in some cases. This is the same as LTE RLF report as Huawei commented. As far as the cell can be identified by the network, the RLF report is useful. Then, for identifying the cell, we tend to agree with ZTE for inclusion of CGI. |
| Nokia | When UE performs RRC Connection Re-establishment on detection of RLF, The Re-establishment message already contains key information related to the RLF event. Additional information to be included in this report needs to be analysed based on the required actions at UE. For example, the neighbour cell report inclusion in the RLF report may require additional measurement to be performed by UE at the time of Re-establishment which can if avoided if not required from NB-IoT UE. In case If the UE fails in Re-establishment, the UE can include this failure report when later establish RRC connection with the network. In this case also the information contents and actions required at UE for such reporting needs to be as minimum as possible. |
| Lenovo | In legacy LTE network, UE will report RLF report availability(rlf-InfoAvailable) in RRC connection (re-)establishment. Then network will ask for UE RLF report by request/reponse procedure based on this availability indication. It is network decision when RLF report will be transmitted to network. For NB-IOT network, RLF report is not urgent information, it is desired that the RLF report available indication will also be used in RRC connection (re-)establishment procedure to let network decide whether RLF reporting is required. |
| Ericsson | The question related to location was mainly to specify that if RLF is reported without location info it may be difficult to localize the problematic coverage area and the problem could be located at cell level only. Even though NB-IoT UE supports OTDOA based positioning method, there is no guarantee that UE would send location info. Generally, the cell size of NB-IoT cell would be larger/deeper than the LTE cell. Thus, it would be beneficial to get position co-ordinates of the failure location. We agree that the legacy LTE mechanism should be used and it is up to the NW to get the RLF report once UE will report that rlf report is available. |
| Deutsche Telekom | We think that the RLF reporting as we have it in LTE should be the baseline. There is no urgency geeting the RLF reports, so no need for immediate reporting, but using the scheme of LTE RLF reporting where at restablishment (or at next RRC connection setup) the UE indicates availability of the RLF report to the nextwork. It would be also helpful to complement the RLF report with some information about the CE level the UE used at RLF. This is especially helpful for coverage optimisation (RLF might happen in CE0, CE1 or CE2 for different reasons). RLF reporting should be mandatory for a NB-IoT UE. A single RLF report storage should be acceptable to minimise the efforts for the UE implementation. The 48h limit should also be extended as many of the NB-IoT devices will hardly be active once a day … 480 h seems OK. Additional information (e.g. neighbour cell measurements, available location information, time) should be considered to be included. |
| Qualcomm | While we understand that in most cases, upon RLF, UE will go through process of reestablishing a connection, we think that UE should not be required to establish an RRC connection or start an EDT session for the sole purpose of reporting RLF report. LTE RLF reporting mechanism can be taken as baseline. I.e., availability of RLF report is indicated to the network, and network uses UE info request/response mechanism to retrieve the report. In that respect, “immediate reporting” is not required; it is reported upon request from NW. |
| Intel | We aslo think the LTE RLF reporting mechanism should be basedline. As also discussed in another email discussion, same mechanism can be used for RLF report, RACH report or any other report. |

------------------

Question 2: 		Complementary to above Question 1; Companies are encouraged to provide their detailed view on When should the RLF report be sent? One of the agreement made is “SON information can be reported along with EDT”, so this should be also taken into account.

- As in legacy, shown above in Figure 1.

- RLF is logged and reported later at convenience by the UE using EDT

- Possibility to have Both Options. For Non-EDT UE Option a) is preferred and For EDT UE Option b) is preferred.

- Any other. Please illustrate

| Company | When should the RLF report be sent |
| --- | --- |
| ZTE | We prefer d): RLF is logged and reported later at subsequent LTE RRC connection (re-)establishment. Different from SON/ANR report, in most cases, the RLF will trigger the RRC re-establishment that provides a suitable reporting opportunity for RLF records, e.g, in Msg5, RRCConnectionReestablishmentComplete message If RRC re-establishment is not triggered, the UE could just store the RLF records for later report. If RRC re-establishment is not successful, UE could make use of later RRC procedures to report. The following options can be considered: - If EDT is triggered, e.g., by other application service, - If the related reporting contents are not too large and can be accommodated in EDT Msg3, the stored RLF records can be reported in EDT Msg3; - If the stored RLF records are too large to be accommodated in EDT Msg3, the UE can only report RLF-Available indication in EDT Msg3, and eNB can fallback the EDT procedure to legacy procedure if the eNB wants the RLF reporting. - If legacy RRC connection establishment procedure is triggered, the stored RLF records can be reported in Msg5, e.g, alone with RRCConnectionSetupComplete or RRCConnectionResumeComplete messages. |
| Huawei, HiSilicon | Based on our reply to question 1, where the RLF report is reported at the time of the re-establishment procedure, we think the question on EDT is irrelevant. In case we agree to report later in the case re-establishment does not happen, we do not think the report should be reported via EDT. EDT has been introduced to optimise the UE power consumption by avoiding the transition to RRC_CONNECTED for UEs only have one UL data packet, we should keep it as it is. |
| NEC | Except for EDT, as responsed to Question 1, we think the option a) legacy approach should be considered as baseline by introducing the rlf-InfoAvailable in Msg5 (e.g. RRCConnectionReestablishmentComplete-NB). If the battery consumption is seen as a problem, the RLF report can be directly included in the Msg5. However, since the RLF detection would be a rare case, we do not see critical issue in legacy approach. If RAN2 would go for Msg5 approach, then it should be discussed whether a network control (e.g. 1 bit SIB indication) is necessary or not for RLF reporting (considering this is the network control in LTE). For EDT, we can understand that the EDT is extremely low power consumption and it would be good to keep as it is. However, as the RLF would be rare, the fall back from EDT for RLF reporting would be acceptable. To achieve this, the rlf-InfoAvailable can be included in the Msg 3 (RRCEarlyDataRequest-NB) instead of Msg5. In summary, an alternative optin e) can be as follows: - When the RLF is detected, the UE stores the RLF information. - When the UE perfoms the RRC connection (re-)establishment for non-EDT after the RLF, the UE includes the RLF information in Msg5. - When the UE perfoms the RRC connection (re-)establishment for EDT after the RLF, the UE includes the rlf-InfoAvailable in Msg3 (RRCEarlyDataRequest-NB). - if the UE is instructed to fall back by RRCConnectionSetup-NB including a request for RLF information, the UE includes the RLF information in RRCConnectionSetupComplete-NB. Additionall, we may consider the 1 bit SIB indication for network control to perform the actions above. |
| Nokia | The RLF information stored at UE in cases where re-establishment was not successful can be reported as part of next RRC connection establishment and also as part of next EDT attempt if the remaining bits in the EDT payload is sufficient to report this event. In our view it should be option b for EDT UE and for non-EDT UE also it can be optimised over a. UE can report the RLF without waiting for UE information request in the new connection. This will reduce one additional downlink RRC signaling. |
| Lenovo | We think it is possible that UE report its logged RLF information by EDT procedure at convenience. Considering that UL grant in EDT Msg.3 may be small, the procedure could be: - When UE perfoms the RRC connection (re-)establishment for EDT after the RLF, or UE performs EDT for data, the rlf-InfoAvailable will be considered in Msg3 (RRCEarlyDataRequest-NB). - if UE is configured to fall back by RRCConnectionSetup including a request for RLF information, UE will response the RLF information in RRCConnectionSetupComplete. |
| Ericsson | We agree with Huawei that using EDT procedure to report may not be relevant here. Moreover, if UE appends the data and report together, it will require additional complexity to delinate this information to be processed by eNB. Considering it is NAS container and transparent to eNB, EDT procedure may not be suited for this. We agree that report can be sent using the legacy RRC connection establishment procedure, the stored RLF records can be reported in Msg5, e.g, alone with RRCConnectionSetupComplete or RRCConnectionResumeComplete messages. |
| Deutsche Telekom | As in legacy LTE. No need to include it into the EDT message. RRConnectionSetupComplete seems to be a possible choice. |
| Qualcomm | As indicated in Q1: UE should not be required to establish an RRC connection or start an EDT session for the sole purpose of reporting RLF report. We prefer option c with modification: For non-EDT: similar to LTE. If UE initiates EDT for reasons other than solely to report RLF: the UE can report RLF-Available indication in EDT Msg3, and eNB can fallback the EDT procedure to legacy procedure if the eNB wants the RLF reporting. Including the report itself in EDT Msg3 should be avoided to reduce complexity and due to RRC part of EDT msg3 not being integrity protected. |
| Intel | In our view option “a” is sufficient as in LTE the stored RLF report could be sent in UL DCCH. But in UP-EDT, the stored RLF report can be multiplexed with UP data in Msg3. However, EDT should not be missued to go to RRC_CONNECTED. |

------------------

Currently for LTE an extensive RLF reporting is done which includes the parameters below.

- Absolute Time Stamp

- PCI (failed PCI Cell and/Or Last Serving Cell PCI):

- CGI

- C-RNTI

- Last Serving Cell RSRP/RSRQ

- Neighbor cell RSRP/RSRQ

- Location Info

Question 3: 	Companies are encouraged to provide their input regarding which of the parameters provided above should be included in the RLF report and why, especially if it is proposed to exclude any of the parameters above.

| Company | Parameters in RLF report |
| --- | --- |
| ZTE | The following legacy information should be included: - Absolute Time Stamp - PCI (failed PCI Cell and/Or Last Serving Cell PCI): - CGI e) Last Serving Cell RSRP/RSRQ f) Neighbor cell RSRP/RSRQ We understand these information are enough to identify the problem cell, the time when the RLF problem occurred, the coverage quality of the problem cell and its surrounding coverage situation at that time. For C-RNTI, as the eNB would not use the RLF reporting information to optimize per-UE mobiliby issue, we think such UE identity is not needed. For Location Info, taken into account that it requires additional position measurement, reporting Location Info is also not suitable for power sensitive NB-IoT UE. |
| Huawei, HiSilicon | The absolute time stamp is not needed if the reporting take place at the time of the re-establishment. The CGI is not needed for the UP solution (and not included in LTE) as it can be derived from the PCI in the RRCConnectionReestablishmentRequest message. It will be needed for the CP solution. CRNTI is not needed as the purpose of the report is coverage optimisation not mobility robustness. Neighbour cell measurements are not available as, when RLF occurs, the UE is in connected mode. Location information can be optional as for LTE, but this will only available if some devices have standalone positioning capability (e.g. they are equipped with a GPS chip) |
| NEC | The legacy parameters a), b), c), e), f) and g) shoud be included. For d) C-RNTI, we also think this is not necessary as this is to be used for MRO (mobility) purpose in LTE. For g) location info, we agree with Huawei that this should be included if available. |
| Nokia | b) c) e) and f) are required. Failed cell information also may be required in case if the RLF is reported later when re-estbalishment fails. Other information element can be optional. C-RNTI information of last cell is not required as UE can be identified based on RRC-Resume-Id for UP solution across cells. |
| Lenovo | The parameters a), b) , c), e), f) should be included, g) is optional based on UE capability. - C-RNTI is for mobility robustness, it is not necessary for NB-IOT network. |
| Ericsson | We agree with Lenovo. |
| Deutsche Telekom | Following should be included: - Absolute Time Stamp - PCI (failed PCI Cell and/Or Last Serving Cell PCI): - CGI - Last Serving Cell RSRP/RSRQ - Neighbor cell RSRP/RSRQ - Location Info (OPTIONAL, e.g. if NB-IoT device has GNSS capabilities) New: information about the CE level. |
| Qualcomm | Following should be included: - ECGI should be included. - Last Serving Cell RSRP/RSRQ should be included Following are optional: - absolute time stamp can be beneficial (optional). - Location info (optional) Following are not needed: - PCI is not needed as ECGI can uniquely identify the cell - Neighbor cell RSRP/RSRQ not needed. |
| Intel | We think a), c), e) need to be considered. We are not sure on the use of neighbor cell measurement for NB-IoT (f) (other than network optimization). PCI (b) and C-RNTI (d) are not needed. Location info (g) can be optional. |

------------------

Question 4:	Are there any NB-IoT specific content that can be relevant to include in the RLF report? Companies are encouraged to provide any additional parameters compared to the ones above stating why it would be beneficial.

| Company | Additional parameters in RLF report |
| --- | --- |
| ZTE | We propose to add PCI of target cell, e.g., reestablishmentCellId in the legacy RLF-Report to this NB-IoT RLF reporting. Such parameter would be useful to identify the most suitable target cell when RLF occurs. |
| Huawei, HiSilicon | None if the reporting take place during the re-establishment. The eNB knows the information of the target cell. The re-establishment cell ID (PCI, CGI) if we enable reporting at connection establishment. |
| Nokia | The CE level assigned to the UE prior to the RLF could be useful information. Because RLF due to wrong assignment of CE level or number of repetitions can be identified with this information. |
| Deutsche Telekom | CE level (as also proposed by Nokia) - Might be discussed if CE level configuration for RLF should be included .. e.g. RLF only for certain CE levels. - Might also be discussed if area configuration for RLF should be included .. e.g. based on TAC, Cell ID as for logged MDT in LTE ? - We do want to optimise our networks, but we do not want to put to much burden on the NB-IoT devices. E.g. selectively activating it might be a good compromise. Re-establishment cell ID (PCI, CGI) (as also proposed by Huawei/HiSilicon) Anything which is related to the UPLINK loss as NB-IoT is mainly UL focussed and RLF from LTE was mainly downlink focussed a different approach is needed here. For Uplink failure reporting the UE Power Class is needed as information for the report (e.g. it might be different if the RLF happened for a 14 dBm or 20/23dBm UE) |
| Qualcomm | Coverage level that was used to establish the connection where RLF took place. |

------------------

Question 5:	Which mode of EDT is preferred: Immediate or Logged reporting

- UE can append the RLF report assuming that it fits to one of the potential UL grants provided for EDT.

- When location information is unavailable or if buffer is full or after certain duration (e.g. 24 hours has passed since last data transmission), then the UE provides the report using EDT.

| Company | mode of reporting for EDT (immediate or logged) |
| --- | --- |
| ZTE | According to our comments for Question 2 and Question 3, we think Logged reporting with option a) could be used in some scenarios. For option b), we think it’s not needed as we have mentioned such RLF reporting is not urgent for network coverage optimization. If buffer is full, the UE can just replace the oldest record with the latest one. |
| Huawei, HiSilicon | Based on our answer to question 1 and 2, EDT should not be used for RLF reporting. Even if we agree to referre reporting for the case where re-establishment does not happen, we don’t want to add any additional complexity to EDT. RLF reporting can be done in connected mode. |
| NEC | As responsed to Question 1 and 2, if the EDT is triggered when the UE stores the RLF information, the UE shall indicates the rlf-InfoAvailable in RRCEarlyDataRequest-NB and then it is up to network whether to retrieve it at that time or later (e.g., subsequet connection setup). |
| Nokia | If the UE is EDT capable including this information if it fits within EDT paloyload is prefered. Triggering EDT just for sending RLF report is not prefered. |
| Lenovo | The same view as NEC. If EDT is used for RLF reporting as UE in EDT mode, the rlf-InfoAvailable is considered in Msg.3, then network will decide the time when UE reports the RLF information. |
| Ericsson | We agree with Huawei. EDT should not be used for RLF reporting. Reporting can be done when UE gets connected using the legacy RLF reporting procedure. |
| Deutsche Telekom | No need to use EDT |
| Qualcomm | As explained in Q1 and Q2, option b is not preferred. UE should not be required to establish an RRC connection or start an EDT session for the sole purpose of reporting RLF report. UE can store the report from the latest RLF. We think immediately last RLF is enough. For non-EDT: similar to LTE. If UE initiates EDT for reasons other than solely to report RLF: the UE can report RLF-Available indication in EDT Msg3, and eNB can fallback the EDT procedure to legacy procedure if the eNB wants the RLF reporting. Including the report itself in EDT Msg3 should be avoided to reduce complexity and due to RRC part of EDT msg3 not being integrity protected. |
| Intel | EDT initiating condition should not be changed and we do not prefer to add any complexity to EDT. If it wants to go to RRC_CONNECTED, it simply can use the non-EDT. |

------------------

Question 6:	How is the content relayed to the network from a UE that does not support or use EDT?

- Introduce a new unsolicited RRC message for RLF-NB

- Appending the failure report in Re-establishmentRequest message

- Reuse legacy procedure to report RLF as shown below and mentioned in [5]

Figure.2 UE reporting procedure for RLF

- Other. Please specify.

| Company | How for non-EDT |
| --- | --- |
| ZTE | - Msg5. According to our comments for Question 2, Msg5, e.g., RRCConnectionReestablishmentComplete, RRCConnectionSetupComplete or RRCConnectionResumeComplete messages are suitable, which can save UE power consumption mostly. |
| Huawei, HiSilicon | We have a preference for option d) MSG5. We are not so much concerned about the additional power consumption as this should be a rare event, but we would like to keep the UE complexity low and not introduce new procedures unless absolutely necessary. Again we cannot understand why EDT comes here. |
| NEC | We also think d) Msg5 seems better. On top of this, we would like to here other companies view on whether any network control is necessary or not, as commented in answer to the Question 2. For example, if the network sends 1 bit SIB indication for RLF report, then the UE having RLF information sends it in the Msg5. Otherwise, the UE does not send. |
| Nokia | Msg5 is better choice to reduce the overall ignalling associated with the RLF reporting for non-EDT. The ENB can control the contents if it has to be filtered via system information if required. |
| Lenovo | We prefer option.d) Msg.5. We think network control by 1 bit SIB for RLF reporting is not necessary, it will trigger all of UE in the cell read the system information for this one-shot option, it is power wasting and unefficient. |
| Ericsson | Yes Msg5. |
| Deutsche Telekom | Msg5 |
| Qualcomm | Option c, i.e., as described in Lenovo’s document [5] Considering that there are other SON reports also under discussion, we think UE information request/response mechanim should be introduced for NB-IoT. The information that will likely be transmitted with such mechanism are at least (subject of 3 different email discussions): RACH report, RLF report, ANR. We should strive to design a common framework for all SON reports, instead of optimizing for each report separately in different ways. |
| Intel | Option C: We also prefer to use a unified approach for RLF report, and other possible reports. For Msg5, new indication of report availability in Msg3 may be required as RLF is not frequent. |

------------------

Should the CP Reestablishment Request message be extended to include the PCI and C-RNTI information? In legacy, for UP ReestablishmentRequest, C-RNTI and PCI are included as part of that message, however for CP they are not included. In order to include these parameters in CP Reestablishment Request message, this would require that a new Rel-16 version of this message is defined as there are not enough spare bits.

Question 7:	Companies are requested to provide their input on this. Should C-RNTI and PCI be included in CP Reestablishment message, if yes, how should this be supported. Companies are encouraged to provide their opinion.

| Company | Supporting RLF parameter in CP Reestablishment Request message |
| --- | --- |
| ZTE | No. except the case that RLF reporting along with EDT, we don’t think Msg3, e.g,, CP Reestablishment Request message would be used for including RLF reporting information. Since the size of Msg3 (in non-EDT case) is very limited, generally we only consider including very important information for RRC connection setup in Msg3. Moreover, there has no obvious timeliness difference for RLF reporting between in Msg5 and in Msg3, we cannot see the obvious reason to extend Msg3. |
| Huawei, HiSilicon | No. The PCI (and possibly CGI) can be included in the RLF report. The CRNTI is not needed. |
| NEC | No. We tend to agree with ZTE. |
| Nokia | CGI/PCI are sufficient. C-RNTI is not required as there is no context transfer involved in case of CP solution. |
| Lenovo | No. We agree to ZTE view. |
| Ericsson | We agree with Huawei. |
| Deutsche Telekom | Agree with Huawei. |
| Qualcomm | No. Such information is part of RLF as proposed in Q3 |
| Intel | No, Please see answer in Q3. |

------------------

# Summary

The following companies (9) participated in the email discussion: ZTE, Huawei, HiSilicon, NEC, Nokia, Lenovo, Ericsson, Deutsche Telekom, Qualcomm, Intel. Based on the comments provided, the following observations can be made:

Question 1:

Nine companies have answered. All companies think there is no need for new mechanism for RLF report, which would take place before the subsequent RRC connection (re-)establishment following an RLF.

Three companies propose that the subsequent RRC connection re-establishment procedure following the RLF is used to send the RLF report.

Six other companies suggest that the UE should send an indication (rlf-InfoAvailable) during the following RRC connection re-establishment process to the eNB about having an available RLF report, and then it is up to the network when to request for this report. This would be similar mechanism as in legacy LTE.

It is noted that the initial re-establishment following an RLF fails or cannot be initiated rarely, however the operation in these situation can be discussed. However, optimization of such rare case is not critical.

Question 2:

Nine companies have answered. Three companies think that the RLF report is send during the initial RRC re-establishment procedure, one of which explicitly state that the procedure is optimized such that the network doesn’t need to request for the report separately. Five companies propose the legacy approach as a baseline, where rlf-InfoAvailable indication is included in Msg5 of the RRC Connection re-establishment procedure, and the RLF report is then received via legacy UEInformationRequest/UEInformationResponse procedure when the network decides to request for it.

Three companies state that there is no need to use EDT. Three companies propose having rlf-InfoAvailable indication in Msg3 of EDT procedure, and then the network may choose to fallback to legacy procedure from EDT in case it wants to request the RLF report. Three companies suggest that the UE may include the RLF report into the EDT Msg3, if the remaining bits available are sufficient.

Based on the answers to questions 1 and 2, we propose the following:

- The UE informs network about having an RLF report available via rlf-InfoAvailable indication during the initial RRC connection re-establishment process after RLF. The rlf-InfoAvailable parameter is included in Msg5 of the RRC connection re-establishment process.

It is up to the network to decide when to request for the RLF report, using UEInformationRequest/UEInformationResponse procedure.

In case the initial RRC connection re-establishment fails or cannot be initiated, the operation can be further discussed in RAN2. For simplicity, however, we propose similar behaviour as in above proposal:

In case of no/failed initial RRC connection re-establishment, the UE may store the RLF report for later connection (re-)establishment, where the UE similarly indicates to the network about having an available RLF report via rlf-InfoAvailable indication.

Regarding EDT, the summary and proposals are made in question 5.

Question 3:

Nine companies have answered. See the following table as a summary for the required legacy parameters:

| Option | Supporting companies | Comments |
| --- | --- | --- |
| a) Absolute Time Stamp | 7 companies support (2 of which optionally) |  |
| b) PCI (failed PCI Cell and/Or Last Serving Cell PCI): | 7 companies support | Two companies think PCI is not needed. |
| c) CGI | 8 companies support and 1 company supports conditionally | One company noted that is not needed for UP solution, while is needed for CP solution |
| d) C-RNTI | No companies support (1 company didn’t state whether supports or not) | Related to mobility robustness, therefore not needed for NB-IoT. |
| e) Last Serving Cell RSRP/RSRQ | 9 companies support |  |
| f) Neighbor cell RSRP/RSRQ | 6 companies support | One company noted that RLF occurs when UE is in connected mode, thus no neighbour measurements available |
| g) Location Info | 8 companies support optionally | Requires positioning capability from the UE, and may be power consuming, thus optional |

Based on the answers to this question, we propose the following:

The following legacy parameters are supported for RLF reporting in NB-IoT: 1) Absolute Time Stamp, 2) PCI (failed PCI Cell and/Or Last Serving Cell PCI), 3) CGI, 4) Last Serving Cell RSRP/RSRQ, 5) Neighbor cell RSRP/RSRQ (OPTIONALLY if available), 6) Location Info (OPTIONALLY).

Question 4:

Rapporteur summary: Five companies have answered. Three companies propose adding PCI and/or CGI of target cell reestablishmentCellId (in case RLF reporting in RRC connection establishment is allowed). Three companies propose including coverage enhancement level information. One company proposes including information related to UPLINK loss, such as UE power class.

Based on the answers to this question, we propose the following:

The re-establishment cell ID (PCI, CGI) and CE level assigned to the UE prior to the RLF is included in the RLF report.

RAN2 to discuss during the meeting about any further information added into the RLF report.

Question 5:

Rapporteur summary: Nine companies have answered. Three companies think that there is no need to use EDT. Two companies think that if RLF report fits within EDT payload, the UE may append the RLF report into the EDT payload. Three companies think that the UE shall indicate rlf-InfoAvailable indication into the Msg3 of EDT procedure, and it is up to the network when to retrieve the report. It is also noted by one company that EDT should not be triggered solely for the purpose of RLF report.

Regarding EDT, based on the answers to questions 1,2 and 5, we have two alternative proposals:

EDT is not supported for RLF reporting.

Alternatively,

rlf-InfoAvailable indication is included in the Msg3 of the EDT procedure to indicate to the network about an available RLF report.

Question 6:

Nine companies have answered, and eight companies state that Msg5 could be used. Considering the answers to questions 1 and 2, there are some confusion on the intended behaviour what each company mean, whether the RLF report itself is included in Msg5, or whether the rlf-InfoAvailable indication is included in Msg5. All companies however think either that the RLF report should be included in Msg5 of the RRC Connection (re-)establishment procedure, or that the UE should inform the availability of RLF report, and the network requests for the report using UEInformationRequest/UEInformationResponse procedure as in legacy LTE. Taking into account the responses to questions 1, 2, please refer to the Proposals 1-3.

Question 7:

- Rapporteur summary: 7 companies have answered. All companies agree that C-RNTI is not needed to be included, and that the PCI and possibly CGI is sufficient. It is also noted that except for EDT, Msg3 should not be used for RLF reporting, and thus, Msg3 is not needed to be extended.

# Conclusion

Based on the discussion in section 3, we have the following proposals:

Proposal 1 The UE informs network about having an RLF report available via rlf-InfoAvailable indication during the initial RRC connection re-establishment process after RLF. The rlf-InfoAvailable parameter is included in Msg5 of the RRC connection re-establishment process.

Proposal 2 It is up to the network to decide when to request for the RLF report, using UEInformationRequest/UEInformationResponse procedure.

Proposal 3 In case of no/failed initial RRC connection re-establishment, the UE may store the RLF report for later connection (re-)establishment, where the UE similarly indicates to the network about having an available RLF report via rlf-InfoAvailable indication.

Proposal 4 The following legacy parameters are supported for RLF reporting in NB-IoT: 1) Absolute Time Stamp, 2) PCI (failed PCI Cell and/Or Last Serving Cell PCI), 3) CGI, 4) Last Serving Cell RSRP/RSRQ, 5) Neighbor cell RSRP/RSRQ (OPTIONALLY if available), 6) Location Info (OPTIONALLY).

Proposal 5 The re-establishment cell ID (PCI, CGI) and CE level assigned to the UE prior to the RLF is included in the RLF report.

Proposal 6 RAN2 to discuss during the meeting about any further information added into the RLF report.

Proposal 7 EDT is not supported for RLF reporting.

Proposal 8 rlf-InfoAvailable indication is included in the Msg3 of the EDT procedure to indicate to the network about an available RLF report.

# References

RP-181451, “New WID on Rel-16 NB-IoT”, RAN#80.

R2-1817558	Discussion on UE reporting the SON parameters	Ericsson

R2-1816403	Discussion on RLF report	Huawei, HiSilicon

R2-1817784	Further consideration on RLF and ANR report	ZTE Corporation

R2-1817549	Reporting of RA and RLF in NB-IOT SON	Lenovo, Motorola Mobility