3GPP TSG-RAN2 Meeting #105	R2-1901272

Athens, Greece, 25 February - 1 March 2019

Agenda Item:	11.11.4 Power saving in RRC_CONNECTED

Source: 	Ericsson

Title:  	Power efficient CA and DC

Document for:	Discussion and Decision

# Introduction

Carrier Aggregation (CA) and Dual Connectivity (DC) enable the NW to configure additional capacity for a connection when needed. However when SCells for CA/DC are activated, but not used to carry any traffic, this implies additional power consumption for the UE. Even when the SCell is configured and deactivated power is consumed to perform RRM measurements. Possible power saving enhancements for CA/DC are discussed in this contribution.

# Background

NR:

When an SCell is added via RRC signalling, the SCell is deactivated. The SCell can be activated/deactivated through Activation/Deactivation MAC CE signaling or upon expiration of the sCellDeactivationTimer timer.

When the SCell is deactivated the UE is not required to monitor PDCCH, not required to report CSI measurements and SRS, UL-SCH, RACH and PUCH transmissions are not supported. However the UE is required to measure and report RRM measurements according to the measurement object configured for the SCell.

In section 8.3 in 38.133 UE requirements for SCell activation and deactivation delay, i.e. the delay between receiving SCell activation command and transmission of valid CSI report, are specified. The activation delay depends on whether the cell is known (has been detected) or not, whether the cell is in FR1 or FR2, the SCell measurement cycle, and the SMTC periodicity.

LTE:

In LTE an SCell can be activated or put into dormant state immediately upon configuration. In dormant state [1] the UE is not required to monitor PDCCH, similar as in deactivated state, but the UE is required to measure and report RRM and CQI. The UE can be configured with a (short) CQI reporting cycle, to enable a faster transition to activated state compared to coming from deactivated state. The dormant SCell state is more power consuming than when the SCell is deactivated, but it allows a faster activation.

The UE can also be configured to perform measurements in Idle mode for CA with measIdleConfig IE in the RRCConnectionRelease message for a certain time indicated by T331. The UE can indicate available Idle mode measurements during connection setup with idleMeasAvailable IE, and the NW can retrieve the available measurements via UEInformationRequest message.

REL-16 CA/DC enhancements

CA/DC enhancements in NR, which include among other signalling enhancements and early idle mode measurement reporting, are evaluated in the REL-16 CA/DC work item [2].

# Discussion

The two main aspects, which determine whether an SCell should be activated are:

- Traffic demand

- SCell quality

In case information about 1 and/or 2 is missing, it is possible that SCells are activated but they are not needed or when they are not suitable to carry data.

Traffic demand:

BSR reporting indicates the instantaneous UL buffer status in the UE to the gNB. BSR is reported when the first data enters the UL buffer, periodically when periodicBSR-Timer expires or re-transmitted when retxBSR-Timer expires. In case TCP is used the UL buffer can be expected to follow the available end-to-end throughput, e.g. due to Slow Start and Congestion Avoidance features. The reported BSR values, including the detected trends in the report BSR values, are a valuable tool to trigger activation and deactivation of SCells.

For NB-IoT, in the context of UE differentiation, signalling has been discussed that could be beneficial for the eNB to know to schedule and configure the NB-IoT UE. Two approaches have been discussed:

- Signalling from CN based on UE subscription info

- Signalling where eNB is able to store and retrieve a UE profile transparently in MME

UE subscription info signalling was agreed, i.e. the agreed Subscription Based UE Differentiation Information, signaled on S1 between MME and eNB, can be found in TS 36.413 section 9.2.1.140:

| IE/Group Name | Semantics description |
| --- | --- |
| Periodic Communication Indicator | This IE indicates whether the UE communicates periodically or not, e.g. only on demand. |
| Periodic Time | This IE indicates the interval time of periodic communication, the unit is: second |
| Scheduled Communication Time | This IE indicates the time zone and day of the week when the UE is available for communication. |
| >>Day of Week | BIT STRING (SIZE(7)) |
| >>Time of Day Start | INTEGER (0..86399, …) |
| >>Time of Day End | INTEGER (0..86399, …) |
| Stationary Indication | ENUMERATED(stationary, mobile, …) |
| Traffic Profile | “single packet” indicates single packet transmission (UL or DL), “dual packets” indicates dual packet transmission (UL with subsequent DL, or DL with subsequent UL), “multiple packets” indicates multiple packets transmission. |
| Battery Indication | “battery powered” indicates that the UE is battery powered and the battery is rechargeable/replaceable, “battery powered not rechargeable or replaceable” indicates that the UE is battery powered but the battery is not rechargeable/replaceable,, “not battery powered” indicates that the UE is not battery powered. |

In addition to the CN subscription info RAN2 agreed that it would be beneficial to introduce a vendor specific UE differentiation information IE that includes a UE profile [3]. This UE profile (aka Local RRM Policy Information) signaling [4-6] was not implemented in RAN3, but the SID for the REL-16 study on SON/MDT [7] was revised to include a NOTE [8]:

Note: this includes studying the Local RRM Policy Information storage and retrieval per UE for LTE and NR.

The gNB can monitor the traffic of the UE and build a traffic profile e.g. through machine learning. This UE traffic profile could be stored transparently in the AMF, similar as the UE capabilities, and retrieved from the AMF when the UE enters connected mode again. Such a traffic profile can be tailored to the gNB specific information needs (e.g. type of information, complexity, granularity, etc), and can adapt to changes in the UE traffic behaviour. Such a UE profile can already be fetched after RRCResumeRequest message and the available information in the UE profile can be used to configure CA/DC appropriately in RRCSetup message. Any UE assistance information from the UE would come later, and require additional RRC reconfiguration signalling, including delay.

Proposal 1: RAN2 to study the benefit of UE differentiation to enable optimal SCell configuration in MSG4.

In our view it is not clear if it is beneficial if the UE sends information about the expected traffic in the near future, which would complement the BSR signalling, which indicates the instantaneous traffic demand to activate SCells. In our understanding such any such additional information would be based on information from higher layers. The UE may have information about which application(s) are running and what type of traffic to expect. From live networks it can be observed that the mobile UE on average generates 20 RRC connection setups/h/UE. The main connection duration is 10 sec, and the median 1 sec, i.e. there are a lot of short connections. It is assumed that there is a large number of short and low traffic volume sync connections triggered by applications when the screen goes on, and then other connections triggered by a user (e.g. video, messaging, WEB browsing, file sharing/download, voice, etc), which may be of a high(er) traffic volume. Short sync/message connections may not require CA/DC establishment, while others may. Furthermore streaming video/voice traffic has inherent different characteristics from (bursty) data connections such as WEB browsing and file sharing/download. However different applications may be running in parallel, and it needs to be evaluated in detail, because the pain is usually in the details, in what form UE assistance information can be provided, such that it is useful for the NW to use. One critical question in this discussion is whether the NW should activate SCell based on expected traffic by using UE assistance information, or whether this should be done on actual traffic observed in DL buffer, and reported via BSR for UL buffer. It should also be noted that CA/DC improvements are being discussed that speed up the signalling and activation, which can make the existing BSR signalling sufficient for SCell activation:

Proposal 2: RAN2 to carefully evaluate if there is a need for UE assistance information in addition to the BSR signalling to assist the activation of SCells.

Potentially UE assistance information can be used to configure SCell(s), but as indicated above, in our view, the use of UE differentiation information is better, as this information is available after MSG3 and can be configured in MSG4 already.

BSR is reported after it has been configured in RRCSetup, RRCResume or RRCReconfiguration message. It would be beneficial when BSR reporting is available early during the connection setup, such that the CA/DC configuration in RRCResume message can take into account the UL buffer status. The UE could use the stored UE AS Context to report BSR in RRCResumeRequest message:

Proposal 3: RAN2 to evaluate the benefit of reporting BSR in RRCResumeRequest message.

SCell measurements:

In LTE there is the dormant SCell state, where the UE is not required to monitor PDCCH but still is required to measure RRM and CSI [1]. The SCell can be configured as dormant during connection setup in MSG4, and then activated when traffic demand dictates. It should be noted that the power saving gain of the dormant state, compared to an activated SCell which does not carry any data, and thus is in cDRX, is expected not to be that high. SCells in CA, and SCells in the same master or secondary group for DC share the same DRX operations when activated. SCells should only be activated when there is traffic to carry. Thus SCells are typically deactivated at the same time, unless some SCells remain activated to allow a fast activation.

The main delay factor in the activation of SCells is the measurement delays and not the signalling delay. RRM measurements are used to determine which SCells to configure. CSI measurements are used to determined when the SCell quality is good enough for activation. Power saving and fast SCell activation are contradictive, i.e. for fast SCell activation measurements ahead of time are needed, and it is possible that the UE performs measurements on SCells that are not activated, because there is no traffic demand.

It might be beneficial to configure SCell(s) via RRC signalling during connection resume already, and avoid RRC signalling to configure SCell(s) later. However the SCells are in deactivated state after RRC configuration, and the UE is required to perform RRM measurements when the cell is deactivated. When there is no traffic demand the configured SCell(s) may never be activated, nevertheless the UE has spent power on RRM measurements. Therefore it is proposed to evaluate a new SCell state in NR that allows the cell to be configured and deactivated, but the UE is not required to perform any RRM nor CSI measurements, i.e. this is a SCell state that favours power saving at the expense of the activation delay. In many cases, when RRC connection is established for higher layer applications to synchronize, no additional SCell need to be configured. But when SCells are deemed necessarily later, they can be activated without signalling overhead:

Proposal 4: RAN2 to evaluate the benefit of a new latent SCell state in which the SCell is only configured via RRC, but the UE is not required to perform any measurements.

When the gNB wants to activate one or more latent SCells than only MAC CE signalling is needed. When the UE receives a MAC CE activation, this triggers the UE to start RRM and CSI measurements according to the (latent) RRC configuration. Potentially the gNB can use measurement reporting from the PCell/SpCell to activate latent SCells on adjacent frequencies, or measurements reported from other UEs in proximity of the UE on other frequencies:

# Summary

RAN2 is kindly asked to discuss power saving enhancements for CA and DC:

Proposal 1: RAN2 to study the benefit of UE differentiation to enable optimal SCell configuration in MSG4.

Proposal 2: RAN2 to carefully evaluate if there is a need for UE assistance information in addition to the BSR signalling to assist the activation of SCells.

Proposal 3: RAN2 to evaluate the benefit of reporting BSR in RRCResumeRequest message.

Proposal 4: RAN2 to evaluate the benefit of a new latent SCell state in which the SCell is only configured via RRC, but the UE is not required to perform any measurements.

# References

- RP-182006, Signalling for euCA (Enhancing LTE CA Utilization), Nokia, CR 36.331, RAN#81

- RP-182076, WID on DC and CA enhancements, RAN2, WID, RAN#81

- R2-1803820, LS on UE differentiation in NB-IoT, LS out, To: RAN3, CC: CT1, SA3, RAN2#101

- RP-181150, Introduction of UE differentiation, Ericsson, Vodafone, Huawei, CR 36.300

- RP-181142, Introduction of UE differentiation, Huawei et all, CR 36.413, RAN#80

- RP-181145, Introduction of UE differentiation, Huawei at all, CR 36.423, RAN#80

- RP-182105, Study on RAN-centric Data Collection and Utilization for LTE and NR, SID, CMCC, RAN#81

- R3-19xxxx, Status Quo and way forward for UE RRM policy handling , Ericsson, DISC, RAN3 #103