3GPP TSG-RAN WG2 Meeting #105	R2-1901285

Athens, Greece, 25 Feb - 1 Mar 2019

Agenda item:	11.8.2

Source: 	Huawei, HiSilicon

Title: 	Considerations on broadcasting of location assistance data

Document for:	Discussion and Decision

Introduction

In LTE, broadcasting of location assistance data is supported. In this contribution, we provide our consideration on broadcasting of location assistance data in NR.

Discussion

In LTE, the following table shows the assistance data elements defined in 36.355[1] could be broadcasted:

|  | posSibType [12] | assistanceDataElement |
| --- | --- | --- |
| GNSS Common Assistance Data (clause 6.5.2.2) | posSibType1-1 | GNSS-ReferenceTime |
|  | posSibType1-2 | GNSS-ReferenceLocation |
|  | posSibType1-3 | GNSS-IonosphericModel |
|  | posSibType1-4 | GNSS-EarthOrientationParameters |
|  | posSibType1-5 | GNSS-RTK-ReferenceStationInfo |
|  | posSibType1-6 | GNSS-RTK-CommonObservationInfo |
|  | posSibType1-7 | GNSS-RTK-AuxiliaryStationData |
| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-1 | GNSS-TimeModelList |
|  | posSibType2-2 | GNSS-DifferentialCorrections |
|  | posSibType2-3 | GNSS-NavigationModel |
|  | posSibType2-4 | GNSS-RealTimeIntegrity |
|  | posSibType2-5 | GNSS-DataBitAssistance |
|  | posSibType2-6 | GNSS-AcquisitionAssistance |
|  | posSibType2-7 | GNSS-Almanac |
|  | posSibType2-8 | GNSS-UTC-Model |
|  | posSibType2-9 | GNSS-AuxiliaryInformation |
|  | posSibType2-10 | BDS-DifferentialCorrections |
|  | posSibType2-11 | BDS-GridModelParameter |
|  | posSibType2-12 | GNSS-RTK-Observations |
|  | posSibType2-13 | GLO-RTK-BiasInformation |
|  | posSibType2-14 | GNSS-RTK-MAC-CorrectionDifferences |
|  | posSibType2-15 | GNSS-RTK-Residuals |
|  | posSibType2-16 | GNSS-RTK-FKP-Gradients |
|  | posSibType2-17 | GNSS-SSR-OrbitCorrections |
|  | posSibType2-18 | GNSS-SSR-ClockCorrections |
|  | posSibType2-19 | GNSS-SSR-CodeBias |
| OTDOA Assistance Data (clause 7.4.2) | posSibType3-1 | OTDOA-UE-Assisted |

As shown this table, the broadcasting assistance data are mainly for A-GNSS, RTK and OTDOA. The purpose of broadcasting location assistance data is to reduce network signalling and reduce load on network elements (e.g., LMF, AMF, gNB) for massive recipient UEs in each cell, and to reduce latency and power consummation during obtaining assistance data at a UE. However, the gain is not obvious when the number of UEs performing positioning is few.

Moreover, if NR supports to broadcast assistance data, there will be some works to study how to broadcast them to ensure the alignment with the current on-demand SI broadcasting mechanism. However, considering that the time allocated to positioning in RAN2 is limited and there are more urgent problems to discuss (e.g., OTDOA, UTDOA, DL-AOD, UL-AOA, multiple RTT, etc), broadcasting of assistance data could be discussed in later releases.

Proposal: Considering the limited time allocated to positioning in RAN2 and more urgent problems to discuss (e.g., OTDOA, UTDOA, DL-AOD, UL-AOA, multiple RTT, etc), broadcasting of assistance data could be discussed in later releases.

Conclusion

In this contribution, we provide our consideration on broadcasting of location assistance data, and we get the following observation and proposals:

Proposal: Considering the limited time allocated to positioning in RAN2 and more urgent problems to discuss (e.g., OTDOA, UTDOA, DL-AOD, UL-AOA, multiple RTT, etc), broadcasting of assistance data could be discussed in later releases.

References

[1] 3GPP TS 36.355, "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[2]