3GPP TSG-RAN WG2 #105	Tdoc R2-1901306

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	    10.4.1.3.4

Source: 	    Ericsson

Title:		    Email discussion report [104#31][NR] Reconfig with sync terminology

Document for:	    Discussion and decision

# 1 Introduction

This is the report of the following email discussion:

[104#31][NR] Reconfig with sync terminology (Ericsson)

Review the handover and reconfig with sync terminology to identify if any ambiguity or problem is caused by the mix of terminology and prepare a CR to address it.

Intended outcome: CR submitted to next meeting

Deadline:  Thursday 2019-02-07

In RAN2#104, there was only a short online discussion based on the following CR [1]:

R2-1817531	Handover terminology clean-up	Ericsson	CR	Rel-15	38.331	15.3.0	0663	-	F	NR_newRAT-Core

The reason for the CR is that in some cases it is unclear what is referred to with “handover” in 38.331. In descriptive text, terms like inter frequency handover, inter system handover, inter RAT handover are used and in these cases it is clear what is referred to. Also in inter node messaging, handover preparation and command are used, and it is clear that it is for inter node handover, involving change of security key.

However, in normative field descriptions and field conditions, it needs to be fully clear what are the usage and conditions of the fields, and here the use of handover can be ambiguous, as it may in some cases imply a serving cell change with security key change, which was the meaning in LTE, but is not always the case in NR, where serving cell change can be performed also without security key change.

The term “N2 handover” is used to denote the handover case where a new Kamf is received from the CN. Though the term “N2 handover” is used in the referenced 33.501, it is not a term used in RAN specifications. Thus, we prefer to replace it with “NG handover”, since NG interface is defined in 38.401.

In this email discussion, all the changes proposed in [1] are listed to give companies possibility to provide their input.

# 2 Discussion

In section 5.3.5, there is a general description of the RRC reconfiguration procedure. The RRC reconfiguration procedure is used e.g. to execute handover. During the study item phase, it was agreed that handover in NR can be performed with or without security key change. When EN-DC was introduced, three types of RRC reconfigurations were listed in 36.331, RRC reconfiguration with sync including security key change, RRC reconfiguration with sync not including security key change and RRC reconfiguration not including sync nor security key change. To clarify terminology or NR standalone operation, it would be good to add the corresponding lines also in 38.331, see below:

### 5.3.5	RRC reconfiguration

#### 5.3.5.1	General

Figure 5.3.5.1-1: RRC reconfiguration, successful

Figure 5.3.5.1-2: RRC reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/modify/release RBs, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups. As part of the procedure, NAS dedicated information may be transferred from the Network to the UE.

The following cases of RRC reconfiguration with sync are defined:

-    RRC Reconfiguration with sync and security key refresh, i.e. the procedure involves RA to the PCell/PSCell, including MAC reset, re-establishment of RLC and PDCP triggered by explicit L2 indicators and refresh of security; and

-    RRC Reconfiguration with sync but without security key refresh, i.e. the procedure involves RA to the PCell/PSCell, including MAC reset and RLC re-establishment and PDCP data recovery (for AM DRB) triggered by explicit L2 indicators.

-

In EN-DC, SRB3 can be used for measurement configuration and reporting, to (re-)configure MAC, RLC, physical layer and RLF timers and constants of the SCG configuration, and to reconfigure PDCP for DRBs associated with the S-KgNB or SRB3, provided that the (re-)configuration does not require any MeNB involvement.

Question 1: Do companies agree to clarify the different types of RRC reconfiguration in 5.3.5?

| Company | Yes/No? | Comments |
| --- | --- | --- |
| ZTE | No | We don’t see the strong motivation to define these types of RRC reconfiguration. In addition, considering the on-going R16 WI on mobility enhancement, some enhancements on the table may lead to considerable complexity in the definition of RRC reconfiguration type (i.e. If we want to cover all the cases, then lots of RRC reconfiguration type need to be defined in the future). For example: - RRC Reconfiguration with RACH and security key refresh but without the MAC reset/RLC re-establishment/PDCP re-establishment: e.g. eMBB based solution - RRC Reconfiguration with the security key refresh but without involving RACH: e.g. role change procedure of the DC based solution. There’s no need to perform MAC reset/RLC re-establishment/PDCP re-establishment either in this case. For the given definitions of RRC reconfiguration type, some issues have been identified as below: - In case of full configuration, RLC/PDCP should be released then added. So it should be either listed as a new type or the different behavior should be added in one of the listed types. - The listed three types imply that the RLC re-establishment can only be performed in case of reconfiguration with sync. However, it is specified in ASN.1 that the RLC re-establishment can be set to true even for the case of RRC reconfiguration without sync: For SRB2 and DRBs, it is also set to TRUE during the resumption of the RRC connection or the first reconfiguration after reestablishment. - It’s not clear how to distinguish/determine the different RRC reconfiguration types in MR-DC and NR-DC. For example, the relationship between PCell/PSCell and the security key refresh should be clarified (it is worth noting that the security info is included in the RRC message generated by MN, but not in the RRC message generated by SN). In summary, we prefer not to give definitions for different types of RRC reconfiguration. If majority companies think the definition is useful, then we think the issue identified above should be solved and a forward compatible definition is preferred to cover the potential enhancement in Rel-16. |
| Nokia, Nokia Shanghai Bell | No | We would first like to understand what would be the aim of differentiating the procedures, and what it would involve: We are not sure this is needed. 1) Presumably, the intent would be to have three different procedural texts, but this is not clear 2) There would likely be (possbily a lot) of repetition between the procedures, much like we have in LTE. This increases repetition in RRC, which may also lead to errors. |
| Sam | Yes | We think it would be good to clarify the terms for these main cases as they are already used e.g. to clarify presence of information within conditions, which we think is important. This becomes clear if we look at cases like addressed by question 5, 6 and 9. We think that in some conditions we should refer to procedure types while in other cases we need to refer to a mobility scenario e.g. change of PCell/ PSCell. Some further remarks: - A change of SPCell requires a reconfiguration with sync and possibly also with key refresh i.e. CU/ DU split), the other way around is not true i.e. network can apply a certain reconfiguration procedure even without any mobility - Both reconfiguration procedure and mobility scenario independently apply for MCG and SCG (and terminology should cover both consistently) - It would be good to define terms to use for the main mobility scenario’s i.e. change of PCell/ PSCell, SCG addition, change of SCG - We (still) prefer to have some short acronyms for the reconfiguration procedures (for use in condition names and in speak) and previously suggested to use (in R2-1812165) - Refresh for reconfiguration with synch and key refresh - Resync for reconfiguration with synch but without key refresh Some remarks about the actual TP - To address the ZTE comment we propose to add in the definitions of the first two reconfiguration types that RLC and PDCP actions is triggered by explicit L2 indicators. We could possibly also refer to fields defining which type is used e.g. mobilityControlInfo - For the 1st and 2nd bullet we should clarify re-establishment concerns all L2 entities associated with the concerned CG (i.e. RLC entities of RLC bearers of the concerned CG, PDCP entities of RBs using the key associated with the concerned CG) |
| Huawei, HiSilicon | No. | It is pre-mature to classify the RRC reconfiguration into such three types. In current text of procedures and field descriptions involving the RRC reconfiguration, more specific use case and conditions can be specified such as whether it is used for PCell handover/change, PSCell addition, Scell addition, etc. The proposed text can increase the risk not matching exactly with the existing procedure and field description and cause confusion. In addition the above proposed three type classifications potentially add restriction to the technical scope of the RRC Reconfiguration capability, we suggest pending this proposed change for further discussion and study. |
| Ericsson | Yes, with update | First some clarifications on the purpose: - The purpose of the list would be to clarify and define the cases of “Reconfiguration with sync”, which is used in several places of the spec. Primarily that in NR, reconf with sync can be performed with or without security refresh. This is a change from LTE, thus good to point out. - The purpose is not to include different procedural text for the different types of RRC reconfigurations, which would indeed cause duplication as pointed out by Nokia. - The purpose is not to limit the cases of different RRC reconfiguration. There are indeed current and possibly future cases that are not covered in the list, as pointed out by ZTE and Huawei. We think like Samsung that it would be good to add a definition for the different Reconfiguration with sync cases. In order to address some of the concerns raised, we have made some updates to the proposed text above that can be further discussed. For the time being. We left out the linking of MCG and SCG key refresh and the re-establishment of RLC and PDCP for these cases. This can be added once the procedural text is in place. |
| NTT DOCOMO | Yes | We agree on the background of this proposal. Compared to 36.331, 38.331 looks not so clear what type of reconfiguration is used for handover, PSCell change, etc. although such the term is used in the other NR specs as well. |
| Turkcell | Yes | We agree on the clarification for the different types of RRC reconfiguration. |
| MediaTek | Yes | We agreed with Samsung that it would be good to clarify the “reconfiguration with sync” procedure. The updated TP looks fine to us. |
| Qualcomm | No | There is already a section for “RRC reconfiguration with sync” and it covers both w/ and w/o key change. Adding the proposed text does not bring additional clarity and possibly more confusion going forward with further enhancements to the procedures. |
| LG | Yes | As the “reconfiguration with sync” is used as a single word, we propose to specify it as “ReconfigurationWithSync” similar to what we already have in field description. Then, we can categorize it into two, i.e. “ReconfigurationWithSync with security refresh” and “ReconfigurationWithSync without security refresh”. |

Rapporteur summary: There were concerns with the initially proposed text, e.g. that it would pose limitation on the different types of RRC reconfigurations. Rapporteur clarified that this was not the purpose and provided an updated version only listing two cases of reconfiguration with sync. Out of the five companies responding after the update, four companies agree with the proposed text. One company mentioned that there is already a section on RRC reconfiguration with sync. We assume they refer to 5.3.5.5.2, but this section does not mention the relation to the security key being updated or not. Thus, with the majority of companies supporting the proposed text, we would propose the following:

Section 5.3.5.1 is updated according to the CR to include a description of the cases of RRC Reconfiguration with sync, with and without security key refresh.

In section 5.7.2.4, the term “handover” should be clarified. It is listed together with RRC connection re-establishment, which is an RRC procedure, but there is no RRC procedure called “handover”. Therefore, the suggestion is to replace “handover” with “RRC reconfiguration with sync”, and to include a proper definition for RRC reconfiguration with sync in 38.331. The risk with using just handover is that using LTE terminology, handover included always security key change, but intra NR handover can occur also without security key change.

#### 5.7.2.4	Failure to deliver ULInformationTransfer message

The UE shall:

1>	if AS security is not started and radio link failure occurs before the successful delivery of ULInformationTransfer messages has been confirmed by lower layers; or

1>	if PDCP re-establishment or release/addition for the corresponding SRB (e.g due to key refresh upon cell change or RRC connection re-establishment) occurs before the successful delivery of ULInformationTransfer messages has been confirmed by lower layers:

2>	inform upper layers about the possible failure to deliver the information contained in the concerned ULInformationTransfer messages.

Question 2: Do companies agree to replace “handover” above with “RRC reconfiguration with sync”?

| Company | Yes/No? | Comments |
| --- | --- | --- |
| ZTE | No | In our understanding, the intention of the sentence is mainly for the case that re-establishment or release/addition has been processed on SRB for ULInformationTransfer transmission, in which case the buffered RLC PDU will be discarded. So another way can be that we capture such description directly instead of the procedure level description. For instance: - if PDCP re-establishment or release/addition for the corresponding SRB occurs before the successful delivery of ULInformationTransfer messages has been confirmed by lower layers |
| Nokia, Nokia Shanghai Bell | No | In the above case, just saying “serving cell change” could be sufficient, i.e. “1> If serving cell change occurs before successful delivery of ULInformationTransfer messages has been confirmed by lower layers;” Also, we could simply define what “handover” means, given that “RRC reconfiguration with sync” is a mouthful that can also create ambiguities (e.g. especially when further we add “with field XX” to the sentence). Alternatively, we could simply use shorthand “RRC resync” to indicate “RRC reconfiguration with sync” if we want to use the term in many places. |
| Samsung | No | We think a change is required and agree with ZTE that it seems better to clarify what causes the message loss. I.e. ‘if mobility’ should not be kept. If desired, we could still have an e.g. to clarify some scenario’s in which this occurs (as upon key change/ refresh e.g. upon change of PCell, connection re-establishment). We think that (for NR) SRB release only applies for SRB3, although we also have release of an RLC bearer for SRB1 and SRB2. Anyhow, we think that neither of these cases results in loss of the concerned message |
| Huawei, HiSilicon | No. | The first requirement already covered the general link failure case. The second requirement intend to cover the failure case in mobility involved procedures. However the term mobility in the requirement is a very general/loose term as is used in TS38.300. Clear requirement with specific procedure terms should be used here. The term “handover” is not well defined. It is also not clear whether RRC reconfiguration with sync covered all the cases. |
| Ericsson | Yes, with update | We agree it is better to be explicit in this case, so we updated the proposed text according to input from ZTE and Samsung. |
| NTT DOCOMO | Yes | The updated TP looks better and so fine with us. |
| Turkcell | Yes | We think that the updated proposed text is suitable. |
| MediaTek | Yes | We agree the updated TP. |
| Qualcomm | No | Agree that the current text is not sufficiently clear due to the generic meaning of mobility. However, it is better to state the actual cause, for example as in ZTE proposal. |
| LG | Yes | We propose to replace “cell change” with “PCell change or PSCell change”. |

Rapporteur summary: Also on this question there were initial concerns about the proposed changes not being clear enough. The proposed text was therefore update based on input and after that seems agreeable by four out of five companies. Thus, we hope we ca agree on following proposal:

Section 5.7.2.4 is updated according to the CR to clarify the cases of failed ULInformationTransfer message.

In section 6.2.2, the field description for keySetChangeIndicator in RRCReconfiguration includes the word “handover”. The current text is misleading, since the keySetChangeIndicator can be set also for inter cell handover. As key change requires always RRCReconfiguration with sync, regardless whether inter or intra cell, it would be better to use this terminology here.

Furthermore, the term N2 handover is used. Though the term “N2 handover” is used in the referenced 33.501, it is not a term used in RAN specifications. Thus, the suggestion is to replace it with “NG handover”, since NG interface is defined in 38.401.

In summary, the following changes are proposed for the field description:

keySetChangeIndicator Indicates whether UE shall derive a new KgNB. If reconfigurationWithSync is included, value true indicates that a KgNB key is derived from a KAMF key taken into use through the latest successful NAS SMC procedure, or NG handover procedure with KAMF change, as described in TS 33.501 [11] for KgNB re-keying. False indicates that the new KgNB key is obtained from the current KgNB key or from the NH as described in TS 33.501 [11].

Question 3: Do companies agree with the changes proposed to keySetChangeIndicator field description?

| Company | Yes/No? | Comments |
| --- | --- | --- |
| ZTE | Yes | Okay with the change. |
| Nokia, Nokia Shanghai Bell | Partly | This aligns with usual conventions of referring to the field names in when having conditions on the presence. However, the current text could perhaps be made clearer as to what the field does e.g. “Indicates whether UE shall derive a new KgNB. If ...” |
| Samsung | Yes, but | We think we should actually talk about reconfiguration with sync and key refresh |
| Huawei, HiSilicon | NO, partly | There is no need to impose new restriction/requirement that the keySetChangeIndicator should be tied with reconfigurationWithSync. It is the network to decide when to include reconfigurationWithSync. We think that the restriction “If reconfigurationWithSync is included” should not be added. Other changes are fine. |
| Ericsson | Yes, with update | We have added also a clarification as proposed by Nokia. |
| NTT DOCOMO | Yes | The updated TP looks better and so fine with us. |
| Turkcell | Yes | We think that the updated proposed text is suitable. |
| MediaTek | Yes | The updated TP looks fine to us |
| Qualcomm | Yes | Since key change can only happen via reconfigurationWithSync, this is a reasonable change. |
| LG | Yes |  |

Rapporteur summary: 9 out of 10 companies support the updated change. One company had concerns that the added text would add a new requirement that keySetChangeIndicator can only be used together with reconfigurationWithSync. However, the old text used the term “intra cell handover”, which also requires reconfigurationWithSync, so it is in that respect no change. In summary, the following is proposed:

In section 6.2.2 the field description for keySetChangeIndicator is updated according to the CR to clarify the usage.

In section 6.3.2, the field description for smtc in ReconfigurationWithSync of CellGroupConfig includes “NR PSCell change and intra-NR handover”. In this case, to avoid confusion whether security key is changed or not, it would be more correct to replace with “NR PSCell change and NR PCell change” to avoid the handover term:

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and NR PCell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of NR PCell change, the smtc is based on the timing reference of source PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

Question 4: Do companies agree with the changes proposed to smtc field description?

| Company | Yes/No? | Comments |
| --- | --- | --- |
| ZTE | Yes | Okay with the change. |
| Nokia, Nokia Shanghai Bell | No | We are now replacing “cell change” with “handover”. Again, if we define “handover” somewhere, all of these changes may become unnecessary. Or then we could simply just use “RRC resync” as a shorthand to refer to “RRC reconfiguration with sync”. |
| Samsung | Yes |  |
| Huawei, HiSilicon | Not now | We understand the motivation. However, we believe the problem roots from that the terms “handover” and “XXCell change” are not well defined. Without the terms being well defined in the spec, these terms are open for interpretation and changing terms doesn’t help much. The term “handover” is well known but the concept may not be precise enough to be used in specific stage 3 requirement. “XXCell change” is even loose which can be interpreted to any change. So what do “handover’ and “XXCell change” mean in our mind? For example, Does the handover mean: a UE changes its connection from one PCell to another PCell involving protocol stack reset, access, security key change? Does PCell Change mean: a UE changes its connection from one PCell to another PCell involving protocol stack reset, access, with or without security key change? Does it include PCell to PSCell change? What does PSCell change mean? And so on… Consider first to define the terms in the spec., then use the right term at different places for clarification. |
| Ericsson | Yes | We think here it is better to use PCell/PSCell change instead of RRC reconfiguration with sync, as here the mobility is key, and RRC reconfiguration with sync can occur without mobility, i.e. without change of PCell/PSCell. We see no greater need to define PCell change or PSCell change, these should be clear that it means a change of the PCell/PSCell, regardless whether security key is refreshed or not. |
| NTT DOCOMO | Yes | The proposed change is fine with us. |
| Turkcell | Yes | The changes in smtc field description is fine with us. |
| MediaTek | Yes |  |
| Qualcomm | Yes | As “handover” is not clearly defined, this option is better. However, it would also be good to define PCell change, for example at the first point of usage. Also, unlike LTE, we do not have a section for handover anymore and thus it is preferable to use the “reconfiguration w/ sync” consistently instead. |
| LG | Yes |  |

Rapporteur summary: 8 out of 10 companies support the updated change. Therefore, we make the following proposal:

In section 6.3.2 the field description for smtc is updated according to the CR to use the term “NR PCell change” instead of “Intra NR handover”.

In section 6.3.2, the field description for firstActiveDownlinkBWP-Id in ServingCellConfig includes “PCell handover”. In this case, “PCell change” would be preferable, since security key change is not always required here:

firstActiveDownlinkBWP-Id If configured for an SpCell, this field contains the ID of the DL BWP to be activated upon performing the RRC (re-)configuration. If the field is absent, the RRC (re-)configuration does not impose a BWP switch. If configured for an SCell, this field contains the ID of the downlink bandwidth part to be used upon MAC-activation of an SCell. The initial bandwidth part is referred to by BWP-Id = 0. Upon reconfigurationWithSync (PCell change, PSCell addition/change), the network sets the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id to the same value.

Question 5: Do companies agree with the changes proposed to firstActiveDownlinkBWP-Id field description?

| Company | Yes/No? | Comments |
| --- | --- | --- |
| ZTE | Yes | Okay with the change. |
| Nokia, Nokia Shanghai Bell | No | See question 4 |
| Samsung | Yes, but | We think the reference to reconfigurationWithSync should be removed. I.e. we think that upon reconfig with sync firstActiveDownlinkBWP-Id should be optional i.e. it is mandatory only in case of mobility upon PCell change and PSCell addition/change |
| Huawei, HiSilicon | Not now | Same comments as for question 4. |
| Ericsson | Yes | Regarding the comment from Samsung, there presence of the IE is defined in the existing condition, copied below: SyncAndCellAdd This field is mandatory present, Need N, for a SpCell upon reconfigurationWithSync (PCell handover, PSCelladdition/change) and upon RRCsetup/RRCResume. The field is mandatory present, Need M, for an SCell upon addition. For SpCell, the field is optionally present, Need N, upon reconfiguration without reconfigurationWithSync. In all other cases the field is absent. |
| NTT DOCOMO | Yes |  |
| Turkcell | Yes | firstActiveDownlinkBWP-Id field description change is suitable. |
| MediaTek | Yes |  |
| Qualcomm | Yes | Same as Q4 response |
| LG | Yes |  |

Rapporteur summary: 8 out of 10 companies support the change. Therefore, we make the following proposal:

In section 6.3.2 the field description for firstActiveDownlinkBWP-Id is updated according to the CR to use the term “PCell change” instead of “PCell handover”.

In section 6.3.2, the field condition SyncAndCellAdd for ServingCellConfig also includes “PCell handover”. Also here, this should be changed to “PCell change”, since security key change is not always required:

| SyncAndCellAdd | This field is mandatory present, Need N, for a SpCell upon reconfigurationWithSync (PCell change, PSCelladdition/change) and upon RRCsetup/RRCResume. The field is mandatory present, Need M, for an SCell upon addition. For SpCell, the field is optionally present, Need N, upon reconfiguration without reconfigurationWithSync. In all other cases the field is absent. |
| --- | --- |

Question 6: Do companies agree with the changes proposed to SyncAndCellAdd field condition?

| Company | Yes/No? | Comments |
| --- | --- | --- |
| ZTE | Yes | Okay with the change. |
| Nokia, Nokia Shanghai Bell | No | See question 4 |
| Samsung | Yes, but | See our response to question 5 |
| Huawei, HiSilicon | Not now. | Same comments as for question 4. |
| Ericsson | Yes |  |
| NTT DOCOMO | Yes |  |
| Turkcell | Yes |  |
| MediaTek | Yes |  |
| Qualcomm | Yes | Same as Q4 response |
| LG | Yes |  |

Rapporteur summary: 8 out of 10 companies support the change. Therefore, we make the following proposal:

In section 6.3.2 the field condition for SyncANdCellAdd is updated according to the CR to use the term “PCell change” instead of “PCell handover”.

In section 6.3.2, the field condition HOAndServCellAdd in ServingCellConfigCommon includes “for inter-cell handover”. The use of the word “handover” could be misleading, since field is mandatory for any kind of reconfigurationWithSync including PCell change, and does not require security key change:

| HOAndServCellAdd | This field is mandatory present upon PCell change and upon serving cell (PSCell/SCell) addition. Otherwise, the field is absent, Need M. |
| --- | --- |

Question 7: Do companies agree with the changes proposed to HOAndServCellAdd field condition?

| Company | Yes/No? | Comments |
| --- | --- | --- |
| ZTE | Partially yes | We can simply say “upon PCell change”. In other words, there’s no need to emphasize reconfigurationWithSync. |
| Nokia, Nokia Shanghai Bell | No | See question 4 |
| Samsung | Yes, but | We in general agree the comment from ZTE i.e. when the condition applies for a mobility scenario we should not refer to a reconfiguration procedure We furthermore think that this IE needs more general checking - According to current conditions downlinkConfigCommon can only be provided upon actual mobility and is mandatory in such case i.e. no delta signalling while for some other subfields e.g. uplinkConfigCommon, we just have need M. This seems incorrect - I.e. whether we can change common fields (SI) without mobility should apply consistently for all fields (we note that we probably need multiple conditions as some fields are truly optional) Note that our general understanding is as follows: - For SCells common config/ SI can only be changed by release & addition of the cell - For PCell & PSCell common config/ SI can be changed by reconfiguration with sync - Upon reconfiguration with sync for SPCell, delta signalling applies for common config/ SI. This also applies in case of change of SPCell Bullets a) and b) seem very clear from the specification, but the situation seems less clear for bullet c). We note that we explictly agreed to have delta signalling upon HO (change of PCell) for the common RACH configuration, |
| Huawei, HiSilicon | Not now | Same comments as for question 4. Again, it is depending on the definition of “PCell change”. Does it include Scell addition? Furthermore, the use cases of the ServingCellConfigCommon and the three conditional fields should be reviewed to see whether the condition specification is precise. |
| Ericsson | Yes, with update |  |
| NTT DOCOMO | Yes | The updated TP looks better and so fine with us. |
| Turkcell | Yes | The updated proposal is better and suitable. |
| MediaTek | Yes | The updated TP looks fine. |
| Qualcomm | Partly | Prefer ZTE’s suggestion (assuming we will define PCell change as suggested in Q4). |
| LG | Yes |  |

Rapporteur summary: 8 out of 10 companies support the updated change. Therefore, we make the following proposal:

In section 6.3.2 the field condition for HOAndServCellAdd is updated according to the CR to use the term “PCell change” instead of “inter cell handover”.

In section 5.3.14.1, the term “handover” should be clarified, since change of PCell can be performed without security key change:

### 5.3.14	Unified Access Control

#### 5.3.14.1	General

The purpose of this procedure is to perform access barring check for an access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers according to TS 24.501 [23] or the RRC layer.

After a change of PCell in RRC_CONNECTED the UE shall defer access barring checks until it has obtained valid UAC information (from SIB1) from the target cell.

Question 8: Do companies agree to replace “handover” above with “RRC reconfiguration with sync”?

| Company | Yes/No? | Comments |
| --- | --- | --- |
| ZTE | Partially yes | We can simply say “After a change of PCell”. In other words, there’s no need to emphasize RRC reconfiguration with sync. |
| Nokia, Nokia Shanghai Bell | No | See question 4 |
| Samsung | Yes, but | Agree with ZTE |
| Huawei, HiSilicon | Yes, but | We don’t see the need to keep “handover” and “change of PCell”. We could simply have “In RRC_CONNECTED the UE shall defer access barring checks until it has obtained valid UAC information (from SIB1) from the target cell” since the condition of deferring is clearly specified already. |
| Ericsson | Yes, with update | Agree with comments from ZTE and Samsung. |
| NTT DOCOMO | Yes | The updated TP looks better and so fine with us. |
| Turkcell | Yes | The updated proposal is better and suitable. |
| MediaTek | Yes | We agree the updated TP |
| Qualcomm | Partly | Agree with ZTE. In response to HW, we need to have a condition for which the “target cell” is applicable. |
| LG | Yes | To be aligned with other places, it would be better to change “change of PCell” to “PCell change”. |

Rapporteur summary: 8 out of 10 companies support the updated change. Therefore, we make the following proposal:

In section 5.3.14.1 the second paragraph is updated according the CR to remove the words “handover resulting in”.

In section 6.2.2, the field condition MasterKeyChange in RRCReconfiguration includes “Handover with change of the security algorithms”. The second sentence includes “If ReconfigurationWithSync is included for other cases”. We would propose to clarify this into:

| MasterKeyChange | This field is mandatory present in case masterCellGroup includes ReconfigurationWithSync and RadioBearerConfig includes SecurityConfig with SecurityAlgorithmConfig, indicating a change of the security algorithms. If ReconfigurationWithSync is included for other cases, this field is optionally present, need N. Otherwise the field is absent. |
| --- | --- |

Question 9: Do companies agree to update the field condition description as indicated above for MasterKeyChange?

| Company | Yes/No? | Comments |
| --- | --- | --- |
| ZTE | Yes | Okay with the change. |
| Nokia, Nokia Shanghai Bell | No | See question 4 |
| Samsung | Yes, but | We think we should talk about Reconfiguration with sync and key refresh. Note that we understand that security config is mandatory upon change of termination but may indicate same algorithm (i.e. not having delta signalling there results in more complicated formulation for MasterKeyChange) : |
| Huawei, HiSilicon | No. | Again, the new change of adding ReconfigurationWithSync to replace “Handover” imposed un-necessary restriction. Actually, the issue was introduced by CR#0243r4 which added the phrase “of Handover with change of” into the original field description which ties MasterKeyChange with Handover. The text before the CR#023r4 changes allows more flexibility and is preferable. Consider to simply remove the term “Handover” in 38.331f40 field description, for example “This field is mandatory present in case of Handover with change of the security algorithms (as indicated in SecurityAlgorithmConfig in SecurityConfig, included in the received RadioBearerConfig). If ReconfigurationWithSync is included for other cases, this field is optionally present, need N. Otherwise the field is absent.” |
| Ericsson | Yes | Regarding the comment from Samsung, in this case, it is not correct to talk about “RRC reconfiguration with sync and key refresh, since this is the field condition for key refresh. So, we need to indicate when key refresh is mandatory, and that is when there is a change of the security algorithm. Thus, the current text is correct.Regarding the comment from Huawei, we don’t see additional restriction from having the “ReconfigurationWithSync”, since it is needed anyway for change of security algorithm. |
| NTT DOCOMO | Yes | The proposed change is fine with us. |
| Turkcell | Yes | The MasterKeyChange change is suitable. |
| MediaTek | Yes | The proposed change looks fine to us |
| Qualcomm | Yes |  |
| LG | Yes |  |

Rapporteur summary: 8 out of 10 companies support the updated change. Therefore, we make the following proposal:

In section 6.2.2 the field condition for MasterKeyChange is updated according the CR to clarify the usage.

Companies are invited to suggest further occurrences of “handover” in 38.331 that should be clarified in the table below.

| Company | Section | Comments |
| --- | --- | --- |
| LG | 38.321 5.13, 5.19 | It would be also good to remove “handover” from MAC specification. |

Rapporteur summary: There was no clear proposal here, but rapporteur agrees it would be good to review also 38.321 with regards to usage of the term “handover”. Possible changes to 38.321 can be discussed based on company contributions.

Summarizing the discussion, we have updated the CR [2] based on the proposals in this email discussion. Thus, the final proposal is to agree the CR.

RAN2 to agree the CR presented in [2].

# 3 References

R2-1817531, “Handover terminology clean-up”,	Ericsson,	CR0663, Rel-15, 38.331, 15.3.0.

R2-1901307, “Reconfig with sync terminology”, Ericsson, CR0906, Rel-15, 38.331, 15.4.0.

# 4 Conclusions

Rapporteur would like to thank all companies participating in the email discussion. Thanks to many good comments we have increased the quality of the draft CR. Based on the discussion, the following proposals are made:

Proposal 1	Section 5.3.5.1 is updated according to the CR to include a description of the cases of RRC Reconfiguration with sync, with and without security key refresh.

Proposal 2	Section 5.7.2.4 is updated according to the CR to clarify the cases of failed ULInformationTransfer message.

Proposal 3	In section 6.2.2 the field description for keySetChangeIndicator is updated according to the CR to clarify the usage.

Proposal 4	In section 6.3.2 the field description for smtc is updated according to the CR to use the term “NR PCell change” instead of “Intra NR handover”.

Proposal 5	In section 6.3.2 the field description for firstActiveDownlinkBWP-Id is updated according to the CR to use the term “PCell change” instead of “PCell handover”.

Proposal 6	In section 6.3.2 the field condition for SyncANdCellAdd is updated according to the CR to use the term “PCell change” instead of “PCell handover”.

Proposal 7	In section 6.3.2 the field condition for HOAndServCellAdd is updated according to the CR to use the term “PCell change” instead of “inter cell handover”.

Proposal 8	In section 5.3.14.1 the second paragraph is updated according the CR to remove the words “handover resulting in”.

Proposal 9	In section 6.2.2 the field condition for MasterKeyChange is updated according the CR to clarify the usage.

Proposal 10	RAN2 to agree the CR presented in [2].