3GPP TSG-RAN WG2 Meeting #105                                                 R2-1901331

Athens, Greece, 25th Feburary – 1st March, 2019

Agenda Item:	11.12.4

Source:	MediaTek Inc.

Title:	New use case: UE energy saving

Document for:     Discussion and Decision

# 1	Introduction

For the study of RAN-centric Data Collection and Utilization for LTE and NR, this paper proposes to RAN to collect data to facilitate UE energy efficiency monitoring, which is a key performance indicator in [3].

The objective of this use case is to measure UE energy efficiency through RAN data collection, therefore, the effect of power saving features can be objectively reflected, which in the end leads to network and UE to work together to maximize UE battery lifetime and enhance user experience.

# 2	Discussion

According to GTI white paper [1], high-end mobile devices with 5G modem are expected to feature advanced components e.g. display, processors, graphic engine, etc. Added up, these render power management to be very challenging. In other words, power efficiency is critical for every single component, in particular for the modem itself, regardless of the power source used (e.g. battery capacity and technology).

Battery lifetime is an important criterion directly affecting user experience – no matter how good the other performance aspects of a device may be, if its battery lifetime is poor, user experience will be greatly impacted. To ensure a smooth migration from 4G to 5G, the battery lifetime should thus be, under similar criteria, on par or better using 5G than 4G.

Therefore, network and UE vendors have a common interest in ensuring that the 5G NR ecosystem is able to maximize battery lifetime for all its users in all conditions. This means network and UE ought to cooperate to ensure lower baseline power and maintain better efficiency with all data rates, as shown below.

Figure 1: UE power variation with data rate

Observation 1:	In comparison with LTE, it is critical for NR modem to have lower baseline power and maintain better power efficiency per bps with all data rates.

3GPP has started an effort, i.e. power saving SI [2], to achieve this target. From the study, as shown on Figure 2, for different traffic types, roughly less than 36% of the total power consumption occurs in sleep state (~RRC_Idle mode). Said otherwise, reducing the power consumption in RRC_Connected mode is the primary target. As the UE behavior in RRC_Connected mode is controlled by the network, the joint role of the UE and the network to reduce power consumption in this state is clear.

Figure 2: Power consumption distribution for different traffic models

Observation 2:	Most of UE power is consumed in RRC_Connected mode and power saving requires joint efforts between network and UE.

For 5G/NR design, network energy efficiency and UE energy efficiency are listed as two key performance indicators in [3].

For network energy efficiency, both qualitative and quantitative KPIs are proposed. In case of quantitative KPI, bit per Joule is used see §7.19 in [3].

For UE energy efficiency, which means the capability of a UE to sustain much better mobile broadband data rate while minimizing the UE modem energy consumption both qualitative and quantitative KPIs are required. Although only a qualitative KPI is captured now, it is also specified that based on the study, more detailed quantitative assessment can be performed and a conclusion on this matter is expected from the on-going power saving study [2].

Observation 3:	UE energy efficiency is one of the 5G KPIs – a quantitative assessment is required.

On one hand, although it is always possible to measure power consumption of a UE , in the UE, it is not sufficient as it is a) vendor dependent, and b) not trackable by operators. On the other hand, as explained above, it is the very power consumption resulting from the usage of 5G NR technology that ought to assessed – as argued above, this is directly linked to how the network operates the UE i.e. what kind of configurations it uses with the UE. Such configurations following 3GPP specifications, their energy efficiency (and in turn UE energy efficiency) can therefore be fully assessed and tracked in a reliable fashion. As a result, it is our understanding that the definition of a reliable UE energy efficiency (UE EE) KPI consistent across UE vendors, and fully trackable in 5G networks by operators is feasible.

The definition of a UE EE KPI would further allow operators to assess the effect of a given feature (e.g. power saving feature) on the UE power consumption. .

Observation 4:	of the definition of a reliable UE EE KPI is deemed to be feasible based on 5G that will reliably reflect, irrespective of vendors, the power efficiency of 5G NR and its features in the UE.

The main difficulty of a standard UE EE KPI has been lying in the lack of a commonly agreed power consumption model by vendors. However, this hurdle has recently been removed with the latest progress of the RAN1 study on NR UE power saving – a UE power consumption model is agreed as shown in Table 1 below.

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) |

Table 1: UE power consumption model for FR1 [4]

Using the above model, one can estimate UE power consumption based on link configuration and the scheduling record.

Similar to the definition of network EE KPI, the UE EE KPI is proposed to be represented as bit per Joule. This means that in addition to power consumption, data volume information is also needed to represent the UE EE. Below is such a definition of a UE EE KPI:

UE_EE=Data Volumei=power stateEnergy Consumption (i)

- Data Volume [bit] represents the traffic volume for the UE for a given duration.

- Energy Consumption [Joule] represents the total energy consumption of the UE for a given duration, which is the summation of power consumption (i.e. product of Relative Power and time duration) in all Power State.

With such definition, UE EE measurement can be objectively carried out by any entity that has the link configuration, scheduling record, and throughput/data volume information, in a consistent fashion across UE vendors.

Observation 5:	With the power state model agreed in RAN1 study, a reliable UE EE KPI can be objectively measured with link configuration, scheduling record, and throughput/data volume information.

With such definition as a baseline, further aspects can be further discussed for UE EE measurement

- Network measurement and/or UE measurement

- Signaling procedure, e.g. L2 measurement, MDT, etc.

- QoS association

- Deployment scenario association, SA/NSA, DC, CA, etc.

Related discussion can take place in related work items.

# 3	Conclusions

Proposal:	Agree to add UE energy saving as a new use case and the corresponding text proposal.

# 5	Reference

- GTI Sub-6GHz 5G Device White Paper, Chapter 13 Power Consumption

- RP-181463, WID: Study on UE Power Saving in NR

- 3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, V14.3.0 (2017-06)

- Draft TR 38.840 – Study on UE Power Saving (CATT)

- R1-1901298, Adaptation Designs for NR UE Power Saving, MediaTek Inc.

- RP-182105, WID: Study on RAN-centric Data Collection and Utilization for LTE and NR

# 5	Text Proposal for TR 37.816

3.2.x Use case: UE energy saving [Proposed by MediaTek]

According to GTI white paper [1], high-end mobile devices with 5G modem are expected to feature advanced components e.g. display, processors, graphic engine, etc. Added up, these render power management to be very challenging. In other words, power efficiency is critical for every single component, in particular for the modem itself, regardless of the power source used (e.g. battery capacity and technology).

Battery lifetime is an important criterion directly affecting user experience – no matter how good the other performance aspects of a device may be, if its battery lifetime is poor, user experience will be greatly impacted. To ensure a smooth migration from 4G to 5G, the battery lifetime should thus be, under similar criteria, on par or better using 5G than 4G.

Therefore, network and UE vendors have a common interest in ensuring that the 5G NR ecosystem is able to maximize battery lifetime for all its users in all conditions. This means network and UE ought to cooperate to ensure lower baseline power and maintain better efficiency with all data rates, as shown below.

Figure 1: UE power variation with data rate

3GPP has started an effort, i.e. power saving SI [2], to achieve this target. From the study, as shown on Figure 2, for different traffic types, roughly less than 36% of the total power consumption occurs in sleep state (~RRC_Idle mode). Said otherwise, reducing the power consumption in RRC_Connected mode is the primary target. As the UE behavior in RRC_Connected mode is controlled by the network, the joint role of the UE and the network to reduce power consumption in this state is clear.

Figure 2: Power consumption distribution for different traffic models

For 5G/NR design, network energy efficiency and UE energy efficiency are listed as two key performance indicators in [3].

For network energy efficiency, both qualitative and quantitative KPIs are proposed. In case of quantitative KPI, bit per Joule is used see §7.19 in [3].

For UE energy efficiency, which means the capability of a UE to sustain much better mobile broadband data rate while minimizing the UE modem energy consumption both qualitative and quantitative KPIs are required. Although only a qualitative KPI is captured now, it is also specified that based on the study, more detailed quantitative assessment can be performed and a conclusion on this matter is expected from the on-going power saving study [2].

On one hand, although it is always possible to measure power consumption of a UE , in the UE, it is not sufficient as it is a) vendor dependent, and b) not trackable by operators. On the other hand, as explained above, it is the very power consumption resulting from the usage of 5G NR technology that ought to assessed – as argued above, this is directly linked to how the network operates the UE i.e. what kind of configurations it uses with the UE. Such configurations following 3GPP specifications, their energy efficiency (and in turn UE energy efficiency) can therefore be fully assessed and tracked in a reliable fashion. As a result, it is our understanding that the definition of a reliable UE energy efficiency (UE EE) KPI consistent across UE vendors, and fully trackable in 5G networks by operators is feasible.

The definition of a UE EE KPI would further allow operators to assess the effect of a given feature (e.g. power saving feature) on the UE power consumption. .

The main difficulty of a standard UE EE KPI has been lying in the lack of a commonly agreed power consumption model by vendors. However, this hurdle has recently been removed with the latest progress of the RAN1 study on NR UE power saving – a UE power consumption model is agreed as shown in Table 1 below.

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) |

Table 1: UE power consumption model for FR1 [4]

Using the above model, one can estimate UE power consumption based on link configuration and the scheduling record.

Similar to the definition of network EE KPI, the UE EE KPI is proposed to be represented as bit per Joule. This means that in addition to power consumption, data volume information is also needed to represent the UE EE. Below is such a definition of a UE EE KPI:

UE_EE=Data Volumei=power stateEnergy Consumption (i)

- Data Volume [bit] represents the traffic volume for the UE for a given duration.

- Energy Consumption [Joule] represents the total energy consumption of the UE for a given duration, which is the summation of power consumption (i.e. product of Relative Power and time duration) in all Power State.

With such definition, UE EE measurement can be objectively carried out by any entity that has the link configuration, scheduling record, and throughput/data volume information, in a consistent fashion across UE vendors.

The objective of this use case is to measure UE energy efficiency through RAN data collection, therefore, the effect of power saving features can be objectively reflected, which in the end leads to network and UE to work together to maximize UE battery lifetime and enhance user experience.