3GPP TSG-RAN WG2 #105      								R2-1901332

Athens, Greece, 25 Februray – 1 March, 2019

Agenda Item:	11.2.1.2

Souce:	Huawei, HiSilicon

Title:	LBT modeling for MAC

Document for:	Discussion and decision

1 Introduction

A WID on NR-based Access to Unlicensed Spectrum has been approved. This work item is aimed at supporting the following scenarios:

- Scenario A: Carrier aggregation between licensed band NR (PCell) and NR-U (SCell). - NR-U SCell may have both DL and UL, or DL-only. - In this scenario, NR PCell is connected to 5G-CN. - Scenario B: Dual connectivity between licensed band LTE (PCell) and NR-U (PSCell) - In this scenario, LTE PCell connected to EPC as higher priority than PCell connected to 5G-CN. - Scenario C: Stand-alone NR-U - In this scenario, NR-U is connected to 5G-CN. - Scenario D: A stand-alone NR cell in unlicensed band and UL in licensed band (single cell architecture). - In this scenario, NR-U is connected to 5G-CN. - Scenario E: Dual connectivity between licensed band NR and NR-U. - In this scenario, PCell is connected to 5G-CN.

In this contribution, we discuss the issue related to MAC LBT modelling for NR-U and provide our considerations.

2 Discussion

In TS 36.321, LBT modelling has been specified in MAC as following.

The physical layer may perform a listen-before-talk procedure, according to which transmissions are not performed if the channel is identified as being occupied or the physical layer may monitor for PUSCH trigger B [2], according to which transmissions are not performed if PUSCH trigger B is not received. In both cases a MAC entity considers the transmission to have been performed anyway, unless stated otherwise.

UE LBT may impact MAC procedure, e.g. start of timers, cancelling of BSR/PHR trigger as LTE LAA. The question is whether the above sentence can be applicable to NR-U. We believe the answer is yes or no depending on the cases. We are going to discuss this case by case.

2.1 Potential impact for timers and counters

2.1.1 Random access timers: ra-ResponseWindow and ra-ContentionResolutionTimer

For ra-ResponseWindow, in order to retransmit the preamble timely, it was agreed by RAN2 during SI stage that ra-ResponseWindow is not started and power ramping is not applied when the preamble is not transmitted due to LBT failure [1]. Since LBT is performed in PHY, some form of LBT outcome related indication from PHY to MAC is needed. Based on this indication, the MAC layer can determine not to start the ra-ResponseWindow and not to perform power ramping in the next preamble transmission.

TR 38.889 xxxx In legacy RACH, the counters for preamble transmission and power ramping are increased with every attempt. In NR-U, power ramping is not applied when preamble is not transmitted due to LBT failure. This will require an indication from the physical layer to the MAC. In addition, ra-ResponseWindow is not started when the preamble is not transmitted due to LBT failure.

Observation 1: ra-ResponseWindow is not started when the preamble is not transmitted due to LBT failure.

Observation 2: power ramping is not applied when the preamble is not transmitted due to LBT failure.

Proposal 1: LBT outcome indication from PHY to MAC is needed for the determination of starting ra-ResponseWindow and performing preamble power ramping.

Another issue is whether the UE starts the ra-ContentionResolutionTimer when the preamble transmission fails due to LBT. Actually in last meeting, there was some discussion on this issue but no consensus was achieved as some companies think starting this timer relates to the actual transmission of Msg3. But from our point of view, we think different from ra-ResponseWindow, which should not be started in case of LBT failure, this timer should be started even when Msg3 is not transmitted. This is because Msg3 is a transmission on the scheduled grant and even though the UE fails to transmit Msg3 due to unsuccessful channel access, the NW is able to detect this and a proper NW implementation is to schedule a retransmission. In this case the UE needs to start the ra-ContentionResolutionTimer to monitor the PDCCH for HARQ retransmission. If the UE does not start this timer when Msg3 is not transmitted due to LBT failure, then the UE may fall asleep and is not able to receive the scheduled retransmission. In addition, since there is no contention resolution timer running, UE is not able to start another RACH attempt upon the timer expiration. Therefore, the RACH procedure will get struck in this case.

Proposal 2: ra-ContentionResolutionTimer is started/restarted even when the Msg3/HARQ retransmission is not transmitted due to LBT failure.

2.1.2 Impact on random access counters

Currently in NR, there are two counters: PREAMBLE_TRANSMISSION_COUNTER and PREAMBLE_POWER_RAMPING_COUNTER.

- PREAMBLE_TRANSMISSION_COUNTER is used to count the number of PRACH transmissions during a random access procedure and to take appropriate actions when the count reaches the maximum value. It is incremented by 1 in case that RAR reception is not successful or contention resolution is not successful.

- PREAMBLE_POWER_RAMPING_COUNTER is used in calculating power for PRACH (re-) transmissions and is incremented by 1 if UE does not change the transmit beam and the selected SSB/CSI-RS is not changed.

In NR-U, preamble transmission may be blocked due to LBT failure. During the SI, companies reached consensus that the UE does not increase the preamble transmission power when a preamble is not transmitted due to LBT. But for PREAMBLE_TRANSMISSION_COUNTER, despite what RAN1 recommended, i.e., the preamble transmission counter is not increased in case of LBT failure, there were some different views. Some companies think even LBT fails, this counter should be increased, otherwise, it will never reach the maximum value in case of systematic UL LBT failures and will introduce unnecessary delay in triggering RLF.

Actually, in NR, the transmission counter is increased even though a power ramping suspension notification is received from the PHY, which means the preamble transmission for this time is dropped. Therefore, we think similar principle should be applied here, i.e., increase the PREAMBLE_TRANSMISSION_COUNTER even when a preamble is not transmitted due to LBT failure.

Proposal 3: PREAMBLE_TRANSMISSION_COUNTER should be incremented when a preamble is not transmitted due to LBT failure.

In this case, since PREAMBLE_TRANSMISSION_COUNTER is incremented regardless of the LBT outcome, RLF in case of continuous LBT failures can be triggered in time. There is no need to introduce another counter to count RACH LBT failures, which may increase the implementation complexity as well as additional impact on the specification.

Proposal 4: There is no need to introduce another counter to count RACH LBT failures

2.1.3 Configured grant timer

Based on RAN1 agreement during SI stage, the retransmission can be performed via configured grant. In our other company contribution, it is proposed that both new transmission and retransmission can be performed for a HARQ process based on configured grant when this configuredGrantTimer is running. When the configured grant timer is running, retransmission or initial transmission with configured grant is prohibited. While the COT for transmission for configured grant is grabbed by UE itself, in order to retransmit the data using configured grant timely, this timer should not be started when the transmission for configured grant is not transmitted due to LBT failure. In that case, LBT outcome related indication from PHY to MAC is needed.

Proposal 5: configuredGrantTimer is not started when configured grant is not transmitted due to LBT failure.

Proposal 5bis: LBT outcome indication from PHY is needed for starting the configuredGrantTimer.

2.1.4 DRX timers: drx-HARQ-RTT-TimerUL

Based on TS 38.321 [2], this timer is started in the first symbol after the end of the first repetition of the corresponding PUSCH transmission. When PUSCH transmission is blocked by LBT, the network will schedule this UE for retransmission in subsequent resource.  If this timer is not started, drx-RetransmissionTimerUL will not be started. In that case the UE may be sleep and cannot receive any dynamic uplink grant for retransmission. To avoid this issue and also aligned with the legacy feLAA and eLAA, this timer should be started regardless of LBT outcome.

Proposal 6: drx-HARQ-RTT-TimerUL is started regardless of LBT outcome for PUSCH transmission for configured grant and dynamic grant.

2.1.5 BWP and SCell deactivation timers: bwp-InactivityTimer and sCellDeactivationTimer

Based on TS 38.321[2], it specifies that

1> if a MAC PDU is transmitted in a configured uplink grant or received in a configured downlink assignment: 2> restart the sCellDeactivationTimer associated with the SCell.

2> if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or 2> if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or 2> if a MAC PDU is transmitted in a configured uplink grant or received in a configured downlink assignment: 3> if there is no ongoing random access procedure associated with this Serving Cell; or 3> if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in subclauses 5.1.4 and 5.1.5): 4> start or restart the bwp-InactivityTimer associated with the active DL BWP.

Following the legacy design in feLAA, the sCellDeactivationTimer should be started regardless of LBT outcome to avoid going to deactivated state unnecessarily. Following the same rationale, BWP inactivity timer should also be restarted since the current "active" BWP is still active.

Proposal 7: bwp-InactivityTimer is started regardless of LBT outcome for PUSCH transmission for configured grant.

Proposal 7bis: sCellDeactivationTimer is started regardless of LBT outcome for PUSCH transmission for configured grant.

2.1.6 SR Timers and counters

In addition, a counter (i.e. SR_COUNTER) is introduced per SR configuration. Each time the MAC layer instruct the PHY to signal the SR, this counter is incremented by 1. When SR transmission attempts reach the maximum number, the following actions are performed.

4>	notify RRC to release PUCCH for all Serving Cells;

4>	notify RRC to release SRS for all Serving Cells;

4>	clear any configured downlink assignments and uplink grants;

4>	initiate a Random Access procedure (see subclause 5.1) on the SpCell and cancel all pending SRs.

In NR-U, the question is whether SR_COUNTER should be incremented by one if the SR transmission is dropped due to LBT failure. If the SR_COUNTER is incremented by one when SR transmission is blocked by LBT, it may be too easy for the SR_COUNTER to reach the maximum number which results in that the PUCCH/SRS configuration is released unnecessarily. To solve this issue, we propose that the SR_COUNTER will not be incremented in case of LBT failure for SR transmission.

Proposal 8: The SR_COUNTER will not be incremented in case of LBT failure for SR transmission.

The next question is in NR-U, whether to start this timer if the SR is not transmitted due to LBT failure. If the sr-ProhibitTimer is started when SR transmission is blocked due to LBT failure, then UE will not to trigger a new SR transmission until this timer expires, which will introduce additional latency for SR transmission. Therefore, it is proposed to start this timer only upon a notification on successful transmission of the SR from physical layer. Actually in our companion contribution [3], we propose to introduce a LBT indication to indicate the LBT failure, we believe this can also be applied for SR transmission, i.e., if no LBT indication is received from PHY, the sr-ProhibitTimer is started.

Proposal 9: For SR transmissions on NR-U SpCell, the corresponding sr-ProhibitTimer is started if there is no LBT failure notification from the physical layer.

2.2 Potential impact for cancelling of SR/BSR trigger

Based on TS 38.321[2], for BSR and SR it specifies that

All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is transmitted and this PDU includes a Long or Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly.

When an SR is triggered, it shall be considered as pending until it is cancelled. All pending SR(s) triggered prior to the MAC PDU assembly shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted and this PDU includes a BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see subclause 5.4.5) prior to the MAC PDU assembly.

Since BSR MAC CE is generated and included in MAC PDU, the network can schedule this UE for retransmission in the subsequent resource. We observe that there are two aspects for the BSR/SR cancelling:

- Whether BSR/SR should be cancelled in case of LBT failure?

If triggered SRs/BSRs is not cancelled, the UE will generate a new BSR MAC CE in the next resource for new transmission and transmit SR on PUCCH in next PUCCH occasion. To avoid transmit duplicated BSR MAC CE/SR, all SRs/BSRs triggered prior to MAC PDU assembly shall be cancelled even if the MAC PDU is blocked due to LBT failure. Hence, following the same rationale as eLAA/feLAA, BSR and SR should be cancelled

- When BSR/SR should be cancelled?

In legacy eLAA/feLAA, BSR and SR are cancelled at the time of BSR MAC CE assembly as shown in the follows:

All triggered BSRs shall be cancelled in case the UL grant(s) in this TTI can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC control element plus its subheader. All triggered BSRs shall be cancelled when a BSR is included in a MAC PDU for transmission.

While in R15 NR, change is made that BSR and SR is cancelled at the time of BSR MAC CE transmission:

All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader. All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is transmitted and this PDU includes a Long or Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly.

We think for NR-U, the time for BSR and SR cancelation should be aligned with R15 NR.

Proposal 10: All pending SRs or triggered BSRs prior to MAC PDU assembly shall be cancelled when the MAC PDU is transmitted regardless of LBT outcome, if this PDU includes a BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly.

2.3 Potential impact for PHR MAC CE

In LTE and Release-15 NR, PHR contains the PH for each activated serving cell. If the serving cell is scheduled in the time instance when PHR is transmitted, then PH value is based on a real transmission and PCMAX,c is also included. Otherwise, the PH value is based on a reference format i.e., virtual PH and PCMAX,c is not included.

In LTE LAA, since LBT in needed before each transmission on LAA SCells, there was some discussion on whether the PH value reported for the LAA SCell be based on a real transmission or a reference format in case of LBT failure and the scheduled transmission on LAA SCell does not happen in practice. There are several options as listed below:

a) PH value based on a real transmission

b) PH value based on a reference format

c) PH value either based on a real transmission or a reference format

Actually if LBT fails, there is no real transmission and the PH value should be based on a virtual format, however LBT happens just before transmission, the MAC entity needs to construct the MAC PDU before LBT procedure and it assumes there is a real transmission on LAA SCell, in this case, there is no time to make any further update on the PH value based on the LBT result. Therefore, in LTE LAA, RAN2 agreed that the content of a MAC PDU (including any PHR value) will not change after it has been built. As an example, it will not change based on the outcome of LBT. Similar principle also applies to NR-U, hence, we propose that

Proposal 11: The content of a MAC PDU (including any PHR value) will not change after it has been built i.e., it will not change based on the outcome of LBT.

3 Conclusion

Based on the discussions above, we have the following observations and proposals:

Observation 1: ra-ResponseWindow is not started when the preamble is not transmitted due to LBT failure.

Observation 2: power ramping is not applied when the preamble is not transmitted due to LBT failure.

Proposal 1: LBT outcome related indication from PHY to MAC is needed for starting of ra-ResponseWindow and performing preamble power ramping.

Proposal 2: ra-ContentionResolutionTimer is started/restarted even when the Msg3/HARQ retransmission is not transmitted due to LBT failure.

Proposal 3: PREAMBLE_TRANSMISSION_COUNTER should be incremented when a preamble is not transmitted due to LBT failure.

Proposal 4: There is no need to introduce another counter to count RACH LBT failures

Proposal 5: configuredGrantTimer is not started when configured grant is not transmitted due to LBT failure.

Proposal 5bis: LBT outcome indication from PHY is needed for starting the configuredGrantTimer.

Proposal 6: drx-HARQ-RTT-TimerUL is started regardless of LBT outcome for PUSCH transmission for configured grant and dynamic grant.

Proposal 7: bwp-InactivityTimer is started regardless of LBT outcome for PUSCH transmission for configured grant.

Proposal 7bis: sCellDeactivationTimer is started regardless of LBT outcome for PUSCH transmission for configured grant.

Proposal 8: The SR_COUNTER will not be incremented in case of LBT failure for SR transmission.

Proposal 9: For SR transmissions on NR-U SpCell, the corresponding sr-ProhibitTimer is started if there is no LBT failure notification from the physical layer.

Proposal 10: All pending SRs or triggered BSRs prior to MAC PDU assembly shall be cancelled when the MAC PDU is transmitted regardless of LBT outcome, if this PDU includes a BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly.

Proposal 11: The content of a MAC PDU (including any PHR value) will not change after it has been built i.e., it will not change based on the outcome of LBT.

4 References

- TR 38.889

- TS 38.321