3GPP TSG-RAN WG2 #105      								R2-1901333

Athens, Greece, 25 Februray – 1 March, 2019

Agenda Item:	11.2.1.2

Souce:	Huawei, HiSilicon

Title:	Discussion on SR for NR-U

Document for:	Discussion and decision

1 Introduction

RAN1 has discussed about the deployment scenarios for NR-based access to unlicensed spectrum and agreed to study the additional functionality needed beyond the specifications for operation in licensed spectrum in the deployment scenarios as listed in [1].

- Carrier aggregation between licensed band NR (PCell) and NR-U (SCell) - NR-U SCell may have both DL and UL, or DL-only. - Dual connectivity between licensed band LTE (PCell) and NR-U (PSCell) - Stand-alone NR-U - An NR cell with DL in unlicensed band and UL in licensed band - Dual connectivity between licensed band NR (PCell) and NR-U (PSCell)

For dual connectivity and standalone scenario where NR-U cell operates as Spcell, SR transmission on unlicensed spectrum needs to be supported.

In RAN2#103bis, it was agreed that:

Agreements: - Power ramping is not applied when preamble is not transmitted due to LBT failure. - Discuss at next meeting to decide on whether PREAMBLE_TRANSMISSION_COUNTER should always be increased independently on the outcome of LBT

In this contribution, we discuss the issue related to SR for NR-U and provide our considerations.

2 Discussion

The Scheduling Request (SR) is used for requesting UL-SCH resources for new transmission when there is new data arrival.

2.1 SR configuration

In unlicensed spectrum, LBT is applied before performing a transmission. If LBT outcome is not succesful, the transmission will be dropped. In LTE LAA, SR is not supported to be transmitted on unlicensed cells since the primary cell is always operated in licensed spectrum. Therefore, there is no impact on SR procedure in LTE LAA. However as mentioned, RAN1 already agreed to support dual connectivity and standalone scenario for NR-U. In that case the SR resource needs be configured on NR-U SpCell. If the SR transmission is dropped due to LBT failure, the UE has to wait for the next SR opportunity and additional scheduling latency is introduced.

Observation 1: SR transmission on an unlicensed carrier may be blocked due to LBT failure, which will introduce additional scheduling latency.

In Rel-15 NR, multiple SR configurations were agreed to be supported. For NR-U, we believe multiple SR configurations can be inherited to NR-U to differentiate traffics based on their QoS requirement.

Proposal 1: Multiple SR configurations can be supported to NR-U cells, same as R15 NR.

2.2 SR trigger and cancellation

2.2.1 SR trigger

In NR, the SR triggering condition is specified as below:

2>	if a Regular BSR has been triggered and logicalChannelSR-DelayTimer is not running:

3>	if there is no UL-SCH resource available for a new transmission; or

3>	if the MAC entity is configured with configured uplink grant(s) and the Regular BSR was triggered for a logical channel for which logicalChannelSR-Mask is set to false; or

3>	if the UL-SCH resources available for a new transmission do not meet the LCP mapping restrictions (see subclause 5.4.3.1) configured for the logical channel that triggered the BSR:

4>	trigger a Scheduling Request.

In last meeting, some company proposed enhancements on SR triggering, e.g., when configured grant is configured and activated logical channels allowed to use the configured grant on unlicensed spectrum may not trigger SR. This is because data or the regular BSR of these logical channels can be sent over the configured grant.

However, we don’t think it is necessary to have such restriction on SR triggering for the following reasons. Firstly, even though the configured grant is configured and activated, it does not mean every transmission opportunity on the configured grant can be utilized as this depends on whether LBT is successful or not. If the configured grant cannot be utilized and SR is not triggered, then unnecessary transmission delay may be introduced. Secondly, SR is triggered does not mean SR is transmitted, that is, when the configured grant resource comes firstly, a MAC PDU including BSR is transmitted or even all pending data available for transmission is accommodated, then SR is cancelled before the next SR transmission opportunity arrives. Thirdly, in Rel-15 FeLAA, pre-allocated resource AUL which is similar as the configured grant in NR-U was introduced without any enhancement on SR triggering since we think it is not a problem to transmit some unnecessary SR. Therefore, based on the above analysis, there is no need to introduce any enhancement on SR triggering in NR-U.

Proposal 2: No enhancement on SR triggering will be introduced in NR-U. No specification changes are needed.

2.2.1 SR cancellation

When an SR is triggered, it shall be considered as pending until it is cancelled. In R15 NR, SR can be cancelled in the following four cases:

- All pending SR(s) triggered prior to the MAC PDU assembly shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted and this PDU includes a Long or Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see subclause 5.4.5) prior to the MAC PDU assembly.

- All pending SR(s) shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the UL grant(s) can accommodate all pending data available for transmission.

- For each SR configuration, its pending SRs are cancelled when the maximum number of SR retransmission is achieved

- When there is no PUCCH resource for the pending SR, initiate RACH and cancel the pending SR

In NR-U, legacy mechanism can be reused and there is no strong motivation to introduce any enhancement on SR cancellation, therefore, we propose RAN2 to confirm that:

Proposal 3: No enhancement on SR cancellation will be introduced in NR-U. No specification changes are needed.

3 Conclusion

Based on the discussions above, we have the following observations and proposals:

Observation 1: SR transmission on an unlicensed carrier may be blocked due to LBT failure, which will introduce additional scheduling latency.

Proposal 1: Multiple SR configurations can be supported to NR-U cells, same as R15 NR.

Proposal 2: No enhancement on SR triggering will be introduced in NR-U. No specification changes are needed.

Proposal 3: No enhancement on SR cancellation will be introduced in NR-U. No specification changes are needed.

4 References

- RAN2#103 chairman note

- RAN1#91 chairman note

- Xxxx

- R2-1816708 Scheduling Request enhancement for NR Unlicensed, Intel