3GPP TSG-RAN WG2 Meeting #105	R2-1901376

Athens, Greece, 25 Feb - 1 Mar 2019

Agenda item:	12.3.1

Source:	China Telecom

Title: 	Way forward on the progress of Rel-16 LTE feMob

Document for:	Discussion and decision

# 1.	Introduction

One new Rel-16 LTE mobility enhancement work item [1] was approved at RAN#80, the objectives of which are to identify and specify the solutions aiming to reduce user data interruption and improve robustness during handover.

In this paper provides our views on how to further progress the work of Rel-16 LTE feMob WI.

# 2.	Discussion

According to the agreed work plan provided by rapporteur [2], in RAN2 #105, RAN2 should finalize study phase work, i.e. RAN2 should finalize solution evaluation and comparison as much as possible, and make decision on the solution selection.

For handover robustness improvements, RAN2 had already agreed to consider Conditional Handover (CHO) for work phase work. For reduction in user data interruption during handover, RAN2 has not concluded on the solution selection.

During the email discussion [104#61][LTE/feMOB] on solution directions for minimizing user data interruption, we observe that some consensuses have been reached, i.e. non-split bearer solution have more supports. Thus the email discussion is mainly to further progress on the non-split bearer group of solutions. Based on these observations, we suggest RAN2 to only consider non-split bearer solution for work phase work.

Proposal 1: Split-bearer solution is not considered for work phase work.

There are two further branches for non-split bearer solutions group, i.e. single active protocol stack and dual active protocol stack, among which the later one has the majority proponents. Compared with single active protocol stack, the interruption time can be smaller or even 0ms for dual active protocol stack solution, since the UE is able to maintain data transmission from both the source cell and the target cell during handover.

In the email report of [104#61][LTE/feMOB], the simultaneous transmission and reception may need RAN1 and RAN4 confirmation and some analysis is copied as below:

  - Whether simultaneous transmission and reception to/from more than one cell is needed (the answer is likely YES, as otherwise the expense of two protocol stack active would be in vain). Obviously, this is very much depending on the awaited answers from RAN1 and RAN4, so could be challenging to completely resolve it now

RAN1 had sent a LS reply to RAN2 [3] and basically RAN1 has confirmed that simultaneous transmission/reception are feasible under some conditions. In addition, RAN1 also points out that for the cases where simultaneous transmission/reception are not possible, some techniques (e.g. TDM at the physical layer) may be applied to allow for simultaneous connection to source and target cells.

Observation: For all the scenarios that have already been concluded by RAN1, simultaneous transmission/reception are feasible.

Based on the above analysis, we have the following proposal:

Proposal 2: RAN2 agree on non-split bearer with dual active protocol stack solution for handover interruption reduction.

# 3.	Conclusion

In this paper, we provide our views on the feMob WI. Based on the latest standard progress, we try to provide a way forward and we have the following observation and proposals:

Observation: For all the scenarios that have already been concluded by RAN1, simultaneous transmission/reception are feasible under some conditions.

Proposal 1: Split-bearer solution is not considered for work phase work.

Proposal 2: RAN2 agree on non-split bearer with dual active protocol stack solution for handover interruption reduction.

# 4.	References

[1]	RP-181544, New Work Item on even further Mobility enhancement in E-UTRAN, China Telecom.

[2]	R2-1814615, Work plan for even further LTE mobility enhancement, China Telecom.

[3] R2-1900020/R1-1814411, Reply LS on the Interruption time during mobility in LTE, Source: RAN1, To: RAN2, RAN4.