3GPP TSG RAN WG2 Meeting #105	R2- 1901381

Athens, February 25 - March 1, 2019

Agenda Item:	11.8.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Positioning protocol adjustments

Document for:	Discussion & Agreement

# Introduction

In RAN#81, the study item on NR Positioning was revised and approved [1]. More specifically, RAN2 and RAN3 have the task to study the positioning architecture for location services, functional interfaces, protocol and procedures for supporting NR dependent and independent positioning technologies. When developing the positioning architecture, the starting point will be Rel-15 NR positioning architecture/protocol while taking into account the Release 16 LCS architecture enhancement study in TSG SA [2]. The main focus of this document is on an extension of the reporting mechanisms needed for achieving the accuracy requirements and other QoS indicators based on the RAN2 email discussion [3].

# NR positioning reporting requirements

Beyond the regulatory use case with relatively relaxed requirements on the location accuracy, the SI description [1] considers commercial use cases. As a basis, SA1 has defined in TS 22.261 [4] seven service levels based on dedicated sets of requirements (horizontal accuracy, vertical accuracy, availability and latency) to be fulfilled in the 5G Positioning Service Area (mostly coinciding with the communication service area) or a 5G Enhanced Positioning Service Area. The idea of positioning service levels has been taken to RAN1, see for example [5]. It has been agreed in RAN1 #95 [6] that the performance targets (seen as a starting point) like

- 10 m horizontal accuracy and 3 m vertical accuracies outdoors and

- 3 m horizontal accuracy and 3 m vertical accuracies indoors (indoor deployment)

are dedicated to commercial use cases and are regarded as a starting point.

RAN2 has decided to regard the LTE Positioning Protocol (LPP) [7] as a starting point for the definition of a Positioning Protocol for NR. In order to enable the positioning service according the specified accuracies, LPP already supports several technologies, where most of them have at least a component based on timing estimation (OTDOA, UTDOA, or RTT measurements). LPP considers RSTD measurement reports of downlink timing measurements for OTDOA positioning and RTT is provided with the UE-RxTxTimeDiff measurement (UTDOA does not require UE signaling). The achievable accuracy of OTDOA positioning relies however on accurate measurement reports, especially, as more accurate timing measurements (compared to Release 14 UEs) become available with the advent of high bandwidths of 100 MHz in FR1 and 400 MHz in FR2 for NR.

In LPP, the RSTD reporting granularity is bound to the definition of 1Ts = 64Tc. RSTD_delta enables to report with a granularity of up to 0.5Ts = 32Tc.

Observation 1: LPP offers reporting accuracy with a granularity up to 0.5 Ts.

Table 1 displays basic reporting granularities and the resulting quantization errors for different signal bandwidths and the according values of SCS and NFFT. Given a uniform distribution of the quantization error, the maximum absolute error and the RMSE for the distance estimates results in

ϵd, quant=c0⋅Tquant, respectively

RMSEd,quant=c0⋅Tquant12.

Table 1 – Basic Reporting granularity.

| Frequency Range | SCS of uplink signals [KHz] | NFFT | Basic Reporting granularity | Corresponding quantization RMSE [m] | Corresponding max quantization error [m] |
| --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1024 | 65ns= 2Ts = 128Tc | 5.63 | 19.5 |
|  | 30 | 1024 | 33ns= 1Ts = 64Tc | 2.86 | 9.9 |
|  | 60 | 1024 | 16ns= 0.5Ts = 32Tc | 1.39 | 4.8 |
|  | 15 | 2048 | 33ns = 1Ts = 64Tc | 2.86 | 9.9 |
|  | 30 | 2048 | 16ns= 0.5Ts = 32Tc | 1.39 | 4.8 |
|  | 60 | 2048 | 8ns= 0.25Ts = 16Tc | 0.69 | 2.4 |
| 2 | 60 | 2048 | 8ns= 0.25Ts = 16Tc | 0.69 | 2.4 |
|  | 120 | 2048 | 4ns= 1/8Ts = 8Tc | 0.35 | 1.2 |
|  | 60 | 4096 | 4ns= 1/8Ts = 8Tc | 0.35 | 1.2 |
|  | 120 | 4096 | 2ns= 1/16Ts = 4Tc | 0.17 | 0.6 |

In order to achieve meter or sub-meter positioning accuracy the quantization errors from Table 1 are obviously too large in any setting for FR1 and even for most settings in FR2. In principle, they should be at least an order of magnitude below the targeted accuracy. Also the more relaxed accuracy figures agreed in RAN1 as benchmark for commercial use cases (see above, [4][5]) remain unachievable according to this assumption.

Observation 2: The reporting granularity of LPP is too coarse for high accuracy position estimates.

Proposal 1: RAN2 shall consider finer estimation granularity reporting via LPP.

# Considerations on LPP

The reported SLS results for timing based methods in RAN1 #95 and RAN1 Ad-Hoc Meeting 1901 show that the achievable accuracies range from few centimeters up to several meters. The results depend mainly on the scenario, measurement configuration (e.g. bandwidth derived from the SCS and NFFT configurations) and the device capabilities. Limited (computational) capabilities and power restrictions of the UE or bandwidth constraints may make high accurate reporting inefficient and, thus, unnecessary. Hence the following factors can define the needed reporting accuracy while keeping signaling overhead low:

- The required accuracy target for example based on the provided service levels in [4] for providing a diverse set of QoS parameters of commercial use cases. A low demanding service level can relax the reporting requirements.

- Measurement configuration (e.g. signal bandwidth) according to the presented considerations on Table 1 which highly influences estimation accuracy especially in challenging channel conditions.

- The UE capabilities have an impact on the achievable accuracy, for example timing estimators performance differ within a given channel environment as shown in [8].The reduced expected estimator accuracies render coarser reporting granularity necessary (but still an order of magnitude lower than the estimator accuracy).

Proposal 2: RAN2 shall consider flexible estimation accuracy reporting based on the measurement configuration (e.g. SCS and NFFT), UE capabilities and the requested QoS.

# Conclusion

In this contribution, adaptations to the NR positioning reporting are discussed. The following observations and proposal are concluded:

Observation 1: LPP offers reporting accuracy with a granularity up to 0.5 Ts.

Observation 2: The reporting granularity of LPP is too coarse for high accuracy position estimates.

Proposal 1: RAN2 shall consider finer estimation granularity reporting via LPP.

Proposal 2: RAN2 shall consider flexible estimation accuracy reporting based on the measurement configuration (e.g. SCS and NFFT), UE capabilities and the requested QoS.

# References

| [1] | 3GPP RP-182155, „Revised SID: Study on NR positioning support,“ Gold Coast, Australia, 2018-09. |
| --- | --- |
| [2] | 3GPP TR 23.731, "Study on enhancement to the 5GC LoCation Services (LCS)" Release 16, 2018-12. |
| [3] | 3GPP, "Email discussion report on [104#35][NR] Positioning SI (Intel)", 2018. |
| [4] | 3GPP TS 22.261, "Service requirements for next generation new services and markets", Rel. 15, 2018-12. |
| [5] | 3GPP R1-1811464, „NR positioning requirements for commercial use cases,“ Chengdu, China, 2018-10. |
| [6] | 3GPP R1-1813849, „Chairman's notes of AI 7.2.10 - Study on NR positioning support“, RAN1-95, Spokane, US, 2018-11. |
| [7] | 3GPP TS 36.355, "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP) - 15.2", Rel.15, 2019-01. |
| [8] | 3GPP R1-1901185, „ SLS results for NR positioning potential solutions,“ Taipei, Taiwan, 2019-01. |