3GPP TSG RAN WG2 Meeting #105	R2-1901388

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	11.8.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	NR-RTT positioning procedure

Document for:	Discussion & Decision

# Introduction

During the last RAN1 [1] meeting, it was agreed to support RTT measurements with one or more gNBs/TRPs for NR DL and UL positioning:

Agreement: NR should support timing (UL-TDOA) based UL-only positioning techniques in FR1 and FR2 - UL-TDOA with two or more neighboring gNBs/TRPs for NR UL positioning should be supported. NR should support UL-AoA based positioning techniques in FR1 and FR2 - UL-AoA with one or more neighboring gNBs/TRPs for NR UL positioning should be supported Round-trip time (RTT) with one or more neighboring gNBs/TRPs for NR DL and UL positioning should be supported for FR1 and FR2 Agreement: The following measurements for NR DL PRS for serving and neighbor gNBs should be supported: UE RX-TX time difference measurements Agreement: Further study whether it is beneficial to support and if so, how to support, NR based positioning for UEs in RRC-IDLE and/or RRC-INACTIVE states

In this contribution we discuss from the perspective of RAN2 the procedure and protocol impacts to support RTT mechanisms involving multiple gNBs and possibly multiple UEs in a scenario (Multi-RTT).

# RTT Discussions and RAN1 Agreements

Compared to TDOA, RTT has the advantage that no tight synchronization is required between involved infrastructure nodes (LMUs or gNBs). This comes at the expense of extra signaling when compared to TDOA. For TDOA, just one-way transmission from a UE to multiple gNBs in the uplink or from multiple gNBs to the UE(s) in the downlink is sufficient to find the device position.

In LTE, the RTT procedure can be performed using the timing advance procedure (TADV) type1 or type2 for E-CID [2]. This implies the use of UE Rx-Tx time difference measurement to compute the distance between the two ranging devices. Three distance measurements are needed between the UE and different gNBs in order to apply unambiguous RTT-based positioning. However in LTE, the Rx-Tx time difference measurements are restricted to the serving eNB and cannot be performed simultaneously for multiple eNB.

For NR, RAN1 agreed in [1] to support RTT with one or more neighboring gNBs/TRPs for NR DL-UL positioning for FR1 and FR2. Multi-RTT should target to provide a mechanism to enable ranging between multiple gNBs with one or more UEs. The mechanism should also take into account the beamforming applied for transmission and reception in UL and DL cases.

Observation 1: 	A RAN2 procedure to enable multiple RTT measurements with neighbor gNBs/TRPs needs to be defined.

# RTT Procedure

We propose the following steps, which aim at maintaining the functionality of an RTT procedure, but at the same time minimizing the signaling overhead:

- Start positioning process:

  - The LMF may receive a target QoS (like accuracy or latency targets) and UE capabilities

  - Set RTT mode, the LMF can firstly identify if ranging shall be performed between:

    - a single or multiple UEs and only the serving gNB (s-gNB),

    - between a single UE or multiple UEs and multiple gNBs

- Initial Measurements:

In case of (1) a procedure similar to E-CID can be applied. For (2), the UE needs to enable the required measurements from the multiple gNBs involved in the positioning procedure. The LMF or LMC can identify the neighboring gNBs (n-gNBs) to be considered according to the following procedure:

  - Identify the n-gNBs in range based on:

    - UE reports on the detectable gNBs (incl. beam report from n-gNBs), or

    - the network information which resembles the procedure for OTDOA or UTDOA

  - Set the UL resources for the UEs and the DL resources for the serving and neighboring gNBs according to:

    - LMF or s-gNB configures the n-gNB over NRPPa (LMF) or Xn (s-gNB)

    - n-gNBs reserve and acknowledge the resources that are assigned in UL and DL for positioning

    - s-gNB configures the RRC measurements with the target UE(s):

      - Set either UE or gNBs as initiator and responder within the RTT scheme

      - Specify initiator/responder frequency and time resources and group shared resources.

- Measurement Signaling procedure:

  - The initiator (UE/gNB) sends a reference signal(s) to one or more responder nodes (gNB/UE)

  - If the responders receive the signal with sufficient quality, then the responder nodes reply using the allocated resources sending reference signals and report the Rx-Tx time difference measurements

    - to the s-gNB that forwards them to the UE (if the UEs are the initiators).

    - to the s-gNB that can forward them to to the LMF (if the gNBs are the initiators).

  - (Optional) Initiator performs additional measurement for refinement or correction

  - In case a set of responders did not reply or the reply information has low quality, the initiator or the server can request new resources for measurements with adjustment to parameters like transmit power, periodicity, signal duration or timing adjustments.

- Positioning / Ranging Measurements:

  - UE-Based: signaling needed

The information that may be signaled from LMF or gNB to the UE so the UE can estimate its position:

| Information | UE-Based |
| --- | --- |
| gNB reply time or Rx-Tx time difference | Yes |
| gNB transceiver delays (calibration) | Yes |
| gNB coordinates | Yes |

  - UE-Assisted

The information that may be signaled from UE to the LMF

| Information | UE-assisted |
| --- | --- |
| UE reply time or Rx-Tx time difference | Yes |
| UE transceiver delays (calibration) | Yes |

The information that may be signaled from gNB to the LMF for UE assisted are:

| Information | UE-assisted |
| --- | --- |
| gNB reply time or Rx-Tx time difference | Yes |
| gNB transceiver delays (calibration) | Yes |
| gNB coordinates | Yes |

Observation 2: 	The number of neighbor gNBs involved in the RTT measurements can be identified based on the UE report or network information.

Proposal 1: 	Consider the proposed RTT scheme to support ranging with multiple gNBs and multiple UEs.

# Additional RTT Consideration

Hardware delay calibration, TOA estimation quality, timing resolution from Rx-Tx reporting and clock oscillator quality are among the main factors that control the accuracy of the range estimates. For example, the components in the transceiver chain like filters, amplifiers and transmission lines add additional delay to the signal. This makes timing calibration necessary before measurements can be evaluated accurately. Note that a calibration error of 1 ns translates to a one-way distance error of 30 cm. The transceiver delays can vary between different NG-RAN architectures and devices or components from different manufacturers. Even manufacturing tolerances as well as temperature and ageing may affect internal delays of the whole device.

For RAN2, this implies that additional information needed for the accurate RTT position needs to be taken into account, an example for transceiver delays compensation the LPP extension example is shown below:

RTT-Measurements ::= SEQUENCE {

transceiverDelaysFR1 				    0-4096	-- value in ‘Tc’						OPTIONAL 	transceiverDelaysFR2 				    0-4096	-- value in ‘Tc’						OPTIONAL

…

}

Proposal 2: 	Consider reporting required to mitigate factors limiting RTT accuracy like hardware delay calibration or Rx-Tx estimation quality in LPP.

# Conclusion

The following proposals and observation are concluded:

Observation 1: 	A RAN2 procedures to enable RTT measurements with neighbor gNBs/TRPs needs to be defined.

Observation 2: 	The number of neighbor gNBs involved in the RTT measurements can be identified based on the UE report or network information.

Proposal 1: 	Consider the proposed RTT scheme to support ranging with multiple gNBs and multiple UEs.

Proposal 2: 	Consider reporting required to mitigate factors limiting RTT accuracy like hardware delay calibration or Rx-Tx estimation quality in LPP.

# References

[1] 3GPP “Chairman’s notes on AI 7.2.10”, Taipei, Taiwan, 2019-01.

[2] 3GPP TS 36.305, “Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN”, (V15.1.0).