3GPP TSG RAN WG2 Meeting #105                                                                      R2-1901519

Athens, Greece, 25 February – 1 March 2019

Agenda item:	11.4.2.1

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	NR V2X support for Groupcast Communication

Document for:	Discussion

# Introduction

During the RAN2#104 meeting [1], the following agreements were made:

Agreements on groupcast 6: Further discussion is needed on whether groupcast follows same mechanism for unicast, which are agreed in the above. 7: No AS-level mechanism to determine a group manager (i.e. head UE) is studied. FFS for platooning, on the visibility of a group manager (head UE) to AS and AS-level functionalities.

This contribution aims to clarify the need for AS visibility of the platoon (group) leader UE as well as the mechanism in which PC5-RRC can operate in Groupcast.

# Groupcast Communication

## AS Visibility of Platoon Leader UE

Groupcast communications is seen as one of the key enablers for platooning as described in [2] and [3].  Platooning can be defined as a coordinated group of vehicles travelling for long periods with a group leader UE. In this scenario, sensor data and other relevant safety critical information would have to be shared among relevant group members with the group leader UE responsible for managing the platoon operations.

The AS visibility of the group leader UE offers several benefits from a network perspective.

It enables the group’s resource management by the group leader. This enables not only efficient resource allocation in Mode 1 and Mode 2 but also enables the leader to solve the half-duplex problem within the group. This problem comes from a UE not being able to receive data from other UEs while transmitting data. Hence in order to avoid missing important transmissions coordination is necessary.

Group resource management further helps in achieving stringent QoS requirements defined in [2]. By avoiding low QoS transmissions interfering with more important transmissions.

Furthermore, the AS visibility of the group leader can be used to optimize mobility for groupcast communications. Given that the group leader manages group resources it can also assist in enhancing the mobility performance for its group within the network.

It reduces the specification effort to develop a groupcast-specific RRC design by considering unicast PC5-RRC connections to the group leader. This ensures a consistent group RRC state being handled through multiple unicast PC5-RRC links between the leader UE and its members.

Observation 1: AS visibility of the group leader has a set of clear benefits.

Observation 2: Unicast PC5-RRC between the group leader and its members enables a consistent group RRC state.

Proposal 1: Platoon leader UE is visible in the AS layer.

## PC5-RRC for Groupcast

In Rel. 13 D2D, ProSe direct discovery was used in the detection and identification of another proximal UE. The restricted discovery mechanism required explicit permission from the potential UE being discovered, which was supported by Model A and Model B [4]. It should be further investigated if such discovery procedures have to be enhanced in order to be inherited for unicast sidelink (SL) in NR V2X.

Furthermore, one-to-one ProSe direct communication was already realized by the PC5 signaling protocol, which included link establishment, maintenance and release procedures. The transmitting UE initiating the unicast communications acquires the layer-2 ID of the recipient UE either through the discovery procedures or from previously participated one-to-many ProSe communications which already included the recipient UE.

Observation 3: The discovery procedures and PC5 signaling protocol have already been supported in Rel. 13 D2D.

The unicast procedures were initially targeted for proximity-based services and are different from the stringent QoS requirements demanded by NR V2X. RRC signaling includes the functionality which can enhance PC5 link management such as:

- Establishment/maintenance/release of signaling and data radio bearers.

- Different radio configuration control procedures such as ARQ, HARQ configurations, CSI reporting (enhancing feedback control).

- Capability to recover from RLF.

- Measurement setup and reporting.

In addition, PC5-RRC signaling can enable coordinated control among UEs in out-of-coverage scenarios in unicast and groupcast. The introduction of RRC signaling on the sidelink has certain advantages (e.g. link robustness) with respect to the exchange of AS-layer information, which need to be considered.

This decision is up to RAN2 to be finalized as noted in Key Issue 9 and Solution 11 of [5].

Observation 4: RRC signaling offers a set of features, which can enhance PC5 link management.

Proposal 2: RAN2 should study and consider the benefits of SL establishment/maintenance/release procedures in the context of the PC5 signaling protocol and PC5-RRC schemes.

# Conclusion

The following observations were noted based on the above discussion:

Observation 1: AS visibility of the group leader has a set of clear benefits.

Observation 2: Unicast PC5-RRC between the group leader and its members enables a consistent group RRC state.

Observation 3: The discovery procedures and PC5 signaling protocol have already been supported in Rel. 13 D2D.

Observation 4: RRC signaling offers a set of features, which can enhance PC5 link management.

As a result, the following proposals are provided:

Proposal 1: Platoon leader UE is visible in the AS layer.

Proposal 2: RAN2 should study and consider the benefits of SL establishment/maintenance/release procedures in the context of the PC5 signaling protocol and PC5 RRC schemes.

# References

- R2-1818527, “Report from session on V2X (LTE and NR)”, Intel, 3GPP RAN2#104, November 2018.

- TR 22.886, “Study on enhancement of 3GPP Support for 5G V2X Services”, V16.0.0, 3GPP SA#80, June 2018.

- RP-181064, “Study on NR V2X”, Vodafone, 3GPP RAN#80, June 2018.

- TS 23.303, “Proximity-based services (ProSe); Stage 2 (Release 15)”, V15.1.0, 3GPP, June 2016.

- TR 23.786, “Study on architecture enhancements for EPS and 5G System to support advanced V2X services (Release 16)”, 3GPP, V0.9.0, October 2018.