3GPP TSG-RAN WG2 #105				R2-1901529

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.8.2

Source:	Ericsson

Title:	Study on Location functions component in RAN

Document for:	Discussion, Decision

# Introduction

RAN2 has been tasked to study the impacts of having Location exposure functionality in RAN (gNB). Broadly, there are two possibilities where by a RAN node (gNB) could be aware of the target device position.

- Being a client and fetching the location information from location server

- Being a server and calculating the UE position with its own means

RAN2 should study the impacts before concluding on the selection of either of the above two options or to select both options or to reject both.

In this paper, however we find a middle ground where gNB has the component needed to compute the location, but it does not need any new protocol or container-based solution etc.

# Discussion on Location Component Functionality

There can be several reasons where by having location of UE may simplify RAN procedure such as for mobility decisions (if PCI collisions/confusions occurs), input to beam former (not necessarily to track UE in real time but more so to predict number of beams required, narrow beam or wide beams etc), identification of coverage issue (if UE can report location info during RLF) etc.

E-CID procedure has been used in LTE and would also be used in NR which provides necessary measurement results for the Network to pin-point the UE location.

For instance, currently using E-CID procedure UE reports below.

It is expected that angular measurements could also o be part of E-CID which will further help to NW to determine the UE location.

-- ASN1START

# ECID-SignalMeasurementInformation ::= SEQUENCE {

primaryCellMeasuredResults	MeasuredResultsElement	OPTIONAL,

measuredResultsList			MeasuredResultsList,

...

}

# MeasuredResultsList ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsElement

# MeasuredResultsElement ::= SEQUENCE {

physCellId						INTEGER (0..503),

cellGlobalId					CellGlobalIdEUTRA-AndUTRA	OPTIONAL,

arfcnEUTRA						ARFCN-ValueEUTRA,

systemFrameNumber				BIT STRING (SIZE (10))		OPTIONAL,

rsrp-Result						INTEGER (0..97)				OPTIONAL,

rsrq-Result						INTEGER (0..34)				OPTIONAL,

ue-RxTxTimeDiff					INTEGER (0..4095)			OPTIONAL,

...,

[[	arfcnEUTRA-v9a0				ARFCN-ValueEUTRA-v9a0		OPTIONAL		-- Cond EARFCN-max

]],

[[	nrsrp-Result-r14			INTEGER (0..113)			OPTIONAL,

nrsrq-Result-r14			INTEGER (0..74)				OPTIONAL,

carrierFreqOffsetNB-r14		CarrierFreqOffsetNB-r14		OPTIONAL,		-- Cond NB-IoT

hyperSFN-r14				BIT STRING (SIZE (10))		OPTIONAL

]],

[[

rsrp-Result-v1470			INTEGER (-17..-1)			OPTIONAL,

rsrq-Result-v1470			INTEGER (-30..46)			OPTIONAL

]]

}

-- ASN1STOP

It would benefit if UE provides such information in RRC message to the gNB so that the gNB has the necessary components to identify the user location. The RRC Reconfiguration message can be extended for this purpose so that NW can configure the necessary components that is needed and thus UE can provide that in the measurement report.

gNB to support E-CID positioning method.

E-CID be defined also as a gNB based positioning procedure.

The below text proposal be captured in the TR 38.555

# Text Proposal

## 9.2	Procedure and protocol aspects

### 9.2.1	E-CID Based upon gNB

There can be several reasons where by having location of UE may simplify RAN procedure such as for mobility decisions (if PCI collisions/confusions occurs), input to beam former (not necessarily to track UE in real time but more so to predict number of beams required, narrow beam or wide beams etc), identification of coverage issue (if UE can report location info during RLF) etc.

E-CID procedure has been used in LTE and would also be used in NR which provides necessary measurement results for the Network to pin-point the UE location.

For instance, currently using E-CID procedure UE reports below.

It is expected that angular measurements could also o be part of E-CID which will further help to NW to determine the UE location.

-- ASN1START

# ECID-SignalMeasurementInformation ::= SEQUENCE {

primaryCellMeasuredResults	MeasuredResultsElement	OPTIONAL,

measuredResultsList			MeasuredResultsList,

...

}

# MeasuredResultsList ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsElement

# MeasuredResultsElement ::= SEQUENCE {

physCellId						INTEGER (0..503),

cellGlobalId					CellGlobalIdEUTRA-AndUTRA	OPTIONAL,

arfcnEUTRA						ARFCN-ValueEUTRA,

systemFrameNumber				BIT STRING (SIZE (10))		OPTIONAL,

rsrp-Result						INTEGER (0..97)				OPTIONAL,

rsrq-Result						INTEGER (0..34)				OPTIONAL,

ue-RxTxTimeDiff					INTEGER (0..4095)			OPTIONAL,

...,

[[	arfcnEUTRA-v9a0				ARFCN-ValueEUTRA-v9a0		OPTIONAL		-- Cond EARFCN-max

]],

[[	nrsrp-Result-r14			INTEGER (0..113)			OPTIONAL,

nrsrq-Result-r14			INTEGER (0..74)				OPTIONAL,

carrierFreqOffsetNB-r14		CarrierFreqOffsetNB-r14		OPTIONAL,		-- Cond NB-IoT

hyperSFN-r14				BIT STRING (SIZE (10))		OPTIONAL

]],

[[

rsrp-Result-v1470			INTEGER (-17..-1)			OPTIONAL,

rsrq-Result-v1470			INTEGER (-30..46)			OPTIONAL

]]

}

-- ASN1STOP

It would benefit if UE provides such information in RRC message to the gNB so that  gNB has the necessary components to identify the user location. The RRC Reconfiguration message can be extended for this purpose so that NW can configure the necessary components that is needed and thus UE can provide that in the measurement report.

gNB to support E-CID positioning method.

E-CID be defined also as a gNB based positioning procedure.

# Conclusion

In section 2 we made the following observations:

Observation 1	gNB to support E-CID positioning method.

Based on the discussion in section 2 we propose the following:

Proposal 1	E-CID be defined also as a gNB based positioning procedure.

Proposal 2	The below text proposal be captured in the TR 38.555

# References

SA2, 23.731 “Study on Enhancement to the 5GC Location Services”