3GPP TSG-RAN WG2 #105	 TDoc R2-1901651

# Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.4.2.1

Source:	Ericsson

Title:	Mode-1 Implications for Supporting SL HARQ feedbacks

Document for:	Discussion, Decision

# Introduction

The study of the unicast sidelink V2X communications is included in the SID [1]. Current LTE D2D communication framework is mainly tailored for broadcast/groupcast communications, therefore the introduction of one-to-one communication may bring to potentially large specification impacts and L1/L2 system design challenges.

For example, the introduction of unicast communication may call for various enhancements to the sidelink interface, such as SL HARQ feedback.

In our companion paper [2], we provide a general overview related to the introduction of SL HARQ feedbacks. In this paper, we only focus on the implications on the mode-1 scheme both at UE and network side.

# Discussion

In mode-1, the resource allocation is entirely controlled by the gNB. Therefore, the HARQ feedback should be delivered to the gNB.

The first aspect that RAN2 should study is whether the HARQ feedback should be sent by the transmitter or by the receiver peer. On one hand, it seems more reasonable that is the receiving UE that sends directly the HARQ feedback to the gNB, rather than to the transmitter. This would have the benefit to reduce SL overhead, and reduce HARQ feedback loop. However, there is the issue that the receiving UE might not be connected to the same cell in which the transmitting UE is connected. Therefore, it is probably less complex to make the transmitting UE report the HARQ feedback to the gNB.

The additional benefit is that there will be a common UE behaviour on the HARQ protocol works over the SL, irrespective of whether the UE is configured with mode-1 or mode-2 or it is involved in unicast or groupcast communication.

As proposed in our paper [2], it is important both from UE complexity and specification impact that RAN2 aims at a unified SL HARQ protocol framework, irrespective of whether the UE is performing mode-1 or mode-2 or it is involved in unicast or groupcast communication.

For HARQ feedback on mode-1, it is the transmitting UE that sends the HARQ feedback to the gNB, upon receiving the HARQ feedback from the receiving UE via SL.

Another issue related to the SL HARQ/CSI feedback report to the gNB is obviously the need to somehow indicate the link to which the HARQ/CSI feedback refers to, considering that a certain UE might be potentially involved in more than one unicast traffic flow. And of course, for the SL HARQ feedbacks, there should be the possibility to indicate the SL HARQ process(s) to which a given HARQ feedback refers to.

Regarding this issue, it seems that coordination with RAN1 is needed, since that might impact the PUCCH format and content.

There should be the possibility for the gNB to identify the unicast traffic flow to which a certain SL HARQ feedback refers to. Details up to RAN1.

There should be the possibility for the gNB to identify the SL HARQ process(s) to which a certain SL HARQ feedback refers to. Details up to RAN1.

In LTE, the SL grant provided by the gNB comprises resources for both initial transmissions and retransmissions for a given transport block (TB). We believe that this approach can be applicable also to NR when SL HARQ feedbacks are supported. In fact, assuming that the transmitting UE provides HARQ feedbacks following each (re)transmission, the gNB can easily reallocate resources previously reserved for retransmissions. However, since HARQ feedbacks are supported, we believe that it is also important to support the case in which the gNB provides individual SL grant for initial transmissions and retransmissions, depending on the HARQ feedback of each single transmission. This latter approach has the advantage that the gNB can avoid pre-allocating resources for initial transmission and (re)transmissions in one shot, which could lead to inefficient resource allocation especially in high load scenarios. The gNB can also decide the number of HARQ retransmission resources which are indicated in the SL grant depending on reliability/latency requirements.

Therefore, in order to allow full network flexibility when it comes to scheduling decisions, we believe that both scheduling approaches should be supported.

The gNB may provide a SL grant including resources for both initial transmission and retransmission for each MAC PDU (as in LTE), or it can provide independent SL grant for each (re)transmission.

Another aspect is on the PUCCH resources for transmitter UE to report the HARQ feedback to gNB. We believe that PUCCH resources for SL HARQ feedback are dedicated resources configured by the gNB and indicated in DCI together with SL grant. When it comes to where the gNB can put PUCCH resources for the SL HARQ feedback reporting, it is up to the gNB implementation e.g. after each and every PSFCH or sparser in time to save some PUCCH resource. The gNB can configure the PUCCH resource for SL HARQ feedback depending on the PUCCH’s occupancy e.g., if PUCCH is rather empty, more frequent HARQ feedback reporting resources is configured, otherwise sparer HARQ feedback reporting resources is configured.

PUCCH resources for SL HARQ feedback are dedicated resources configured by the gNB and indicated in DCI together with SL grant.

Therefore, the transmitting UE will transmit on the first available PUCCH resource, the HARQ status for a given HARQ process indicating ACK or NACK depending on whether the latest HARQ feedback received on the PSFCH is ACK or NACK.

The transmitting UE reports on the first available PUCCH resources ACK or NACK depending on whether the latest HARQ feedback received on the PSFCH is ACK or NACK.

Obviously, similar to the mode-2 case as described in [2], when the transmitter UE receives ACK on PSFCH, it will not retransmit on resources previously granted for retransmissions by the gNB. On the other hand, if the feedback status is NACK, the UE will retransmit on the resources previously granted for retransmissions by the gNB. If SL HARQ feedback is not received at n+k at the transmitter UE, the transmitter UE will assume the HARQ feedback is NACK.

When the SL HARQ feedback received on PSFCH is ACK, the transmitter UE will not retransmit on resources previously granted for retransmissions by the gNB, if any.

When the HARQ feedback received on PSFCH is NACK, or PSFCH is missing for this (re)transmission, the transmitter UE will retransmit on resources previously granted for retransmissions by the gNB, if any.

As said above, if transmitter UE reports ACK to the gNB, the gNB can reassign the remaining granted resources to other UEs. In this way, the resource utilization is improved. On the other hand, if the gNB receives NACK, a previously assigned resource for retransmission can be overridden by the gNB. In other words, upon receiving NACK, gNB considers the resource previously selected for transmissions is not good and could select another one.

If the gNB receives NACK, a previously assigned resource for retransmission may be overridden by a new SL grant.

It is proposed to agree the TP in the Annex.

Agree the Text Proposal to TP 38.885 in Annex.

# Conclusion

In section 2 we made the following observations:

Observation 1	As proposed in our paper [2], it is important both from UE complexity and specification impact that RAN2 aims at a unified SL HARQ protocol framework, irrespective of whether the UE is performing mode-1 or mode-2 or it is involved in unicast or groupcast communication.

Based on the discussion in section 2 we propose the following:

Proposal 1	For HARQ feedback on mode-1, it is the transmitting UE that sends the HARQ feedback to the gNB, upon receiving the HARQ feedback from the receiving UE via SL.

Proposal 2	There should be the possibility for the gNB to identify the unicast traffic flow to which a certain SL HARQ feedback refers to. Details up to RAN1.

Proposal 3	There should be the possibility for the gNB to identify the SL HARQ process(s) to which a certain SL HARQ feedback refers to. Details up to RAN1.

Proposal 4	The gNB may provide a SL grant including resources for both initial transmission and retransmission for each MAC PDU (as in LTE), or it can provide independent SL grant for each (re)transmission.

Proposal 5	PUCCH resources for SL HARQ feedback are dedicated resources configured by the gNB and indicated in DCI together with SL grant.

Proposal 6	The transmitting UE reports on the first available PUCCH resources ACK or NACK depending on whether the latest HARQ feedback received on the PSFCH is ACK or NACK.

Proposal 7	When the SL HARQ feedback received on PSFCH is ACK, the transmitter UE will not retransmit on resources previously granted for retransmissions by the gNB, if any.

Proposal 8	When the HARQ feedback received on PSFCH is NACK, or PSFCH is missing for this (re)transmission, the transmitter UE will retransmit on resources previously granted for retransmissions by the gNB, if any.

Proposal 9	If the gNB receives NACK, a previously assigned resource for retransmission may be overridden by a new SL grant.

Proposal 10	Agree the Text Proposal to TP 38.885 in Annex.

# References

RP-181429, Study on NR V2X, 3GPP TSG RAN Meeting #80, La Jolla, USA, June 11th -14th, 2018

R2-1901658, On the Support of HARQ feedbacks Over Sidelink, Ericsson, RAN2#105, Athens, Greece, Feb 2019.

# Annex – Text Proposal for TR 38.885

## NR sidelink unicast, groupcast, and broadcast design

For the purposes of physical layer analysis, it is assumed that higher layers decide if unicast, groupcast, or broadcast transmission is to be used for a particular data transfer, and they correspondingly inform the physical layer. When considering a unicast or groupcast transmission, it is assumed that the UE is able to establish which unicast or groupcast session a transmission belongs to, and that the following information is known to the physical layer:

-	Identities:

-	Unicast: destination ID, FFS: source ID

-	Groupcast: Destination group ID, FFS source ID

-	HARQ process ID (FFS for groupcast)

In this and other sections, the following topics are studied for unicast and/or groupcast: HARQ feedback, CSI acquisition, open-loop and/or closed-loop power control, link adaptation, multi-antenna transmission scheme.

For HARQ/CSI feedback for mode-1 operations, it is assumed that HARQ/CSI feedback on mode-1, the transmitting UE sends the HARQ/CSI feedback to the gNB, upon receiving the HARQ/CSI feedback from the receiving UE via SL. The following aspects are also studied for mode-1 HARQ/CSI feedbacks:

- How the gNB identifies the unicast connection to which a certain SL HARQ/CSI feedback refers to.

- How the gNB identifies the SL HARQ process(s) to which a certain SL HARQ feedback refers to.

- The gNB may provide a SL grant including resources for both initial transmission and retransmission for each MAC PDU (as in LTE), or it can provide independent SL grant for each (re)transmission

With respect to mode 1 SL HARQ protocol design, the following aspects are proposed:

- The gNB may provide a SL grant including resources for both initial transmission and retransmission for each MAC PDU (as in LTE), or it can provide independent SL grant for each (re)transmission.

- PUCCH resources for SL HARQ feedback are dedicated resources configured by the gNB and indicated in DCI together with SL grant

- The transmitting UE reports on the first available PUCCH resources ACK or NACK depending on whether the latest HARQ feedback received on the PSFCH is ACK or NACK

- PUCCH resources for SL HARQ feedback are dedicated resources configured by the gNB or the

network and indicated in DCI together with SL grant.

- When the SL HARQ feedback received on PSFCH is ACK, the transmitter UE will not retransmit on resources previously granted for retransmissions by the gNB, if any.

- When the HARQ feedback received on PSFCH is NACK, or PSFCH is missing for this (re)transmission, the transmitter UE will retransmit on resources previously granted for retransmissions by the gNB, if any.

- If the gNB receives NACK, a previously assigned resource for retransmission may be overridden by a new SL grant