3GPP TSG-RAN WG2 #105	 TDoc R2-1901657

# Athens, Greece, 25th Feburary– 1st March 2019

Agenda Item:	11.4.2.2

Source:	Ericsson

Title:		Configuration of Unicast/Groupcast/Broadcast Sidelink V2X    Communications

Document for:	Discussion, Decision

# Introduction

The study of the unicast, groupcast, broadcast sidelink V2X communications is included in the SID [1]. Current LTE D2D communication framework is mainly tailored for broadcast/groupcast communications, therefore the introduction of one-to-one communication may bring to potentially large specification impacts and L1/L2 system design challenges.

In this paper, we provide an overview of some of the technical aspects that should be discussed during the study item phase, mainly focusing on how to differentiate unicast/groupcast/broadcast transmissions in terms of configuration provisioning.

# Discussion

Even though one-to-one direct communication is supported in Prose, e.g. in the context of relay communications, the impact on L1/L2 procedure is minimal. According to [2], the UE can be configured by upper layers to perform one-to-many (i.e. broadcast or groupcast) communications or one-to-one communications. Depending on the type of communication, a L2 link ID is also provided by higher layers, which is then used in the MAC PDU header. Therefore, apart from this small MAC impact, there are no other L1/L2 procedures which are affected by the selection of a unicast transmission scheme. In fact, for V2X all the RAN procedures have been designed with broadcast use cases in mind.

The legacy LTE sidelink design only targets broadcast V2X use cases.

To support sidelink unicast/groupcast/broadcast at access stratum, UE has to know whether a V2X packet should be transmitted by unicast, groupcast or broadcast as well as the relevant L1/L2 ID(s). In the latest reply LS from SA2 to RAN2, SA2 confirms the following information will be provided from upper layer to access stratum:

From reply LS from SA2 to RAN2: Question: RAN2 respectfully asks SA2 to take the above information into account, and provide feedback on whether/how the following can be provided by the upper layers: - Information on whether a certain V2X packet should be transmitted by unicast, groupcast or broadcast; - Destination ID for a specific group for groupcast; - Destination ID for the target UE for unicast; - Source UE ID. SA2 answer: SA2 would like to confirm the following to RAN2: - Yes. Information on whether a V2X packet should be transmitted by unicast, groupcast, or broadcast will be indicated by upper layer to AS layer; - Destination ID for a specific group for groupcast will be provided by upper layer to AS layer; - Destination ID for the target UE for unicast will be provided by upper layer to AS layer; - the source UE ID will be provided by upper layer to AS layer.

Upper layer provides the following information to access stratum: information on whether a certain V2X packet should be transmitted by unicast, groupcast or broadcast; destination ID for a specific group for groupcast; destination ID for the target UE for unicast; source UE ID.

Whether the selection of unicast/groupcast/broadcast has some impact on the resource allocation procedures is obviously in the scope of RAN1 and RAN2.

In Prose Communications, the resource allocation is agnostic with respect to the casting scheme used. The same principles could be used also in V2X, especially when the UE is in IDLE mode or when out of coverage. In fact, configuring different pools for IDLE mode or out of coverage operations depending on the casting scheme does not seem to be reasonable. The network might not be aware of the actual cell load and of the casting scheme in use, therefore this solution would just lead to potentially high and unpredictable resource wastage. On the other hand, we believe that for IDLE mode or out of coverage operations, the mode-2 resource pool design should allow coexistence of different casting schemes; for example, the resource reservation scheme introduced in Rel.14 may be already sufficient at least at low load.

For in coverage and IDLE mode operations, a single mode-2 pool is provided by broadcast signalling for all casting schemes.

For out of coverage operations, a single preconfigured mode-2 pool is provided for all casting schemes.

On the other hand, for CONNECTED mode operations, there is the possibility for the network to provide dedicated pool configurations, either mode-1 and/or mode-2, depending on the casting scheme required by the UE and on the specific V2X service. This would obviously be beneficial for the transmitter which can enjoy less interfered transmissions, and also the receiver which can prioritize the reception of packets on certain resources. 
For example, at connection establishment the UE can indicate to the network that it desires dedicated resources for unicast/multicast transmission for a certain V2X service, and in case the UE is authorized to perform the casting scheme, the network may configure a mode1/mode-2 pool and indicate the casting scheme to use.

For CONNECTED mode operations, the network can configure a mode-1/mode-2 resource pool, on the basis of UE signalling and UE authorization to perform unicast/groupcast/broadcast sidelink V2X communications for a certain V2X service.

In the email discussion [3], it was discussed whether the Proposal 3 above implies that the gNB when providing the mode-1/mode-2 resource pool should somehow know the identities of the UEs involved in the same unicast/groupcast communication, or the link IDs. Taking as an example SL unicast, this would allow the gNB to provide a mode-1 resource pool which is common for both UEs involved in the same SL unicast communication. However, that would put a requirement also on UE side. In fact, whenever mode-1 is supported, both UEs should go in connected mode and receive the mode-1 pool plus the DCI information from the gNB, which is obviously not always feasible, given that the two UEs can be in different coverage status. Instead, we believe that it is much simpler if we just follow LTE rules, i.e. the two UEs get independent mode-1 pool configuration and are scheduled independently by the gNB.

The same reasoning applies also for broadcast and groupcast communication, especially if no platoon header is defined, and also for mode-2 configuration.

For all mode-1 and mode-2 SL communications (at least if no platoon header is defined), each UE involved in the same SL unicast/groupcast/broadcast communication gets the mode-1/mode-2 resource pool configuration independently of each other.

However, that said, we believe it is beneficial if the UE when entering connected mode, provides some indication related to the unicast/groupcast communication for which SL mode-1/mode-2 resources are requested. That would be important since a UE may be involved in multiple SL unicast/groupcast communications, and the gNB may want to provide a different resource pool for the different SL communications in which the UE is currently involved.

A UE may be involved in multiple SL unicast/groupcast communications, for which the gNB may want to provide different mode-1/mode-2 pool configuration.

When entering connected mode to request mode-1/mode-2 resources, a UE indicates for which specific SL group/unicast communication the SL resources are requested.

# Conclusion

In section 2 we made the following observations:

Observation 1	The legacy LTE sidelink design only targets broadcast V2X use cases.

Observation 2	Upper layer provides the following information to access stratum: information on whether a certain V2X packet should be transmitted by unicast, groupcast or broadcast; destination ID for a specific group for groupcast; destination ID for the target UE for unicast; source UE ID.

Observation 3	A UE may be involved in multiple SL unicast/groupcast communications, for which the gNB may want to provide different mode-1/mode-2 pool configuration.

Based on the discussion in section 2 we propose the following:

Proposal 1	For in coverage and IDLE mode operations, a single mode-2 pool is provided by broadcast signalling for all casting schemes.

Proposal 2	For out of coverage operations, a single preconfigured mode-2 pool is provided for all casting schemes.

Proposal 3	For CONNECTED mode operations, the network can configure a mode-1/mode-2 resource pool, on the basis of UE signalling and UE authorization to perform unicast/groupcast/broadcast sidelink V2X communications for a certain V2X service.

Proposal 4	For all mode-1 and mode-2 SL communications (at least if no platoon header is defined), each UE involved in the same SL unicast/groupcast/broadcast communication gets the mode-1/mode-2 resource pool configuration independently of each other.

Proposal 5	When entering connected mode to request mode-1/mode-2 resources, a UE indicates for which specific SL group/unicast communication the SL resources are requested.

# References

RP-181429, Study on NR V2X, 3GPP TSG RAN Meeting #80, La Jolla, USA, June 11th -14th, 2018

TS 23.303, Proximity-based services (ProSe)

R2-19xxxxx, Report of [104#59][NR/V2X] Resource allocation (LG)