3GPP TSG-RAN WG2 #105	 TDoc R2-1901659

# Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.4.2.1

Source:	Ericsson

Title:	On the Support of Sidelink CSI feedbacks

Document for:	Discussion, Decision

# Introduction

The study of the unicast and groupcast sidelink V2X communications is included in the SID [1]. Current LTE D2D communication framework is mainly tailored for broadcast communications, therefore the introduction of one-to-one and one-to-many communication may bring to potentially large specification impacts and L1/L2 system design challenges.

For example, the introduction of unicast and groupcast communication may call for various enhancements to the sidelink interface, such as SL HARQ feedback and SL CSI reports

In this paper, we provide an overview of some of the technical aspects related to the introduction of SL CSI feedbacks.

# Discussion

In our understanding CSI transmissions are beneficial to enforce unicast and potentially also groupcast communication. Yet, how to use such enhancements to aid groupcast communication might be more challenging. In sidelink there is no central controller, and it might be not straight forward to specify UE actions on the basis of multiple CSI feedbacks received. For example, appropriate rules need to be specified to avoid the risk that the transmitter performances are bounded by the worst or best performing receiver in a given group. Specify such rules might not be an easy task. Besides, CSI transmissions may cause large overhead/interference in case of groupcast, which may eliminate the (unclear) benefit that the CSI transmissions might bring, or even lead to performance loss,

The usage of CSI feedback for groupcast communication might lead to large specification effort, and the benefit is unclear.

Therefore, we suggest that RAN2 first study the sidelink CSI feedbacks for unicast transmissions, and down-prioritize the study on sidelink CSI feedbacks for groupcast transmissions, at least for this first NR V2X release.

RAN2 first study the sidelink CSI feedbacks for unicast transmissions, and down-prioritize the study on this for groupcast transmissions.

For the CSI report, different types of CSI report transmission over the SL between UEs can be foreseen. The CSI report can be sent periodically by a receiving UE, or sent by the receiving UE when certain events occur (e.g. upon changes of interference/congestion conditions), or even requested by one UE.

The following CSI report alternatives could be considered:

  - Periodic CSI report.

  - Aperiodic CSI report triggered when certain events occur (e.g. change in the CSI value due to changes of interference/congestion conditions).

  - Aperiodic CSI report requested by one UE to one or more other UEs.

From RAN2 perspective, of course there can be impact in the configuration of the CSI, for example whether periodic CSI and/or aperiodic CSI is configured for a given link, and the periodicity of the CSI itself. However, apart for such configuration aspect, it is not clear what could be the RAN2 impacts.

Obviously upon receiving a CSI, a UE it is expected to adjust the transmitting parameters such as the MCS, the power, the resource allocation for robust link adaptation. Such procedures are mainly related to RAN1 discussion and may not directly affect RAN2 specifications.

The need to specify UE behaviour upon CSI report reception, e.g. in terms of resource allocation, MCS/power adjustments, etc., is left to RAN1 discussion.

For mode-1, the above Proposal 2 is still applicable. However, in mode-1 one may argue that the CSI feedback is directly sent from a receiving UE to the gNB, rather than to the transmitting UE. That would save some SL resources, but it has the drawback of not being compatible with those scenarios in which transmitting and receiving UEs are in different coverage status, for example they are in different cells, or one it is in-coverage, and the other out-of-coverage. Therefore, similar to the case of SL HARQ feedbacks for mode-1, it is better if it is the transmitting UE forwarding the CSI received over the SL to the gNB, rather than the receiving UE.

For CSI feedback on mode-1, it is the transmitting UE that sends the CSI feedback to the gNB on PUCCH, upon receiving the CSI feedback from the receiving UE via SL.

Another issue related to the SL CSI feedback report to the gNB is obviously the need to somehow indicate the link to which the CSI feedback refers to, considering that a certain UE might be potentially involved in more than one unicast traffic flow.

Regarding this issue, it seems that coordination with RAN1 is needed, since that might impact the PUCCH format and content.

There should be the possibility for the gNB to identify the unicast traffic flow to which a certain SL CSI feedback refers to.

# Conclusion

In section 2 we made the following observations:

Observation 1	The usage of CSI feedback for groupcast communication might lead to large specification effort, and the benefit is unclear.

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 first study the sidelink CSI feedbacks for unicast transmissions, and down-prioritize the study on this for groupcast transmissions.

Proposal 2	The following CSI report alternatives could be considered:

a.	Periodic CSI report.

b.	Aperiodic CSI report triggered when certain events occur (e.g. change in the CSI value due to changes of interference/congestion conditions).

c.	Aperiodic CSI report requested by one UE to one or more other UEs.

Proposal 3	The need to specify UE behaviour upon CSI report reception, e.g. in terms of resource allocation, MCS/power adjustments, etc., is left to RAN1 discussion.

Proposal 4	For CSI feedback on mode-1, it is the transmitting UE that sends the CSI feedback to the gNB on PUCCH, upon receiving the CSI feedback from the receiving UE via SL.

Proposal 5	There should be the possibility for the gNB to identify the unicast traffic flow to which a certain SL CSI feedback refers to.

# References

RP-181429, Study on NR V2X, 3GPP TSG RAN Meeting #80, La Jolla, USA, June 11th -14th, 2018