3GPP TSG RAN WG2 #105	Tdoc R2-1901672

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	11.2.3

Source:	Ericsson

Title:	Discussions on channel access priority in NR-U

Document for:	Discussion, Decision

# Introduction

For operations in unlicensed spectrum, LBT- may be applied. In LTE LAA, when performing LBT the transmitter applies one of several Channel Access Priority Classes. LBT is (at least in some cases) to be applied in NR-U as well and hence the selection of Channel Access Priority Class would apply also for NR-U.

This topic was already discussed in the NR-U SI phase but no conclusion was reached. In our understanding, although RAN2 needs some inputs from RAN1, RAN2 can assume that LTE LAA can be taken as a baseline at least from user plane perspective.

In this paper we further discuss this topic. In addition, NR-U supports standalone deployment scenario where PRACH and PUCCH need to be fully supported in an NR-U primary cell, which is not the case in LTE LAA. In LTE LAA, it is only a PDCCH order triggered CFRA that can be transmitted in an unlicensed SCell. At the same time, a PUCCH transmission is only allowed in PCell or a PUCCH SCell. Therefore, specific design efforts must be taken to address the channel access for PRACH and PUCCH in NR-U.

# Channel access priority in LTE LAA

Channel Access Priority Class (CAPC) has been introduced in LTE LAA to differentiate the likelihood to acquire access to an unlicensed channel for different type of traffics. Obviously in LAA, such CAPCs only apply to user plane since RRC/PUCCH/RACH cannot be delivered over an unlicensed spectrum. Therefore, RAN2 can assume that at least when it comes to user plane, LAA principles can be reused for NR-U as well.

Channel Access Priority Classes are used to differentiate the likelihood to acquire access to an unlicensed channel for different type of traffics

As specified in [1], the channel access schemes for NR-based access for unlicensed spectrum can be classified into the following categories:

-	Category 1: Immediate transmission after a short switching gap

-	Category 2: LBT without random back-off

-	Category 3: LBT with random back-off with a contention window of fixed size

-	Category 4: LBT with random back-off with a contention window of variable size

For QoS differentiation, a channel access priority based on the service type has been defined. For example, there are four LBT priority classes are defined for differentiation of contention window sizes (CWS) and MCOT between services.

As specified in the 3GPP TS 36.300, a correspondence between CAPC and service QoS class identifier (QCI) is in the Table 5.7.1-1, which is a baseline for the UE to select CAPC for uplink transmissions. In addition, for a UE with data with non-standardized QCIs in the table, the eNB can signal which LBT type (i.e. Type 1 or Type 2 uplink channel access, where Type 1 corresponds to Category 4 for uplink, while Type 2 corresponds to Category 2 for uplink) the UE applies via uplink grant for uplink PUSCH transmission on LAA SCells, except for Autonomous Uplink (AUL) transmissions. Therefore, it is natural to reuse the same rules for dynamic scheduling for NR-U.

As in LTE LAA, for NR-U dynamic scheduling, the gNB signals which Channel Access Priority Class a UE shall apply for an uplink PUSCH transmission via uplink grant carried in DCI signalling.

For Type 1 uplink channel access for AUL, E-UTRAN configures the Channel Access Priority Class for each logical channel and UE shall select the lowest Channel Access Priority Class (i.e, with a higher number in the Table 5.7.1-1 in the 3GPP TS 36.300) of the logical channel(s) with MAC SDU multiplexed into the MAC PDU. The MAC CEs except padding BSR use the highest Channel Access Priority Class (i.e, the lowest number in the Table 5.7.1-1 in 3GPP TS 36.300).

For Type 2 uplink channel access for AUL, the UE may select logical channels corresponding to any Channel Access Priority Class for UL transmission in the subframes signalled by E-UTRAN in common downlink control signalling.

Regarding the UL in NR-U, RAN2 has agreed to study configured grants. For configured grants it is not known to a gNB in advance which traffic the UE will send. Therefore, it is not feasible that the gNB indicates which CAPC the UE shall apply when transmitting using the grant since the gNB simply doesn’t know at the time of configuration of the grant what the UE later will transmit using the grant. The same rule as for AUL in LAA can be applied here. Similarly, as in LAA, MAC CEs except padding BSR use the highest priority Channel Access Priority Class.

As for LAA, for NR-U configured grants, the gNB configures which Channel Access Priority Class the UE shall apply for a logical channel.

As for LAA, for NR-U configured grants, MAC CEs except padding BSR use the highest priority Channel Access Priority Class.

The UE may multiplex different traffic in one transmission and therefore transmit logical channels having different CAPCs associated with them. For this case it is captured in LTE stage-2 specification (i.e., 3GPP TS 36.300) that the UE selects the lowest priority CAPC (i.e., with a higher number in the Table 5.7.1-1 in the 3GPP TS 36.300) among the CAPCs associated with the multiplexed traffic.

As for LAA, for NR-U configured grants, when multiplexing multiple logical channels in a MAC PDU, the UE uses the lowest Channel Access Priority Class (i.e., with a higher number in the Table 5.7.1-1 in the 3GPP TS 36.300) among the classes associated with the multiplexed logical channels.

# CAPC for scheduled transmissions in NR-U

In addition, NR-U supports standalone and DC deployment scenarios where PRACH and PUCCH need to be fully supported in an NR-U cell, which is not the case in LTE LAA. In LTE LAA, neither RA or PUCCH is supported in an SCell.

Regarding RRC signalling, RAN2 assumed at last RAN2 meeting that the highest access priority is assigned.

From TR 38.889 In addition, access priority for control signaling (transmissions over SRBs) over unlicensed carriers should be introduced for stand-alone and DC NR-U. In this case, it is assumed that control signaling will have the highest access priority.

However, there are other signalling such as paging and SIB which have not been discussed yet. In our understanding also for this signalling, similar treatment as SRB is needed.

For control signalling including transmission over SRB, SIB and paging over unlicensed carrier the highest channel access priority class is applied.

However, that are also types of transmission that might need special treatment when it comes to channel access priority. For example, in NR-U the UE will also be able to perform Random Access meaning the UE will perform MSG1 transmissions (preamble, or preamble + data if 2-step random access is used) and transmissions of MSG3 and MSG5. It needs to be defined which CAPC the UE applies for these transmissions.

In an NR-U primary cell, RA and PUCCH transmissions are supported as in an NR licensed cell

As captured in [1], for transmissions within a COT initiated by a gNB, the channel access schemes are described as below

Table 7.2.1.3.1-1: Channel access schemes for initiating a COT by gNB as LBE device

|  | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) | When the DRS duty cycle ≤1/20, and the total duration is up to 1 ms: 25 µs Cat 2 LBT is used (as in LAA) | When DRS duty cycle is > 1/20, or total duration > 1 ms |
| DRS multiplexed with unicast data | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |
| PDCCH and PDSCH | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |

Note: Applicability of an LBT scheme other than Cat 4 LBT for control messages related to initial/random access, mobility, paging, reference signals only, and PDCCH-only transmissions, e.g. "RACH message 4", handover command, GC-PDCCH, or short message paging transmitted either alone or when multiplexed with DRS have been discussed. Further details related to exceptions in this note can be determined when specifications are developed.

Based on above texts, we can make below observations.

In NR-U, DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) applies Category 2 LBT if the DRS duty cycle ≤1/20, and the total duration is up to 1 ms; otherwise, applies Category 4 LBT

If Category 2 LBT conditions are not fulfilled for DRS transmission, Category 4 LBT would be applied instead, however it is not clear yet to choose which channel access priority class for Category 4 LBT. We think it is reasonable to choose highest channel access priority class (i.e., with lowest value as defined in the Table 5.7.1-1 in the 3GPP TS 36.300) here to ensure a reliable delivery of DRS signal, as suggested in [2].

Category 4 LBT with highest channel access priority class can be used for DRS transmissions if Category 2 LBT conditions are not fulfilled.

As captured in [1], For initiation of a COT by the UE, the channel access schemes in Table 7.2.1.3.1-4 are used.

Table 7.2.1.3.1-4: Channel access schemes for initiating a COT by UE as LBE device

|  | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| PUSCH (including at least UL-SCH with user plane data) | N/A except for the cases discussed in Note 2 below | Channel access priority class is selected according to the data |
| SRS-only | N/A | Cat4 with lowest channel access priority class value (as in LTE eLAA) |
| RACH-only | (see Note 2) | Cat4 with lowest channel access priority class value |
| PUCCH-only | (see Note 2) | Cat4 with lowest channel access priority class value |

Note 1: If the COT includes multiple signals/channels with different channel access categories / priority classes, the highest channel access priority class value and highest channel access category among the channel access priority classes and channel access categories corresponding to the multiple signals/channels applies.

Note 2: Applicability of a channel access scheme other than Cat 4 for the following signals / channels have been discussed and details are to be determined when the specifications are developed:

-	UL control information including UCI only on PUSCH, e.g. HARQ-ACK, Scheduling Request, and Channel State Information

-	Random Access

Based on above captured texts in [1], SRS without PUSCH applies Category 4 LBT with lowest channel access priority class value which is the same as in LTE LAA.

In NR-U, SRS without PUSCH applies Category 4 LBT with highest channel access priority class as in LTE LAA.

In addition, from the table, it is also noted that RA-only and PUCCH-only have been recommended to apply Category 4 LBT with lowest channel access priority class value, however the other UL control signalling such as UCI on PUSCH, CSI, PUCCH-SR, RA triggered for different purposes may be further discussed. Given the fact that the total overhead for these UL control signalling is relatively low compared to the data, it is unnecessary to apply CAPC differentiation for them depending on signalling purposes or service QoS requirements. In other words, these UL control signalling shall be treated with the same CAPC, i.e., Category 4 LBT with lowest channel access priority class value.

The UL control signalling (i.e., PUCCH, UCI only on PUSCH, RA) applies Category 4 LBT with highest channel access priority class.

# Conclusion

In section 2 we made the following observations:

Observation 1	Channel Access Priority Classes are used to differentiate the likelihood to acquire access to an unlicensed channel for different type of traffics

Observation 2	In an NR-U primary cell, RA and PUCCH transmissions are supported as in an NR licensed cell

Observation 3	In NR-U, DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) applies Category 2 LBT if the DRS duty cycle ≤1/20, and the total duration is up to 1 ms; otherwise, applies Category 4 LBT

Observation 4	In NR-U, SRS without PUSCH applies Category 4 LBT with highest channel access priority class as in LTE LAA.

Based on the discussion in section 2 we propose the following:

Proposal 1	As in LTE LAA, for NR-U dynamic scheduling, the gNB signals which Channel Access Priority Class a UE shall apply for an uplink PUSCH transmission via uplink grant carried in DCI signalling.

Proposal 2	As for LAA, for NR-U configured grants, the gNB configures which Channel Access Priority Class the UE shall apply for a logical channel.

Proposal 3	As for LAA, for NR-U configured grants, MAC CEs except padding BSR use the highest priority Channel Access Priority Class.

Proposal 4	As for LAA, for NR-U configured grants, when multiplexing multiple logical channels in a MAC PDU, the UE uses the lowest Channel Access Priority Class (i.e., with a higher number in the Table 5.7.1-1 in the 3GPP TS 36.300) among the classes associated with the multiplexed logical channels.

Proposal 5	For control signalling including transmission over SRB, SIB and paging over unlicensed carrier the highest channel access priority class is applied.

Proposal 6	Category 4 LBT with highest channel access priority class can be used for DRS transmissions if Category 2 LBT conditions are not fulfilled.

Proposal 7	The UL control signalling (i.e., PUCCH, UCI only on PUSCH, RA) applies Category 4 LBT with highest channel access priority class.

# References

3GPP TR 38.889, v 16.0.0.

[2] R1-1900998, Channel access mechanisms for NR-U, Ericsson, RAN1#AdHoc 1901, 21st – 25th January, 2019.