3GPP TSG-RAN WG2 #105	 TDoc R2-1901708

# Athens, Greece, 25th Feburary – 1st March 2019

Agenda Item:	11.4.2.1

Source:	Ericsson

Title:	On NR SL unicast bearer establishment

Document for:	Discussion, Decision

# Introduction

In [1] the motivation to evaluate and enhance QoS framework to support V2X services is explained. As proposed in the same paper, it’s suggested that NR Sidelink (SL) can inherit similar QoS framework as in Uu interface, i.e. based on QoS flow and radio bearer. This paper discusses NR SL unicast bearer establishment.

# Discussion

- For the SL unicast case, it is an open issue whether the bearer configuration has to be exchanged/agreed

- before establishing the bearer between transmitter and receiver. This seems to be beneficial at least for unicast

- so that the receiver side can prepare its AS layer to receive a new traffic type. In this way the receiver can also

- determine whether the channel conditions at the receiver side are suitable for the reliable and robust reception

- of this new traffic type, in which case the bearer can be established otherwise it shall be rejected by the

- receiving UE. As part of the bearer establishing phase, also the AS layer configuration can be exchanged. The

- AS layer configuration may contain parameters to decode packets associated to this new bearer, such as RLC

mode, and other physical layer parameters which can be decided in the WI stage. For SL unicast, a SL radio bearer can be established/de-configured taking into account the QoS properties and channel conditions both at the transmitter side and receiver side during the bearer configuration procedure. Therefore, it seems important that the AS-layer configuration also carries information related to the QoS requirement of the associated V2X traffic.

For the SL unicast case, the bearer configuration carrying AS-related information is exchanged between transmitter and receiver before establishing the bearer.

As a result of this bearer configuration setup proposed in Proposal 1, a given bearer can be eventually established or not depending on the content of the AS information. For example, it can happen that the AS information indicates a certain radio configuration that should be associated to this SL traffic, that unfortunately cannot be satisfied by both UEs involved in the SL unicast. Or most important, it can happen that the QoS requirements that should be fulfilled for this traffic type cannot be guaranteed by both UEs, given the experienced SL channel quality at the two sides.

A SL unicast bearer may be established if the QoS requirements for a given traffic can be fulfilled by both UEs, e.g. on the basis of the experienced SL channel conditions by the two UEs.

The AS-layer information that needs to be exchanged between transmitter and receiver when establishing a SL radio bearer indicates at least the QoS requirement of the associated V2X traffic.

For SL unicast, after the bearer establishment, the actual QoS performance can be monitored in terms of reliability, latency, and data rate. Due to the rapidly changing radio condition in V2X scenario, it might be the case that the actual QoS performance varies a lot during the time. If the required QoS is no longer satisfied, the corresponding should be released, however, frequent bearer establishment/release should also be avoided. This issue is also touched in [3], where the feasibility to introduce RLM/RLF procedure in SL is discussed.

RAN2 studies the maintenance and release of an established SL radio bearer, depending on the experienced SL channel conditions.

Another issue that was touched upon during the email discussion [2] is which UE should trigger the establishment of such SL radio bearer. Some companies seem to hint that it will be only one UE that triggers the establishment of the SL radio bearer and providing the AS configuration to the other UE, namely the UE which initiated the SL connection. However, in our understanding, once the SL connection has been established between two UEs, the SL communication should be bidirectional which implies that both UEs involved in a SL communication can trigger the establishment of a new SL bearer, i.e. any UE can initiate a new V2X traffic towards the other UE. Once a SL connection has been established, a SL bearer establishment can be triggered by any of the UEs involved in the same SL communication.

Once a SL connection has been established, a SL bearer establishment can be triggered by any of the UEs involved in the same SL connection

# Conclusions

Based on the discussion in section 2 we propose the following:

Proposal 1	For the SL unicast case, the bearer configuration carrying AS-related information is exchanged between transmitter and receiver before establishing the bearer.

Proposal 2	A SL unicast bearer may be established if the QoS requirements for a given traffic can be fulfilled by both UEs, e.g. on the basis of the experienced SL channel conditions by the two UEs.

Proposal 3	The AS-layer information that needs to be exchanged between transmitter and receiver when establishing a SL radio bearer indicates at least the QoS requirement of the associated V2X traffic.

Proposal 4	RAN2 studies the maintenance and release of an established SL radio bearer, depending on the experienced SL channel conditions.

Proposal 5	Once a SL connection has been established, a SL bearer establishment can be triggered by any of the UEs involved in the same SL connection

# References

- R2-1817924, QoS principles, Ericsson, RAN2#104, Spokane, USA, Nov. 2018

- R2-19xxxxx, Summary of [104#55][NR/V2X] Unicast (OPPO), Athens, Greece, 25 Feb - 1 Mar 2019

- R2- 1901707, On NR sidelink link management for unicast