3GPP TSG-RAN WG2 #105	Tdoc R2-1901709

Athens, Greece, 25 Feb – 1 Mar 2019

Agenda Item:	10.4.1.1

Source:	Ericsson

Title:	Remaining FFS in TS 38.331

Document for:	Discussion, Decision

# 1	Introduction

Here follows a list of remaining FFSs and Editor’s Notes in TS 38.331 v15.4.0, not considered in the Rapporteur’s CR in [1].

# 2	Discussion

Here follows a list of remaining FFSs and Editor’s Notes in TS 38.331 not considered in the Rapporteur’s CR in [1]. Issues assumed to be resolved in the Late Drop are not listed.

Editor's Note: FFS In which cases upper layers are informed that a resume failure occurred upon the reception of RRC Reject.

Editor's Note: It is assumed that NAS triggers the Unified Access Control specified in 5.3.x before initiating this procedure. UE performs this procedure if the access attempt is allowed according to 5.3.14.

rateMatchPatternToAddModList

Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the nested bitmaps (see TS 38.214 [19], clause 5.1.4.1). FFS: RAN1 indicates that there should be a set of patterns per cell and one per BWP => Having both seems unnecessary.

betaOffsets

Selection between and configuration of dynamic and semi-static beta-offset. If the field is absent or released, the UE applies the value 'semiStatic' and the BetaOffsets according to FFS [BetaOffsets and/or clause 9.x.x) (see TS 38.213 [13], clause 9.3).

initialDownlinkBWP

The initial downlink BWP configuration for a SpCell (PCell of MCG or SCG). The network configures the locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain. The locationAndBandwidth is applicable after reception of Msg4 (FFS how to capture this more precisely).

Table 12.1-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |  |
| RRC reconfiguration (scell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC setup | RRCSetup | RRCSetupComplete | 10 |  |
| RRC Release | RRCRelease |  | NA |  |
| RRC re-establishment | RRCReestablishment | RRCReestablishmentComplete | FFS |  |
| RRC resume | RRCResume | RRCResumeComplete | 10 |  |
| RRC resume (scell addition/release) | RRCResume | RRCResumeComplete | 16 |  |
| Initial AS security activation | SecurityModeCommand | SecurityModeComplete/SecurityModeFailure | 5 |  |
| Initial AS security activation + RRC reconfiguration | SecurityModeCommand, RRCReconfiguration | RRCReconfigurationComplete | FFS | The two DL messages are transmitted in the same TTI |
| Other procedures |  |  |  |  |
| UE assistance information |  | UEAssistanceInformation | NA |  |
| UE capability transfer | UECapabilityEnquiry | UECapabilityInformation | FFS |  |
| Counter check | CounterCheck | CounterCheckResponse | 5 |  |

B.1	Protection of RRC messages (informative)

Four missing messages to be added (see Rapporteur CR in [1]), FFS in Comments column.

# Proposal

RAN2 is asked to note the remaining FSSs and Editor’s Notes in TS 38.311.

Companies are asked to provide input the Rapporteur.

# References

[1]	R2-1902275 (was: R2-1900316), Miscellaneous non-controversial corrections, draft CR 0807 to TS 38.331