3GPP TSG-RAN WG2 Meeting #105	R2-1901740

Athens, Greece, 25 Feb - 01 Mar 2019

Agenda item:	11.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	NG-RAN acting as LCS client

WID/SID:	FS_NR_pos - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In this paper we analyse the use cases and requirements studied by SA1 and documented in SA1 specifications to see whether there is need for NG-RAN to act as LCS client.

# 2	Background

There was a RAN2 email discussion [104#35] on use of NG-RAN as LCS client vs support of location management functions in NG-RAN. One of the open issues is whether it is necessary to have NG-RAN acting as LCS client if location management functionality is also locally supported in NG-RAN. There was also the question of whether any specific use cases are more suited if NG-RAN can act as LCS client or whether all use cases can be addressed by support of location management functions in NG-RAN. The email discussion so far is not conclusive on the need for NG-RAN acting as LCS client if location management functions can be supported in NG-RAN.

# 3	Discussion

## 3.1	Use cases and requirements

There were many studies in SA1 about use cases and service requirements for 5G positioning service and these are well documented in the following SA1 specifications, to mention a few: TR 22.804[1], TR 22.872[2], TR 22.886[3]. The normative specification TS 22.261[4] lays out the requirements for positioning in 5GS without pinpointing a specific use case, but which surely was derived based on the various use cases studied and documented over different SA1 specifications. Based on different use cases and various characteristics under which the use case is expected to work, SA1 had specified different positioning service levels and the associated positioning performance requirements for high accuracy positioning. These are shown in the table below copied from TS 22.261[4].

Table 1: Performance requirements for Horizontal and Vertical positioning service levels from Section 7.3.2.2 in TS 22.261[4]

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Availability | Positioning service latency | Coverage, environment of use sand UE velocity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 4 | A | 1 m | 2 m | 99.9 % | 15 ms | NA | NA | Indoor - up to 30 km/h |
| 5 | A | 0.3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h |
| 6 | A | 0.3 m | 2 m | 99.9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h |
| 7 | R | 0.2 m | 0.2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each others (note 3) |  |  |
| NOTE 1: The objective for the vertical positioning requirement is to determine the floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges). NOTE 2: Indoor includes location inside buildings such as offices, hospital, industrial buildings. NOTE 3: 5G positioning nodes are infrastructure equipment deployed in the service area to enhance positioning capabilities (e.g. beacons deployed on the perimeter of a rendezvous area or on the side of a warehouse). |  |  |  |  |  |  |  |  |

Note that positioning service levels 4 and 6 has stringent positioning service latency requirement. This, we believe, is typical of vertical industries related use cases. Some example vertical industries use cases and the positioning accuracy/latency needed to support these use cases are also documented in Annex B of TS 22.261[4] and copied below.

Table 2: Positioning accuracy needs to support example use cases from vertical industries from Annex B in TS 22.261

| Services/ Use cases | Automotive use cases | Transport, logistics, IoT use cases | Health and wellness, smart cities use cases | Media and entertainment |
| --- | --- | --- | --- | --- |
| Description | Expand detectable range beyond on board sensor capability by sharing views or detected objects among traffic participants, coordinate trajectories among vehicles, sharing coarse driving intention, real-time remote operation of vehicles | Real-time sensing, reporting, feedback, control, remote, asset tracking, monitoring; context-aware services, recommendations at shopping mall, airport | Live video feed (4K, 8K, 3D for remote healthcare (consultation, monitoring) and assisted surgery, real-time commands to control medical devices for treatment (e.g., medication, surgery); remote monitoring, surveillance and guidance for citizens and law enforcement officers. | Media production services based on aggregation of various media feeds at servers; real-time peer-to-peer or server-client sharing of data (object information) for collaborative gaming, live streaming at live events |
| Latency | For mid/long-term environment modelling (dynamic high-definition digital map update): Not critical (100 ms end-to-end) For short term environment modelling (sensor sharing): <20 ms end-to-end For cooperation (coordinated control): - <3 ms end-to-end for platooning , - <10 ms end-to-end for cooperative manoeuvres . - <100 ms end-to-end for coarse driving intention For remote vehicle operation: 10-30 ms end-to-end | For massive connectivity for time-critical sensing and feedback: <30 ms end–to-end. For remote drone operation and cooperative farm machinery: 10-30 ms end-to-end Real-time control for discrete automation: ≤1 ms end-to-end | For real-time video/ telepresence/augmented reality for remote healthcare and assisted surgery, for monitoring and guidance (smart cities): 100 ms end-to-end Real-time command and control for remote medication and surgery: 10-100 ms end-to-end For smart grid: - <5 ms end-to-end for transmission/grid backbone, - <50 ms end-to-end for distribution/grid backhaul, Time-critical sensing and feedback for smart cities: 30 ms end-to-end | For live streaming in crowded areas, services for media production, augmented reality for collaborative gaming etc.: 20 ms end–to-end |

Correlating the information from 22.261[4] shown in Table 2 above with positioning requirements documented in various SA1 specifications, some specific use cases falling in to one of the four vertical industries use case categories shown in the Table 2 above with stringent positioning service latency includes:

- Sensor and State Map Sharing & Collective Perception of Environment explained in TR 22.886[3] (positioning latency ranging from 3 ms to 100 ms (see extended sensors requirements)

- Trolley location in factories as in TR 22.872[2] (positioning latency of 20 ms)

- Inbound logistics for manufacturing, requirement: ”Factories of the Future 15.5” in TR 22.804[1] (positioning latency of 10 ms)

- Augmented Reality in Smart Factories, requirement: ”Factories of the Future 10.8” in TR 22.804[1] (positioning latency of 15 ms)

- Traffic monitoring & Control from TR 22.872[2] (positioning latency requirement of 30 ms)

So, clearly there are use cases where end-to-end positioning latency and latency in the RAN domain can be enhanced to address the positioning performance requirements. This fully justifies the need for performing positioning estimation in the RAN for select use cases like the ones listed above, at least for smart factories. It is also interesting to note that the two positioning service levels 4 and 6 do not have “5G positioning service area” coverage requirement. In other words, these apply to smart factory use cases where cellular network coverage is not used. In smart factories the coverage is provided by a private, dedicated 5G network with the factory being responsible for handling identity management, privacy and security aspects.

Observation 1: There are use cases which justify the use of position estimation in RAN and these are primarily for use cases in a private network environment where privacy, security and identity management are all under the control of the private network owner. For example, note that there is no interaction with cellular network in the smart factory use cases.

Observation 2: Position estimation in RAN in a public cellular network without the need for privacy check and authorization for position information can be done if the resulting position information is used for RAN internal purposes like RRM and mobility management, beam management or for network operations like SON, MDT etc.

Observation 3: There are use cases based on investigation of the SA1 specifications 22.804, 22.872 which requires other positioning service levels than service levels 4 and 6 (as defined in TS 22.261) which have less stringent positioning latency requirements. These use cases can be catered to by using the Rel-15 NR positioning architecture where the position estimation can be either UE-based or LMF-based. The LCS client for these use cases can be located as it is currently in Rel-15 NR positioning architecture.

Observation 4: Based on observations 1 through 3, the use case(s) or the need for NG-RAN acting as LCS client is not clear at all as location management in NG-RAN and location management in LMF can cater to all the use cases identified in SA1 specifications so far.

Based on these observations we make the following proposal:

Proposal 1: Location management functions in NG-RAN should be supported at least for use cases involving the use of a private network environment, where privacy, security and identity management are all under the control of the private network owner. What specific location management functions and which positioning methods are to be supported can be discussed during work item phase.

Proposal 2: Location management functions in NG-RAN should be supported in public networks (PLMN) for use of position information for RAN internal purposes like RRM, mobility management, beam management etc or for network operations like SON, MDT etc.

Proposal 3: RAN2 to discuss whether NG-RAN acting as LCS client can be considered as another tool in a tool box of solutions for use of position information for RAN internal purposes or for network operations, but the additional specification development, implementation and test efforts that comes with multiple options to achieve the same goals must also considered in this discussion.

# 4	Conclusion

After analysing the requirements for positioning use cases from SA1 specifications we make the following observations and proposals:

Observation 1: There are use cases which justify the use of position estimation in RAN and these are primarily for use cases in a private network environment where privacy, security and identity management are all under the control of the private network owner. For example, note that there is no interaction with cellular network in the smart factory use cases.

Observation 2: Position estimation in RAN in a public cellular network without the need for privacy check and authorization for position information can be done if the resulting position information is used for RAN internal purposes like RRM and mobility management, beam management or for network operations like SON, MDT etc.

Observation 3: There are use cases based on investigation of the SA1 specifications 22.804, 22.872 which requires other positioning service levels than service levels 4 and 6 (as defined in TS 22.261) which have less stringent positioning latency requirements. These use cases can be catered to by using the Rel-15 NR positioning architecture where the position estimation can be either UE-based or LMF-based. The LCS client for these use cases can be located as it is currently in Rel-15 NR positioning architecture.

Observation 4: Based on observations 1 through 3, the use case(s) or the need for NG-RAN acting as LCS client is not clear at all as location management in NG-RAN and location management in LMF can cater to all the use cases identified in SA1 specifications so far. However, it can be discussed whether NG-RAN acting as LCS client can be considered as another tool in a tool box of solutions but the additional specification development, implementation and test efforts that comes with multiple options to achieve the same goals must also considered in this discussion.

Proposal 1: Location management functions in NG-RAN should be supported for use cases involving the use of a private network environment, where privacy, security and identity management are all under the control of the private network owner. What specific location management functions and which positioning methods are to be supported can be discussed during work item phase.

Proposal 2: Location management functions in NG-RAN should be supported in public networks (PLMN) for use of position information for RAN internal purposes like RRM, mobility management, beam management etc or for network operations like SON, MDT etc.

Proposal 3: RAN2 to discuss whether NG-RAN acting as LCS client can be considered as another tool in a tool box of solutions for use of position information for RAN internal purposes or for network operations, but the additional specification development, implementation and test efforts that comes with multiple options to achieve the same goals must also considered in this discussion.

# References

- TR 22.804, Study on Communication for Automation in Vertical Domains

- TR 22.872, Study on positioning use cases

- TR 22.886, Study on enhancement of 3GPP Support for 5G V2X Services

- TS 22.261, Service requirements for the 5G system