3GPP TSG-RAN WG2 #105			     R2-1901762

Athens, Greece, 25th February – 1st March 2019

Agenda Item	: 11.7.2.2 (FS_NR_IIOT)

Source	: LG Electronics Inc.

Title	: Measurement gap enhancement for TSN traffic

Document for	: Discussion and Decision

1.	Introduction

In IIoT study item to investigate enhancements to URLLC, the objective related to Time Sensitive Networking enhancements are as follows:

- Time Sensitive Networking related enhancements:

  - Accurate reference timing: Delivery & related process (e.g. SIB delivery or RRC delivery to UEs, Multiple Transmission points) (RAN2/RAN3/RAN1)

  - Enhancements (e.g. for scheduling) to satisfy QoS for wireless Ethernet when using TSN traffic patterns as specified in TR 22.804 (RAN2/RAN1). Note: RAN2 to start the work, RAN1 to take action based on RAN2 progress.

  - Ethernet header compression (RAN2):

    - Analysis of the benefits and the scenario (e.g. what are the formats and size of Ethernet frame to be considered, are VLAN fields included, protocol termination etc.).

    - Definition of the requirements for a new header compression.

  - Performance evaluation of TSN requirements as captured in TR 22.804 clause 8.1 (RAN2/RAN1/RAN3)

NOTE: This task is related to TSN specific requirements, which are not evaluated as part of “Study on physical layer enhancements for NR ultra-reliable and low latency case”. It is not intended to discuss/agree additional simulation assumptions for this case.

Note: RAN2 to start the work, RAN1 to take action based on RAN2 progress

In this contribution, we’d like to address the issues that QoS requirements for URLLC traffic may not met due to measurement gap, and suggest to study the measurement gap related enhancements for TSN traffic in order to satisfy the QoS requirement as well as guarantee the UE coverage.

2.	Discussion

## 5.14	Handling of measurement gaps

During a measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]:

1>	not perform the transmission of HARQ feedback, SR, and CSI;

1>	not report SRS;

1>	not transmit on UL-SCH except for Msg3 as specified in subclause 5.4.2.2;

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer is running:

2>	monitor the PDCCH as specified in subclauses 5.1.4 and 5.1.5.

1>	else:

2>	not monitor the PDCCH.

In order to guarantee the coverage of a UE, the network may configure an RRC_CONNECTED UE to perform measurements and report them in accordance with the measurement configuration. According to the TS38.300, whether a measurement is non-gap-assisted or gap-assisted depends on the capability of the UE, the active bandwidth part (BWP) of the UE and the current operating frequency. In other words, with the measurement configuration, if a UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, the network configures the measurement gaps for the UE based on the UE capability and current operating/target frequency. In gap-assisted scenarios, the UE cannot carry out such measurements without measurement gaps. Based on the current MAC specification of TS38.321, if the network configures the measurement gaps for a UE, the UE shall not perform the data transmission/reception during a measurement gap except for RACH procedure.

Observation 1. Based on the current spec, if the network configures the measurement gaps for a UE, the UE shall not perform the data transmission/reception during a measurement gap except for RACH procedure.

In NR, the length of measurement gap could be from 1.5ms to 6ms depending on the frequency of the serving and target cell, as defined in Table 9.1.2-1 and 9.1.2-2 of TS 38.133, whereas for URLLC traffic, the target for user plane latency is 0.5ms for UL, and 0.5ms for DL. In addition, according to TR 22.804 [3], TSN traffic needs to be transmitted within bounds, e.g., 1ms. Even if the measurement gap is set to 1.5ms in case of per frequency range (per-FR) measurement, the QoS requirements for TSN traffic may not be met. Especially, if TSN traffic has a sporadic pattern, the network cannot predict when the data will be generated. In this case, the measurement gap may be an obstacle to the successful transmission of TSN traffic. Therefore, we suggest that RAN2 further discuss the measurement gap related enhancement for TSN traffic in WI phase, in order to satisfy the QoS requirement of TSN traffic as well as guarantee the UE coverage. For example, a UE can skip the measurement gap or network can indicate to skip the measurement gap when TSN traffic collides with measurement gap.

Observation 2. Even if the measurement gap is set to 1.5ms, the QoS requirements for TSN traffic may not be met for a UE in gap-assisted scenarios.

Proposal. Measurement gap related enhancement for TSN traffic should be discussed in WI phase.

Corresponding Text Proposal to TR 38.825 is provided in R2-19xxxxx.

3.	Conclusion

In this contribution, we’d like to address the issues that the QoS requirements for TSN traffic may not met due to measurement gap, and our observations and proposal are as follows:

Observation 1. If the network configures the measurement gaps for a UE, the UE shall not perform the data transmission/reception during a measurement gap.

Observation 2. Even if the measurement gap is set to 1.5ms, the QoS requirements for TSN traffic may not be met for a UE in gap-assisted scenarios.

Proposal. Measurement gap related enhancement for TSN traffic should be discussed in WI phase.

Corresponding Text Proposal to TR 38.825 is provided in R2-19xxxxx.