3GPP TSG-RAN WG2 Meeting #105	R2-1901884

Athens, Greece, 25th February – 1st March 2019

Agenda Item	: 12.2.4

Source	: LG Electronics Inc.

Title	: Further discussion on D-PUR

Document for	: Discussion and Decision

1.	Introduction

During the email discussion [104#43], RAN2 discussed D-PUR solutions.

In this contribution, we would like to discuss some other issues regarding unexpected UL transmission and access control.

2.	Discussion

D-PUR will be designed for the use cases to provide regular reporting for long period. If unexpected data/signalling transmission from the UE is required, the UE may use the preconfigured resource which is supposed to be used for regular reporting. Or, the UE may need to transit to RRC_CONNECTED to transmit the unexpected transmission.

Observation 1. Unexpected UL transmission could be generated by the UE while the UE transmits regular transmission using D-PUR.

For the unexpected UL transmission, the UE may use early data transmission if EDT conditions are met. This is beneficial because the UE does not need to transit to RRC_CONNECTED. If the conditions are not met, the UE should request legacy RRC connection setup or resume.

Proposal 1. For unexpected UL transmission, the UE may initiate EDT.

In some cases, the UE data reporting may not be tightly regular. Then, the UE may use preconfigured resources in advance for incoming UL transmission. If the UE can expect upcoming data reporting request, the UE can send Buffer Status Report (BSR) to request UL grant for the expected data reporting.

Proposal 2. For unexpected UL transmission, the UE may use preconfigured resource and send BSR for the expected UL transmission.

As legacy data transmission is performed in CONNECTED mode, the UE first performs access control and transmits data if connection is successfully established. However, the network cannot control data transmission is IDLE mode. Therefore, we suggest to discuss whether access control would be necessary for IDLE mode data transmission.

Proposal 3. RAN2 to discuss whether access control would be beneficial in IDLE mode data transmission.

3.	Conclusion

In this contribution, we would like to discuss some other issues regarding unexpected UL transmission and access control.

Observation 1. There could be unexpected UL transmission while the UE transmits regular transmission using D-PUR.

Proposal 1. For unexpected UL transmission, the UE may initiate EDT.

Proposal 2. For unexpected UL transmission, the UE may use preconfigured resource and send BSR for the expected UL transmission.

Proposal 3. RAN2 to discuss whether access control would be beneficial in IDLE mode data transmission.