3GPP TSG-RAN WG2 Meeting #105	                                                            R2-1901977

Athens, Greece, 25th February–1st March 2019

Agenda item:	11.7.4

Source:	CMCC

Title:	Resource-efficient of multi-leg PDCP duplication

Document for:	Discussion

Introduction

The item in the work plan for NR industrial IIoT SI need to be discussed in RAN2 as below:

- L2/L3 enhancements:

a) Data duplication and multi-connectivity enhancements, including (RAN2/RAN3):

1) Resource efficient PDCP duplication e.g. coordination between the nodes for PDCP duplication activation and resource efficiency insurance, avoiding unnecessary duplicate transmissions etc.

2) PDCP duplication with more than 2 copies leveraging (combination of) DC and CA, whereupon data transmission takes places from most two nodes: assessment of the gains, and if beneficial, study the associated solutions.

3) Potential impacts of higher layer multi-connectivity as studied by SA2.

In this contribution, we discuss the resource-efficient of multi-leg PDCP duplication using DC and CA architecture in IIoT.

Discussion

According to the clarification of NR IIoT R16 scope, the resource efficiency of PDCP duplication is adopted as one of the key issues needed to be solved; however, this topic has not been well discussed during last few meetings. In our opinion, the resource efficiency is partially correlated to whether more than 2 RLC legs could be configured for a particular PDCP entity. As indicated in TR22.804, the service availability for some use cases shall achieve as high as 99.999999%, which is quite a stringent requirement when taking latency into consideration as well.

Initially, in our understanding that if the above-mentioned two legs are configured with RLC UM that reduces the latency for transmission to the peer PDCP entity, then the super low error rate should be totally dependent on HARQ at the MAC layer as well as enhanced mechanisms at the PHY layer, which may be hard target to achieve with a stringent latency requirement for each packet. However, from RAN1 LS [1] in RAN1 meeting #AH-1901, the simulations were executed and the conclusion is listed below:

On reliability analysis using single UE link level evaluations, RAN1 makes the following conclusions: - For the cases where the one-way latency target can be achieved, it was observed that the reliability target of 1e-4 to 1e-6 can be achieved with Rel-15 NR for the 5%-ile SINR geometry (e.g. cell-edge UE) in use case I based on the agreed methodology and assumptions from RAN1#95 (without PDCP duplication). It is RAN1 conclusion that PDCP duplication is not always available/applicable.

The simulation analysis in RAN1 indicates that in some use cases, e.g. use case I, most of UE in cell, even the UE at the cell-edge, can achieve the reliability requirement of TSN network when no PDCP duplication is applied in cases, where the one way latency is satisfied.  Based on the conclusion from the RAN1 simulation, the reliability requirement of TSN network could be guaranteed by other methods, i.e. low MCS level used in RAN1 simulation assumption.  Therefore the PDCP duplication is not always needed and the increment of the number of PDCP copies leads the increase of load and redundancy in network.  Since the reliability requirement of TSN can be guaranteed without PDCP duplication, it is not necessary to increase the number of actual transmitted PDCP copies simultaneously Rel-16.

Observation 1: Since the reliability requirement of TSN is guaranteed without PDCP duplication based on RAN1 simulation result, it is not necessary to increase the number of transmitted PDCP copies simultaneously in Rel-16.

Proposal 1: It is proposed not to increase the number of activated legs in Rel-16.

In CA or DC architecture, the condition of links could be varied on different legs. If more than 2 legs are configured for data transmission in CA or DC architecture, it is beneficial to select the suitable RLC legs with better radio condition for PDCP copies. The activation and deactivation of PDCP duplication for DRB is indicated by RRC or MAC CE. The MAC CE, including DRB ID and the selective RLC bearer ID, could be utilized as flexible configuration to inform the UE on the RLC bearer selection

Proposal 2: The MAC CE could be utilized to inform the UE on the RLC bearer selection.

There are two options for this purposes:

- The PDCP duplication activation/deactivation command MAC CE from gNB;

- The PDCP duplication activation/deactivation request/notify MAC CE from UE;

Option 1 is the basic method introduced in NR for duplication activation/activation; however, the current spec TS38.321 only supports the activation/deactivation in terms of DRB. Moreover, it should be noted that the option 2 listed above is not included in the current specification, but we think it is necessary to allow such implementation. The reason is stated as follows:

Although the processing of downlink MAC CE is much faster than RRC configuration, it might still not meet the requirement of the URLLC data transmission. Further Enhancements could be made. For example, the gNB configures UE with conditions of PDCP activation/deactivation. After such condition is met, the UE could send an uplink MAC CE to the gNB to trigger the UL PDCP activation/deactivation. Then UE could trigger the UL data duplication itself, without receiving downlink data duplication MAC CE from the gNB. So, RAN2 is kindly asked to discuss mechanism enhancements to enable further faster PDCP activation/deactivation.

Proposal 3: RAN2 is kindly asked to discuss mechanism enhancement, such as allowing UE to send a MAC CE to the gNB to trigger the UL PDCP activation/deactivation and then trigger the UL data duplication itself, to enable further faster PDCP activation/deactivation.

Conclusions

In this paper, we have discussed the issues on resource efficiency of PDCP duplication in IIoT. The observations and proposal are listed below:

Observation 1: Since the reliability requirement of TSN is guaranteed without PDCP duplication based on RAN1 simulation result, it is not necessary to increase the number of transmitted PDCP copies simultaneously in Rel-16.

Proposal 1: It is proposed not to increase the number of activated legs in Rel-16.

Proposal 2: The MAC CE could be utilized to inform the UE on the RLC bearer selection.

# Proposal 3: RAN2 is kindly asked to discuss mechanism enhancement, such as allowing UE to send a MAC CE to the gNB to trigger the UL PDCP activation/deactivation and then trigger the UL data duplication itself, to enable further faster PDCP activation/deactivation.

References

[1] RP-182068 Revised SID: Study on NR Industrial Internet of Things (IoT)