3GPP TSG-RAN WG2 Meeting #105			 R2-1902023

Athens, Greece, 25th February – 1st March 2019 			resubmission of R2-1818059

Agenda Item:	11.1.2 (NR_IAB-Core)

Source: 	LG Electronics Inc.

Title:         	Consideration on uplink data congestion handling

Document for: 	Discussion and Decision

Introduction

As shown in the below objective in IAB WI description, if UL flow control mechanism is needed, RAN2 may find a solution to support UL flow control mechanism.

- Specification of a flow control mechanism (for DL and, if necessary, for UL) to handle congestion.

However, the current transmission/scheduling mechanisms can fully control the uplink data rate to an IAB node and it is sufficient to control uplink data congestion. This contribution analyses uplink data congestion and the current transmission/scheduling mechanisms.

Discussion

Basically, the ingress data rate from UEs or child IAB nodes can be fully controlled by an IAB node because an intermediate IAB node acts as a gNB-DU to child IAB nodes and UEs. This means that the exact ingress data rate from UEs or child IAB nodes can be determined by the intermediate IAB node. However, the intermediate IAB node cannot estimate exact the egress data rate to the parent IAB node because even if the intermediate IAB node transmits a BSR to the parent IAB node, only the parent IAB node can determine how much UL grant would be allocated based on the received BSR.

Observation 1. On the uplink, the intermediate IAB node can determine exact ingress data rate from child IAB nodes or UE, but cannot determine the egress data rate to the parent IAB node.

With observation 1 above, uplink data congestion can occur only when the ingress data rate from UEs or child IAB nodes is larger than the egress data rate to the parent IAB node. This means that the intermediate IAB node assigns larger amount of UL grants to child IAB nodes than the amount of received UL grant from the parent IAB node continuously. Eventually packets can be discarded only when the intermediate IAB node assigns larger UL grant to the child IAB node than the remaining available buffer space of an uplink buffer, i.e., overflow in an uplink buffer.

However, it doesn’t make sense that even though buffer overflow in the uplink buffer is expected, the intermediate IAB node allocates much larger amount of UL grants to child IAB nodes than the remaining available buffer space in the uplink buffer. With this understanding, normally, no packet discard due to uplink data congestion is predicted because the intermediate IAB node would not allocate larger amount of UL grants to child IAB node or UE than the remaining available buffer space in the uplink buffer.

Observation 2. Packet discard due to uplink data congestion may not happen because the intermediate IAB node does not allocate too much UL grants, which may cause uplink buffer overflow, to child IAB nodes or UEs.

Considering the above observations, the intermediate IAB node would allocate UL grants to child IAB node as much as it can and adjust ingress data rate from child IAB node or UEs by itself to avoid uplink buffer overflow and data congestion. If an intermediate IAB node is congested on uplink, the intermediate IAB node will reduce the amount of allocated UL grants to child IAB nodes to prevent buffer overflow and finally all descendent IAB node from the intermediate IAB node would do same things, i.e., reducing the amount of allocated UL grants to their child IAB nodes or UEs. Furthermore, although new additional indication is introduced as in [2], the intermediate IAB node still cannot estimate how much UL grants can be assigned from the parent IAB node and only can allocate UL grants to child IAB nodes based on the remaining available buffer space without buffer overflow. Thus, even with the additional mechanism to indicate uplink data congestion, total amount of remaining available buffer space in the uplink buffer is not changed and the allocated UL grants to child IAB nodes may not be different regardless of whether new additional indication is introduced or not.

Proposal. RAN2 confirms that no additional flow control mechanism for uplink data congestion is needed in Rel-16 IAB.

Note that: of course, uplink data congestion without buffer overflow may not be resolved for a long time. However, it would be backhaul RLF and needs something else, such as re-routing, BH RLF handling or other QoS management mechanism, not the additional uplink flow control mechanism.

Proposal

In this contribution, we discussed uplink data congestion and found below observations and proposal:

Observation 1. On the uplink, the intermediate IAB node can determine exact ingress data rate from child IAB nodes or UE, but cannot determine the egress data rate to the parent IAB node.

Observation 2. Packet discard due to uplink data congestion may not happen because the intermediate IAB node does not allocate too much UL grants, which may cause uplink buffer overflow, to child IAB nodes or UEs.

Proposal. RAN2 confirms that no additional flow control mechanism for uplink data congestion is needed in Rel-16 IAB.

- References

[1] TR 38.874v1.0.0 Study on Integrated Access and Backhaul

[2] R2-1812249	Uplink Flow Control and congestion control		Intel Corporation