3GPP TSG-RAN WG2 Meeting #105 	                              R2-1902173

Athens, Greece, 25th Feb– 1st Mar 2019

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on SSB type in measurementTimingConfiguration

Agenda item:		10.4.1.9

Document for: 	Discussion and Decision

- Introduction

During RAN2 #104 meeting, CR[1] was agreed to introduce PCI information in measurementTimingConfiguration INM for wideband CC scenario. Meanwhile, RAN3 sent a new LS[2] to ask RAN2 add “CD-SSB” indication in measurementTimingConfiguration. In this contribution, we discussed this issue from RAN2 perspective and provided our views.

- Discussion

After RAN3#102 meeting, RAN3 agreed to send a new LS [2] to RAN2, to ask RAN2 to add “CD-SSB” indication in MeasurementTimingConfiguration inter-node message, the content of the LS is copied as below:

1. Overall Description: RAN3 discussed the need of exchanging SSB type information over Xn and observed that: - It would be helpful for eNB or NG-RAN node to receive information that allow to distinguish between cell defining SSBs and non cell defining SSBs from neighbouring NG-RAN nodes. RAN3 believes that a suitable way to convey SSB type information could be to identify the CD-SSB within the SSB information carried in the MeasurementTimingConfiguration inter-node RRC message. 2. Actions: To RAN2 group. ACTION: RAN3 kindly requests RAN2 to take the above into account and update their specifications if needed.

According to RAN3’s discussion, the main intention is to differentiate whether a neighbour cell can be treated as PCell (e.g. for handover candidates, and cell re-selection candidates). Then receiving node can configure measurements on these frequencies in both system information (e.g. for Idle/Inactive UEs) and MeasConfig (i.e. for Connected UEs).

Observation1: RAN3 wants to add CD-SSB indication to differentiate the SSB which can be measured for cell re-selection and handover purpose.

However, as discussed at length in the past, a CD-SSB is not necessarily associated to a PCell. The SSB associated to a PCell should fulfill the following characteristics:

- SSB is associated with RMSI (CD-SSB)

- SSB is on sync raster (can be determined by comparing the ARFCN of SSB with the corresponding GSCN values defined in RAN4);

- The associated RMSI is broadcasting a TAC

The second characteristic can be determined by the network by comparing the ARFCN value, but the last one cannot be determined by the network, because the TAC information is mandatory present in X2/Xn AP even for cells not broadcasting SIB1. Then it is not enough to use “CD-SSB” indication to identify PCell capable SSBs.

Observation2: CD-SSB information is not enough to identify PCell capable SSBs, because a CD-SSB may not broadcasting TAC information in RMSI.

Proposal 1: 	Inform RAN3 in the reply LS that “CD-SSB” SSB type is not sufficient to identify PCell capable SSBs for cell re-selection/handover measurement purpose.

In addition, for wideband scenario, as we agreed in RAN2#104, PCI information can be indicated to each SSB included in measurementTimingConfiguration. Per the common understanding in RAN3, all PCell and PSCell capable cells should be transmitted as an individual cell over X2/Xn interface, and the sending node can indicate PSCell capable information by setting the “EN-DC support” indicator to “true” in RAN3.

Observation3: In RAN3, all PCell and PSCell capable cells will be transmitted as individual cells over X2/Xn interface.

An example is shown in following figure:

Figure1 An example of X2/Xn transmission for wideband cell

In the example, both SSB1/2/4 will be delivered as individual cells (e.g. with NCGI) over X2/Xn interface, however, the issue is that in this case, each cell/NCGI transferred in X2/Xn interface may indicate multiple SSBs in measurementTimingConfiguration, but it is not clear which one is associated to the concerned cell in X2/Xn AP(e.g. have same PCI and NCGI). Then it is hard for network to decide which SSB frequency should be configured in SIBs for cell-reselection purpose. However, we think this can be easily solved by putting the “cell associated SSB” in first place of the list (as we shown in figure1).

Observation4: Which SSB in measurementTimingConfiguration is associated to the concerned cell (e.g. PCI/NCGI) in X2/Xn is unclear in current spec, but it is natural to indicate the cell associated SSB in the first place of list.

For above example and solution, if network wants to trigger cell-reselection/handover to NCGI=1, network is aware that SSB measurement should be configured on SSB1(freq1); similarly, for NCGI=4, it is clear measurements should be performed on SSB4(freq4), there is no need to add explicit SSB-Type indication in measurementTimingConfiguration.

While for “PCell capable” indication, since SSB1/4 and SSB2 are separately transmitted as individual cells over X2/Xn interface, the PCell capable indicator should be directly linked to the NCGI, not SSB in measurementTimingConfiguration. For example, NCGI=1 and NCGI=4 can be marked as “PCell supported” in X2/Xn AP, and NCGI=2 is marked as “EN-DC supported”(already supported).

Observation5:  PCell capable indicator should link to the NCGI instead of SSB.

Therefore, based on above observations, we think the SSB-Type in measurementTimingConfiguration is not needed, there is no ambiguity of the cell associated SSB based on network implementation. On the other hand, if RAN3 wants to determine whether a cell is PCell capable or not, in our understanding, it can be explicitly indicated in X2/Xn AP as what they have on “EN-DC supported” indicator.

Proposal 2: 	Do not add SSB-Type in measurementTimingConfiguration, the first SSB listed in measurementTimingConfiguration is the one associated to the concerned cell in X2/Xn AP.

Proposal 3: 	Inform RAN3 in reply LS about the decision in P2, and RAN2 understands that whether cell is PCell capable (e.g. for cell-reselection/handover purpose) can be indicated in X2/Xn interface.

- Conclusion and proposals

RAN2 is kindly asked to discuss and adopt the following proposal:

Observation1: RAN3 wants to add CD-SSB indication to differentiate the SSB which can be measured for cell re-selection and handover purpose.

Observation2: CD-SSB information is not enough to identify PCell capable SSBs, because a CD-SSB may not broadcasting TAC information in RMSI.

Observation3: In RAN3, all PCell and PSCell capable cells will be transmitted as individual cells over X2/Xn interface.

Observation4: Which SSB in measurementTimingConfiguration is associated to the concerned cell (e.g. PCI/NCGI) in X2/Xn is unclear in current spec, but it is natural to indicate the cell associated SSB in the first place of list.

Observation5: PCell capable indicator should link to the NCGI instead of SSB.

Proposal 1: 	Inform RAN3 in the reply LS that “CD-SSB” SSB type is not sufficient to identify PCell capable SSBs for cell re-selection/handover measurement purpose.

Proposal 2: 	Do not add SSB-Type in measurementTimingConfiguration, the first SSB listed in measurementTimingConfiguration is the one associated to the concerned cell in X2/Xn AP.

Proposal 3: 	Inform RAN3 in reply LS about the decision in P2, and RAN2 understands that whether cell is PCell capable (e.g. for cell-reselection/handover purpose) can be indicated in X2/Xn interface.

- Reference

- R2-1819060	Addition of PCI in MeasTiming	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.331	15.3.0	0594	1	F	NR_newRAT-Core

- R2-1900037(R3-187274)	  LS on adding SSB type in the MTC inter-node message  Rel-15  From: RAN3  To: RAN2