3GPP TSG-RAN WG2 Meeting #105			 R2-1902192

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.1.3 (NR_IAB_Core)

Source: 	LG Electronics Inc.

Title:         	Transport of Inter-IAB node Control Signaling

Document for: 	Discussion and Decision

Introduction

In the last RAN2 meeting, it was agreed that, upon detecting BH link failure, the node sends BH RLF notification to its child node. It is FFS how to transport BH RLF to the child node. In fact, inter-IAB node control signalling is not clear. This contribution discuss how to support inter-node control signaling, mainly focusing on BH RLF transport.

Discussion

It is expected that BH RLF notification procedure should be initially triggered by RRC, since the radio link monitoring and declaration of RLF belong to conventional RRC functionalities. In addition, upon reception of the BH RLF notification, the receiving node initiates some actions (FFS), such as re-establishment for connection recovery. This observation may lead to the straightforward idea that the BH RLF could be delivered to the child node via RRC message. However, note that an IAB node and its child node do not have peer RRC entities, and therefore they do not establish RRC connection. (Instead, the IAB node relays the MT RRC connection for its child node towards donor node, and the same IAB node itself has a MT RRC connection also towards the node). For this reason, they cannot exchange RRC messages as a normal UE and NR RAN. Instead, they may be able to exchange RRC messages by encapsulating the RRC messages in the inter-node control signalling.

Currently inter-node control signalling mechanism needs further investigation. One exemplary architectural option for inter-node control signalling would be to introduce Adapt Control PDU, and the inter-node control signalling is encapsulated in the Adapt Control PDU. The Fig.1 illustrates the protocol stacks and the signalling flow. From the child node side, the Adapt needs to identify the received Control PDU type, and if the type indicates inter-node RRC messages, the Adapt needs to route the Adapt SDU to its MT RRC entity.

Figure1. Inter-node RRC signalling via Adapt

Proposal 1: Introduce inter-node RRC signaling that is encapsulated as Adapt Control PDU

Proposal 2: BH RLF notification is delivered inter-node RRC signaling

It is now unclear how security is supported for the inter-node (RRC) control signaling. If the above proposals are acceptable, RAN2 can ask SA3 and RAN3 for their view on the proposed inter-node RRC signaling.

Proposal 3: Send an LS to SA3 and RAN3 to ask their view on the proposed inter-node RRC signaling.

Proposal

This contribution discuss how to support inter-node control signaling, mainly focusing on BH RLF transport, and gives the following proposals.

Proposal 1: Introduce inter-node RRC signaling that is encapsulated as Adapt Control PDU

Proposal 2: BH RLF notification is delivered inter-node RRC signaling

Proposal 3: Send an LS to SA3 and RAN3 to ask their view on the proposed inter-node RRC signaling.

- References

[1] TR 38.874v16.0.0 Study on Integrated Access and Backhaul