3GPP TSG-RAN WG2 #105			        R2-1902193

Athens, Greece, 25th February – 1st March 2019

Agenda Item	: 11.10.3	 (LTE_NR_DC_CA_enh-Core)

Source	: LG Electronics Inc.

Title	: Fast setup MR-DC and CA with IDLE and INACTIVE modes

Document for	: Discussion and Decision

1.	Introduction

According to the [1], the objective of this MR-DC and CA enhancement work item is considered as following topics:

- Support of asynchronous and synchronous NR-NR Dual Connectivity [RAN1, RAN2, RAN4] - UE power control [RAN1] - RRC signalling to support of enhanced NR-NR DC [RAN2] - Core requirements to support enhanced NR-NR DC [RAN4] Note: Synchronous DC enhancements in this WID considers only cases not covered in Rel-15 exception sheet for NR WI NR_newRAT-Core. - Early Measurement reporting: Early and fast reporting of measurements information availability from neighbor and serving cells to reduce delay setting up MR-DC and/or CA. [RAN2, RAN4] - This objective applies to MR-DC, NR-NR DC and CA - The objective should consider measurements in IDLE, INACTIVE mode and CONNECTED mode - The impacts on UE power consumption should be minimized - The LTE Rel-15 euCA work should be utilized, when applicable - Efficient and low latency serving cell configuration/activation/setup: Minimizing signalling overhead and latency needed for initial cell setup, additional cell setup and additional cell activation for data transmission. [RAN2, RAN1, RAN4, RAN3] - This objective applies to MR-DC, NR-NR DC and CA - The objective should consider enhancements when starting from IDLE, INACTIVE mode and CONNECTED mode - Fast recovery: Support fast recovery of MCG link e.g. by utilizing the SCG link and split SRBs for recovery during MCG failure while operating under MR-DC. [RAN2, RAN3] - This objective applies to MR-DC and NR-NR DC - Cross-carrier scheduling with different numerologies on the scheduling and scheduled carriers [RAN1, RAN2, RAN4] - This objective applies to CA only. - Target completion by RAN#84.

2.	Discussion

In order to perform fast setup MR-DC and CA, we would like to investigate fast setup MR-DC and CA mechanism starting from RRC_IDLE or RRC_INACTIVE state, respectively.

2.1 RRC_IDLE

According to the LTE euCA, early measurement reporting procedure for CA is proposed. In this method, network configures measurement configuration (i.e. frequency, cells) while a UE performs measurement in RRC_IDLE. Then a UE reports an indication (i.e. idleMeasAvailable) via MSG5 during RRC connection setup procedure only when available reporting cells are found. After security is activated, the UE transmits whole measurement results via UEInformationResponse message. At this point, we can derive some limitations adopting to fast MR-DC setup mechanism. The first limitation is that after successful security is activated, the UE can convey measurement reports upon receiving the UEInformationRequest message.

If we follow this principle in WID, obviously there is no gain compared to the legacy early measurement reporting procedure from a perspective of latency reduction. The second limitation is that there is no exist any AS context (e.g., SCG or SCell configuration) while UE is in RRC_IDLE. Therefore, from network perspective, it is always required to receive measurement results from UE in order to configure MR-DC and NR-CA. Hence, we needs to consider how to reduce configuration of NR-DC and CA with early measurement reporting procedure [2].

Observation 1. If a UE reports idle measurement report after security activation, there is no gain considering latency reduction.

Proposal 1. RAN2 considers to enhance legacy early measurement reporting procedure.

2.2 RRC_INACTIVE aspects

Firstly, let us consider mentioned security issue mentioned in RRC_IDLE section. According to the current NR specification, AS security of the UE is activated upon receiving RRCResume message from network. Hence, there is no security issue while a UE to perform uplink transmission from the MSG5. In addition, when a UE received RRCRelease message, if the suspendcofig is included, the UE is applied the configuration in RRC_INACTIVE. Compared to the RRC_IDLE state, an AS context is stored in a UE while staying in RRC_INACTIVE and its associated network.

If network considers to configure MR-DC configuration upon receiving measurement results from the UE, additional latency for sending i.e., UEInformationResponse, measurement report should not be avoided. Moreover, it seems not to achieve the objective of this WID which endeavours to reduce latency. Therefore, it is necessary to apply the stored suspended configuration of the UE and network to provide fast MR-DC and NR CA setup [3].

Observation 2. If a UE reports measurement results, additional latency required compared to use stored Inactive configuration.

Proposal 2. RAN2 needs to consider efficient way to reduce latency of MR-DC configuration with stored Inactive configuration

3.	Conclusion

In this contribution, we address that RAN2 needs to enhance the legacy early measurement reporting procedure for MR-DC/CA and hence our observations and proposals are as follows.

Observation 1. If a UE reports idle measurement report after security activation, there is no gain considering latency reduction.

Proposal 1. RAN2 considers to enhance legacy early measurement reporting procedure.

Observation 2. If a UE reports measurement results, additional latency required compared to use stored Inactive configuration.

Proposal 2. RAN2 needs to consider efficient way to reduce latency of MR-DC configuration with stored Inactive configuration

References

[1] RP-182076, WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements.

[2] R2-1902014, Fast setup MR-DC and NR CA with early measurement reporting, LGE

[3] R2-1902175, Fast cell setup for MR-DC and CA in RRC inactive, LGE