3GPP TSG-RAN WG2#105 meeting	Tdoc R2-1902218

# Athens, Greece, 25th February- 1st March 2019

Agenda Item:		11.10.3

Souce:	Samsung

Title:	Non-connected measurements, defining initial baseline

Document for:		Discussion and decision

# Introduction

This contribution aims to define a baseline for the reporting of measurements performed in idle/ inactive upon transition to connected i.e. early measurement reporting. The contribution suggests not to blindly re-use the mechanism defined in LTE but evaluates which key elements would be appropriate to re-use in NR RRC REL-16. The paper focusses particularly on the following aspects:

- Need to support additional measurements and use of broadcast and dedicated signalling

- When and where to perform measurements

- When to report results (including message to be used)

For all these aspects, a clear proposal/ proseded way forward is suggested all aiming to define a solid and simple initial baseline.

# Discussion

Early measurement reporting is available both in UMTS and LTE. As usual, it seems sensible to use the mechanism defined in LTE as starting point for NR. We agree the mechanisms already defined in UMTS and LTE should be considered, but think we should carefully review the different elements and decides which items make sense to introduce in NR. I.e. not all options defined in LTE (or UMTS) should automatically be ported to NR. We think that for the initial NR baseline we should identify which of the main elements of the LTE mechanism are really needed in NR i.e. also trying to avoid copying items that were introduced in a rush and may not be specified carefully.

Measurements to perform

A first question is what measurements the UE performs for this feature i.e. whether

- The UE only reports results available from measurements performed for idle/ inactive mobility or

- The UE may also be configured to perform additional measurements

We think this is a key question as with a) the feature does not impose an additional (measurement) burden on the UE. I.e. with a) it seems a solution only with broadcast information seems possible, as any UE who supports the feature can simply report what it has available.

Although a) seems attractive from UE perspective, it seems insufficient as a network may employ certain frequencies purely to configure SCells/ SCG cells in connected. I.e. for these frequencies the UE will not have result available from measurements performed for re-selection i.e. there is an additional burden. We think that such additional burden should not be imposed on all UEs in idle/ inactive i.e. for such additional measurements there is a need for dedicated signalling so we can selectively configure for UEs requiring this.

Proposal 1	Support a means by which the UE may be configured to perform addition measuements (e.g. frequencies not used for re-selection)

Proposal 2	Introduce dedicated signalling so the additional measurements can selectively configure for particular UEs (avoiding battery draf for UEs not requiring such measurements)

Proposal 3	Introduce broadcast signalling that merely controls the reporting of the results available from measurement performed for re-selection (aka reporting configuration)

When and where to perform measurements

When the UE is not configured to perform additional measurements, there seems no realy need to limit when and where to perform measurements. I.e. the UE can just follow what is indicated by the cell it is currently camping on. If however the UE is performing additional measurements and dedicated signalling is used, it may be good to introduce some means to avoid that UE performs these measurements indefinitely.

There seem to be 2 basic means to avoid the UE is burdened indefinitely to perform the additional measuremetns. I.e. by defining that the UE stops additional measurments and it’s reporting upon:

- Duration i.e. upon expiry of timer

- Area i.e. upon leaving a particular area (set by configuration)

We think that duration seems primary candidate as it is simple, general and since these results seem less needed for UEs staying inactive for very long. We think that also introducing the option to define an area can be regarded as a further enhancement/ optimisation.

Proposal 4	For the (additional) measurements configured by dedicated signalling, introduce a means to avoid UE performs the measurements indefinitely.

Proposal 5	As a baseline, introduce a timer by which network can limit the duration the UE performs the (additional) measurements configured by dedicated signalling.

When to report measurements

We think that additional measurements are only configured as addition to measurements performed for re-seletion. Consequently, we can assume UE reports only following access in cells broadcasting the concerned information.

Proposal 6	The UE only reports early measurements (includes results of measurements configured by dedicated signalling) in cells broadcasting related information

A further key aspect concerns the message by which the UE transfers the measurement results. The primary candiates seem to be:

- Within a message used during transition to connected e.g. SetupComplete, ResumeComplete

- By a separate UE information transfer procedure, after successful completion of transition to connected. In this option, UE reports availability of results during transition to connected

We think that option b) as used in LTE introduces delays, is cumbersome and not really needed given that the size of the information seems limited seems (i.e. UE merely reports the last results, if still valid i.e. performed recently). There may however be a need to avoid transfer of the early measurements prior to security activation. If so, option b) may need to be supported for the case of connection setup. Regardless, it seems beneficil to introduce option a) at least for the connection resume case.

Proposal 7	For the transfer of early measurements, support the following options

- Within a message used during transition to connected, at least for the case of connection resume

- By a separate UE information transfer procedure, after successful completion of transition to connected (with UE indicating availability during transition to connected)

# Conclusion & recommendation

This contribution aims to define a baseline for the reporting of measurements performed in idle/ inactive upon transition to connected i.e. early measurement reporting. The document includes the following proposals that RAN2 is requested to discuss and conclude:

Proposal 1	Support a means by which the UE may be configured to perform addition measuements (e.g. frequencies not used for re-selection)

Proposal 2	Introduce dedicated signalling so the additional measurements can selectively configure for particular UEs (avoiding battery draf for UEs not requiring such measurements)

Proposal 3	Introduce broadcast signalling that merely controls the reporting of the results available from measurement performed for re-selection (aka reporting configuration)

Proposal 4	For the (additional) measurements configured by dedicated signalling, introduce a means to avoid UE performs the measurements indefinitely.

Proposal 5	As a baseline, introduce a timer by which network can limit the duration the UE performs the (additional) measurements configured by dedicated signalling.

Proposal 6	The UE only reports early measurements (includes results of measurements configured by dedicated signalling) in cells broadcasting related information

Proposal 7	For the transfer of early measurements, support the following options

- Within a message used during transition to connected, at least for the case of connection resume

- By a separate UE information transfer procedure, after successful completion of transition to connected (with UE indicating availability during transition to connected)

# References

[1] TS 38.331 Radio Resource Control